I. Definition of Display Panel and Keys

Menu up and down confirmation

Menu key: select function

Up arrow: parameter increment

Down arrow: decreasing parameter

Confirm key: OK and save.

Second, the menu function

Press the menu key after powering on and turning on, and the menu menu will appear in turn; Press the up or down key to modify the function parameters, and press the Enter key to save the current function and parameters (with power-off memory after saving).

Menu menu:

| A001 |  | A512 | Set the address code, modify the address code (A001~A512) up or down, and confirm to save. |
| --- | --- | --- | --- |
| CH04 |  | CH147 | Switch the three channels CH04, CH10 and CH55 up or down, and confirm to save. Default CH10 |
| M000 |  | M127 | Effect mode, switch the built-in effect up or down, and confirm to save. |
| S000 |  | S255 | Modify the running speed of hexagon built-in effect up or down (S000~ S255), and confirm to save. |
| Soud |  | Soud | Voice control mode. |
| R255 |  | R000 | R Dimming the red light bulb, modify the brightness of the red light bulb up or down (R000~ R255), and press the Enter key to save. |
| G255 |  | G000 | G dim the green light bulb, modify the brightness of the green light bulb up or down (G000~ G255), and press the Enter key to save. |
| B255 |  | B000 | B Dim the blue light bulb, modify the brightness of the blue light bulb up or down (B000~ B255), and press the Enter key to save. |
| W255 |  | W000 | W Monochrome lamp brightness, modify the monochromatic lamp brightness up or down (W000~W255), and press the Enter key to save. |
| T000 |  |  | Display the temperature, such as T045, indicating that the current lamp temperature is 45℃. |

Third, master-slave control

Two or more identical lamps are connected by DMX three-core signal lines, all lamps are set to any address code A001~A512, and any one is set as the master, while other lamps are slaves, and all slave displays do not flicker; When the master adjusts the effects of gradual change, pulse change, jump, voice control and self-walking, all slaves synchronize the effects of gradual change, pulse change, jump, voice control and self-walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

2. All lamps and lanterns must be used when the DMX512 console is closed.

Fourth, the factory settings

Press the menu key for 3 seconds to enter the factory setting when any address code is A001~A512. Factory settings mainly include the functions of each output power of lamps and lanterns, fan setting mode, setting temperature protection point and sending parameters. The factory sets any mode and presses the menu button for 3 seconds to exit.

Factory Settings Table:

| R255 |  | R020 | R red light bulb current setting, modify the hexagonal red light bulb current (R020-R255) up or down, and confirm to save. |
| --- | --- | --- | --- |
| G255 |  | G020 | G green light bulb current setting, modify the hexagonal green light bulb current (G020-G255) up or down, and confirm to save. |
| B255 |  | B020 | B blue light bulb current setting, modify the hexagonal blue light bulb current up or down (B020-B255), and confirm to save. |
| W255 |  | W020 | W monochrome lamp current setting, modify the monochrome lamp current setting up or down (W020~W255), and press the Enter key to save. |
| FAN1 |  | FAN0 | Fan setting: start the fan as soon as the FAN0 light bulb lights up, and start the fan as soon as the FAN1 reaches the set temperature protection point, and press the Enter key to save. |
| T040 |  | T-105 | Set the temperature protection point, modify the parameters (40℃~75℃) up or down, and press the Enter key to save. |
| Send |  | Send | Send the factory setting parameters of this machine up or down to all other lamps with three-core signal lines connected in parallel; Confirm the sending parameters and press the menu button for 3 seconds. Exit, deny the parameter and press the confirm key to cancel sending. |

V. DMX512 console

After electrifying, the address codes of all lamps and lanterns are set, and all lamps and lanterns are connected to the DMX512 console in parallel with three-core signal lines, and the address codes will stop flashing, indicating that the DMX512 console signal has been sent to the lamps and lanterns. According to each channel description, the DMX512 console is used to control related functions.

CH4 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | Linear dimming of red lamp beads |
| 2 | 000-255 | Linear dimming of green lamp beads |
| three | 000-255 | Linear dimming of blue lamp beads |
| four | 000-255 | Linear dimming of white lamp beads |

CH10 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | Total dimming |
| 2 | 000-255 | stroboscopic |
| three | 000-255 | white light |
| four | 000-255 | red |
| five | 000-255 | green |
| six | 000-255 | blue |
| seven | 000-255 | model |
| eight | 000-255 | speed |
| nine | 000-255 | background color |
| 10 | 000-255 | Background tone light |

CH55 channel description:

| Channel value | Mode code | effect |
| --- | --- | --- |
| one | 000-255 | Linear dimming of the first group of red lamp beads |
| 2 | 000-255 | Group 1 Linear Dimming of Green Lamp Beads |
| three | 000-255 | Linear dimming of the first group of blue lamp beads |
| four | 000-255 | Group 2 Linear Dimming of Red Lamp Beads |
| five | 000-255 | Group 2 Linear Dimming of Green Lamp Beads |
| six | 000-255 | Linear dimming of the second group of blue lamp beads |
| … | … | … … |
| fifty-two | 000-255 | Group 18 Linear Dimming of Red Lamp Beads |
| 53 | 000-255 | Group 18 Linear Dimming of Green Lamp Beads |
| 54 | 000-255 | Group 18 Linear Dimming of Blue Lamp Beads |
| 55 | 000-255 | Linear dimming of white lamp beads |

VI. Technical parameters:

Voltage: AC100~220V 50/60HZ

Power: 72*0.25W(18W)+ 60W

Lamp beads: 72 5050 tri-color LED lamp beads+long white light lamp beads

Control mode: DMX512, self-propelled, master-slave, voice control and RDM function.

Channels: CH04, CH10 and CH55

Dimming: 32bit 0~100% linear dimming

Features: dyeing+flash+auxiliary light

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: 20*13*18 CM

Weight: 1.6KG