First, the display panel

Menu up and down confirmation

Key definition:

Menu key: select function

Up arrow: parameter increment

Down arrow: decreasing parameter

Confirm key: OK and save.

Second, the menu function

Press the menu key after powering on and turning on, and the menu menu will appear in turn; Press the up or down key to modify the function parameters, and press the Enter key to save the current function and parameters (with power-off memory after saving).

Menu menu:

| A001 |  | A512 | Modify the address code (A001~A512) up or down, and press the Enter key to save. A001 is the default. |
| --- | --- | --- | --- |
| C004 |  | C144 | Modify channels C004, C12 and C144 up or down, and confirm to save. The default is CH12. |
| M000 |  | M096 | Modify the built-in effect (M000~M096) up or down, and press OK to save. The default value is M000. |
| S000 |  | S255 | Modify the running speed of the built-in effect up or down (S000~S255), press the Enter key to save, and the default value is S010. |
| R255 |  | R000 | Modify the brightness of the red light bulb (R000~ R255) up or down, and press the Enter key to save. R255 is the default. |
| G255 |  | G000 | Modify the brightness of the green light bulb (G000~G255) up or down, and press the Enter key to save. The default value is G255. |
| B255 |  | B000 | Modify the brightness of the blue light bulb (B000~ B255) up or down, and press the Enter key to save. B255 is the default. |
| W255 |  | W000 | Modify the brightness of the white light bulb (B000~ B255) up or down, and press the Enter key to save. B255 is the default. |
| T000 |  |  | Display the temperature, such as T045, indicating that the current lamp temperature is 45℃; 10K thermistor display T000 is not installed. |

Third, the factory settings

In case of any address code from A001~A512, press the menu key for 5 seconds to enter the factory setting. The factory settings mainly include the functions of lamp output power, fan setting mode, setting temperature protection point and sending parameters. Press the menu button for 5 seconds to exit any mode set by the factory.

Factory Settings Table:

| R255 |  | R032 | Modify the red bulb current (R032~ R255) up or down, and press the Enter key to save. R220 is the default. |
| --- | --- | --- | --- |
| G255 |  | G032 | Modify the green bulb current (G032~G255) up or down, and press the Enter key to save. The default value is G220. |
| B255 |  | B032 | Modify the blue bulb current (B032~ B255) up or down, and press the Enter key to save. B220 is the default. |
| W255 |  | W032 | Modify the white bulb current (w032~ w255) up or down, and press the Enter key to save. The default value is w220. |
| Dir0 |  | Dir7 | Modify the direction of running water (d0~ d7) up or down, and confirm to save. d0 is the default. |
| FAN0 |  | FAN1 | Fan setting: start the fan when the light bulb of FAN0 is on, and start the fan when the temperature of FAN1 reaches 40 degrees. Press the Enter key to save. |
| T040 |  | T070 | Set the temperature protection point, modify the parameters (40℃~70℃) up or down, and press the Enter key to save. The default value is T065. |
| Send |  | Send | Send the factory setting parameters of this machine up or down to all other lamps with three-core signal lines connected in parallel; Press the menu button for 5 seconds to confirm the sending parameters, and press the confirm button to cancel the sending if the parameters are denied. |

Fourth, master-slave control

Two or more identical lamps are connected by DMX three-core signal lines, all lamps are set to any address code A001~A512, and any one is set as the master, while other lamps are slaves, and all slave displays do not flicker; When the master adjusts the gradient, pulse change, voice control and self-propelled effect, all slaves synchronize the gradient, pulse change, voice control and self-propelled effect.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

2. All lamps and lanterns must be used when the DMX512 console is closed.

V. DMX512 console

Set the address codes of all lamps and lanterns after power-on, and then connect all lamps and lanterns to DMX512 console in parallel with three-core signal lines. The address codes will stop flashing, indicating that DMX512 console signals have been sent to lamps and lanterns, and use DMX512 console to control related functions according to the channel table.

CH04 Channel Table Description:

| passage | Channel value | Basic function of channel |
| --- | --- | --- |
| one | 000-255 | R red lamp bead linear dimming |
| 2 | 000-255 | G linear dimming of green lamp beads |
| three | 000-255 | B linear dimming of blue lamp beads |
| four | 000-255 | W white lamp bead linear dimming |

CH12 Channel Table Description:

| passage | Channel value | Basic function of channel |
| --- | --- | --- |
| one | 000-255 | Total dimming |
| 2 | 000-255 | stroboscopic |
| three | 000-255 | R red lamp bead linear dimming |
| four | 000-255 | G linear dimming of green lamp beads |
| five | 000-255 | B linear dimming of blue lamp beads |
| six | 000-255 | W white lamp bead linear dimming |
| seven | 000-255 | Macro color |
| eight | 000-255 | model |
| nine | 000-255 | speed |
| 10 | 000-255 | direction |
| 11 | 000-255 | background color |
| 12 | 000-255 | Background tone light |

CH144 channel table description:

| passage | Channel value | Basic function of channel |
| --- | --- | --- |
| one | 000-255 | Linear dimming of the first red lamp bead |
| 2 | 000-255 | Linear dimming of the first green lamp bead |
| three | 000-255 | Linear dimming of the first blue lamp bead |
| four | 000-255 | Linear dimming of the first white lamp bead |
| … | … | … … |
| 141 | 000-255 | Linear dimming of the 36th red lamp bead |
| 142 | 000-255 | Linear dimming of the 36th green lamp bead |
| 143 | 000-255 | Linear dimming of the 36th blue lamp bead |
| 144 | 000-255 | Linear dimming of the 36th white lamp bead |

VII. Technical parameters

Voltage: AC100~240V 50/60HZ

Power: 200W

Lamp beads: 36 8W four-in-one LED lamp beads

Control mode: DMX512, self-propelled, master-slave, voice control, RDM.

Channels: select channels CH04, CH12 and CH144.

Dimming: 32bit 0~100% linear dimming

Features: dyeing+flash

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: weight: