Use the instructions

854-A394-A529-V12 Thousand-Hand Guanyin with Auxiliary

Light Manual

Please read the manual carefully before installing and using the product

Table of Contents

1. Precautions and Installation Precautions and Installation	3

1.1. Statement	3

1.2. Maintenance and servicing	3

1.3. Product Precautions	3

1.4. Product Introduction	4

2. Display panel and button definitions	4

2.1 Control Panel	4

2.2 Menu Functions	5

3. Master-slave control	7

4. DMX512 control	7

4.1  Address code	7

4.2  CH29Channel table	8

4.3 CH62 Channel Table	9

4.4  CH170Channel table	9

4.5 CH254 Channel Table	11

4.6 CH308 Channel Table	12

4.8  CH332 channel table	15

5. Common faults	21

1. Precautions and Installation Precautions and Installation

1.1. Statement

Thank you for choosing our company's products! This product is in good condition and fully packaged when leaving the factory. To ensure your safe and effective use of this product, please read this instruction manual carefully and thoroughly before using it. This manual contains important information about installation and use. Please follow the instructions for installation and operation, and keep this manual safe for easy use.

Technical changes to this manual are subject to no prior notice.

1.2. Maintenance and servicing

- This fixture is for professional use only.

- If the external flexible cable or wire of this fixture is damaged, it should be replaced using specialized wires or wires provided by the manufacturer or its service agents.

- The light source contained in this fixture may only be replaced by the manufacturer or its service agent, or by a qualified personnel of a similar entity.

- Always connect the product to a protected circuit (circuit breaker or fuse). Ensure the product has proper electrical grounding to avoid the risk of electric shock or fire.

- To avoid unnecessary wear and extend its service life, when not in use, the power supply should be completely cut off through a circuit breaker or unplugged.

- The casing may heat up during operation. This product should be installed in a well-ventilated location.

- When transferring products from extreme temperature environments (for example, moving from cold trucks to warm, humid conditions), the internal electronics may condense. To avoid malfunctions, allow the product to fully adapt to the surrounding environment before powering on.

- It is well known that flash can trigger seizures. Users must comply with local notification laws regarding flash usage.

1.3. Product Precautions

- Before cleaning the product or replacing the fuse, disconnect the power first.

- When not in use, replace and secure all protections for power, data, USB, or other ports to  ensure they meet IP protection standards.

- When installing this product overhead, please use a safety rope.

- Connect this product to a grounded and protected circuit.

- When the product is turned on, check the light source.

- Do not place any flammable materials within 20  cm of this product during operation or power  supply.

- Do not operate this product if the casing, lens, or cable is damaged.

- Only handles or lifting/mounting brackets can be used to handle this product.

- To avoid unnecessary wear and extend its service life, when not in use, the power supply should be completely cut off through a circuit breaker or unplugged.

- If serious operational issues occur, use should be stopped immediately

1.4. Product Introduction

- Voltage: AC100~240V 50/60HZ

- Power: 12*60W (800W)

- LED beads: 12 four-in-one 60W LED beads, 432 2015 golden light beads, 432 2015 cool white beads

- Control methods: DMX512, self-propelled, master-slave, voice control, with RDM function.

- Channel: CH29. CH62. CH170. CH254. CH308. CH332

- Dimming: 32-bit 0~100% linear dimming

- Features: Single swing + dyeing + beam

- Operating temperature: -30°C~50°C

- Flicker frequency: 1~30Hz

- Appearance: Metallic, black

- Connection methods: DMX512 input/output / power input/output.

- IP Rating: IP20

- Size:

- Weight: 26.5KG

2. Display panel and button definitions

2.1 Control Panel

Menu key: Enter/return to the main menu

Up key: Menu function to select upwards or add parameters

Down key: Menu function to select down / Decrease parameters

Confirm key: Select menu functions, confirm and save menu function parameters when modifying.

2.2 Menu Functions

After powering on and resetting the whole machine, press the menu key to enter the main menu (exit after 30 seconds of no operation on the main menu interface). See the main menu menu (5 main menu options in total) for details; Use the up or down key to select the main menu, confirm to modify the function parameters of each menu, then press confirm to save the current function parameters (saving will have power-off memory).

| Main menu | Submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System settings | 1. DMX address | Modify the DMX address code up or down (address code options range 001-512), press confirm to save, default is 001. |  |
|  | 2. Encoder | Modify on/off by moving up or down, confirm to save, and close by default, Set the lighting motor software/mechanical reset; turning on sets for software reset, and off sets for mechanical reset. |  |
|  | 3. Y inversion | Modify on/off by moving up or down, confirm to save, default to close. |  |
|  | 4. Amplify the reversal | Modify on/off by moving up or down, confirm to save, default to close. |  |
|  | 5. No signal | Adjust the signal to keep/reset up or down, press confirm to save, default reset to zero. |  |
|  | 6. Voice control sensitivity | Adjust the voice sensitivity up or down (selectable range 00-99), confirm to save, default 009. |  |
|  | 7. Reset | Modify Yes/No up or down, hold the OK key, save the default, default: No. |  |
|  | 8. Restore factory settings | Modify Yes/No up or down, hold the OK key, save the default, default: No. |  |
|  | 9. Advanced settings | *** *** *** *** After entering the correct password, you can set the lamp to lock. |  |
| 2. Operating mode | 1. Operating mode | Select the running mode DMX/voice control/automatic by moving up or down, confirm to save, default DMX. |  |
|  | 2. Channel selection | Select channels up or down: CH29.CH62.CH170.CH254.CH308.CH332, confirm key to save, default CH29. |  |
|  | 3. Y mode amplitude | Modify the Y mode amplitude up or down (selectable range 000-255), confirm to save, default 000. |  |
| 3. Manual control | 1、Y | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 2. Y fine-tuning | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 3. Y velocity | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 4. Y Mode | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 5、CTO | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 6. White CTO | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 7. Color macro | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 8. Pattern | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 9. White light pattern | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 10. LED mode | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 11. LED speed | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 12. LED tail tail | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 13. White light mode | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 14. White light speed | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 15. White light tail drag | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 16. Background color | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 17. Background color dimming light | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 18. Dimming | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 19. White light dimming | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 20. Strobe | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 21. White light flashing | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 22. Enlarge | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 23. Function | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 24. Red | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 25. Green | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 26. Blue | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 27. White | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 28. Cool white dimming | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
|  | 29. Warm white dimming | Modify the value up or down (adjustable range 000-255), confirm to save, default is 000. |  |
| 4. Display settings | 1. Language Choice | Select Chinese/EN (English) up or down, confirm to save, default is Chinese. |  |
|  | 2. Inverted display settings | Select up or down to turn off or enable reflection settings, confirm to save, default is off. |  |
|  | 3. Screen saver | Select Turn Screen Protector on or off by moving up or down, press confirm to save, which is enabled by default. |  |
|  | 4. Software version | 05TB program identification code, cannot be modified. |  |
| 5. Factory setup | Please enter your password when displayed! Change up or down to 138, then press the confirm button to enter factory settings. | 1. Y1 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 2. F1 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 3. Y2 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 4. F2 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 5. Y3 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 6. F3 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 7. Y4 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 8. F4 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 9. Y5 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 10. F5 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 11. Y6 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 12. F6 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 13. Y7 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 14. F7 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 15. Y8 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 16. F8 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 17. Y9 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 18. F9 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 19. Y10 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 20. F10 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 21. Y11 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 22. F11 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 23. Y12 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 24. F12 axis calibration | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 25. Y travel | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 26. Focusing stroke | Modify parameters up or down (range 000-255), confirm to save, default 128. |
|  |  | 27. Red 1 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 28. Green 1 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 29. Blue 1 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 30. White 1 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 31. Red 2 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 32. Green 2 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 33. Blue 2 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 34. White 2 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 35. Red 3 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 36. Green 3 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 37. Blue 3 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 38. White 3 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 39. Red 4 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 40. Green 4 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 41. Blue 4 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 42. White 4 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 43. Red 5 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 44. Green 5 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 45. Blue 5 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 46. White 5 | Modify parameters up or down (range 000-255), confirm to save, default 252. |
|  |  | 47. Red 6 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 48. Green 6 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 49. Blue 6 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 50. White 6 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 51. Red 7 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 52. Green 7 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 53. Blue 7 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 54. White 7 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 55. Red 8 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 56. Green 8 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 57. Blue 8 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 58. White 8 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 59. Red 9 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 60. Green 9 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 61. Blue 9 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 62. White 9 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 63. Red 10 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 64. Green 10 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 65. Blue 10 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 66. White 10 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 67. Red 11 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 68. Green 11 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 69. Blue 11 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 70. White 11 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 71. Red 12 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 72. Green 12 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 73. Blue 12 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 74. White 12 | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 75. LEDYF direction | Modify parameters up or down (range 000-255), confirm to save, default 000. |
|  |  | 76. Current | Modify parameters up or down (range 000-255), confirm key to save, default 200. |
|  |  | 77、Logo | Select 000 /001 up or down, press confirm to save, default 000. |
|  |  | 78. Temperature | Adjust the temperature parameters up or down (range 40-70), confirm to save, default 050. |
|  |  | 79. Temperature - Fan | Closed/Standard/Quiet |
|  |  | 80、LED | LEDA/LEDB |
|  |  | 81. Color mixing mode | Up or down RGBW/CMY, confirm key to save, default RGBW. |
|  |  | 82. PWM frequency | Up or down at 600/1200/2000/4000/6000/15000Hz, confirm key to save, default 1200Hz. |
|  |  | 83. Dimming curve | Linear/square/inversesquare/S-curve |
|  |  | 84. Dimming modes | Fast/smooth |
|  |  | 85. Exercise with the light off | Linear/square/inversesquare/S-curve |
|  |  | 86. Time resets | Yes/No |

3. Master-slave control

If two or more identical fixtures are connected using DMX three-core signal cables, all fixtures should be modified to any address code 001~512, set any one as the master and the others as the slave, and all slave displays should not flicker; When adjusting gradient, pulse, jump, voice-controlled, and auto-driven effects with the main unit, all slave effects synchronize with gradient, pulse, jump, voice-controlled, and auto-driven effects.

Special attention: 1. Only one main unit can be installed per group of lighting fixtures. If there are multiple main units, all the fixtures will flicker randomly and not synchronize.

- All fixtures must be used only when the DMX512 console is turned off for the master-slave units to function.

4. DMX512 control

4.1  Address Code

Set all lamp address codes correctly. Connect all fixtures in parallel with DMX three-core signal cables to the DMX512 console. When the address codes stop flashing, it means the DMX512 console signal has been sent to the fixtures. Follow the channel instructions to control related functions with the DMX512 console.

4.2 CH29 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Y coarse adjustment (Y mode direction). |
| 2 | 000-255 | Y micro-adjustment body (Y mode amplitude). |
| 3 | 000-255 | Y speed |
| 4 | 000-255 | Y mode (Y_Mode). |
| 5 | 000-255 | CTC |
| 6 | 000-255 | White light CTC |
| 7 | 000-255 | Color Chart |
| 8 | 000-255 | Pattern |
| 9 | 000-255 | White light pattern |
| 10 | 000-255 | LED built-in effects |
| 11 | 000-255 | LED built-in effect speed |
| 12 | 000-255 | LED built-in effect tail time |
| 13 | 000-255 | White light built-in effect |
| 14 | 000-255 | White light built-in effect speed |
| 15 | 000-255 | White light built-in effect drag time |
| 16 | 000-255 | LED background color |
| 17 | 000-255 | LED background color dimmed light |
| 18 | 000-255 | Total dimming |
| 19 | 000-255 | White light is always dimmed |
| 20 | 000-255 | Shutter |
| 21 | 000-255 | White light flashing (Shutter). |
| 22 | 000-255 | Magnify the whole picture |
| 23 | 000-255 | Function |
| 24 | 000-255 | Red (color selection for effects). |
| 25 | 000-255 | Green (color selection for effects). |
| 26 | 000-255 | Blue (color selection for effects). |
| 27 | 000-255 | White (color selection for effects). |
| 28 | 000-255 | Cold white always dimmed the light |
| 29 | 000-255 | Warm white always dimmed the lighting |

4.3 CH62 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Y1 rough adjustment (Y mode direction). |
| 2 | 000-255 | Y1 fine-tuning |
| 3 | 000-255 | Y2 coarse tuning (Y mode amplitude). |
| 4 | 000-255 | Y2 fine-tuning |
| 21 | 000-255 | Y11 coarse tuning |
| 22 | 000-255 | Y11 is fine-tuned |
| 23 | 000-255 | Y12 coarse tuning |
| 24 | 000-255 | Y12 fine-tuning |
| 25 | 000-255 | Y speed |
| 26 | 000-255 | Y mode (Y_Mode). |
| 27 | 000-255 | CTC |
| 28 | 000-255 | White light CTC |
| 29 | 000-255 | Color Chart |
| 30 | 000-255 | Pattern |
| 31 | 000-255 | White light pattern |
| 32 | 000-255 | LED built-in effects |
| 33 | 000-255 | LED built-in effect speed |
| 34 | 000-255 | LED built-in effect tail time |
| 35 | 000-255 | White light built-in effect |
| 36 | 000-255 | White light built-in effect speed |
| 37 | 000-255 | White light built-in effect drag time |
| 38 | 000-255 | LED background color |
| 39 | 000-255 | LED background tone light coarseness |
| 40 | 000-255 | Total dimming |
| 41 | 000-255 | White light is always dimmed |
| 42 | 000-255 | Shutter |
| 43 | 000-255 | White light flashing (Shutter). |
| 44 | 000-255 | Zoom in 1 |
| 55 | 000-255 | Magnification 12 |
| 56 | 000-255 | Function |
| 57 | 000-255 | Red (color selection for effects). |
| 58 | 000-255 | Green (color selection for effects). |
| 59 | 000-255 | Blue (color selection for effects). |
| 60 | 000-255 | White (color selection for effects). |
| 61 | 000-255 | Cold white always dimmed the light |
| 62 | 000-255 | Warm white always dimmed the lighting |

4.4 CH170 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | Total dimming and fine adjustment |
| 3 | 000-255 | Shutter |
| 4 | 000-255 | White light flashing (Shutter). |
| 5 | 000-255 | CTC |
| 6 | 000-255 | Pattern |
| 7 | 000-255 | White light pattern |
| 8 | 000-255 | Color Chart |
| 9 | 000-255 | White light built-in effect |
| 10 | 000-255 | White light built-in effect speed |
| 11 | 000-255 | White light built-in effect drag time |
| 12 | 000-255 | Function |
| 13 | 000-255 | Y1 coarse tuning |
| 14 | 000-255 | Y1 fine-tuning |
| 15 | 000-255 | Zoom in 1 |
| 16 | 000-255 | Red 1 is coarse tone |
| 17 | 000-255 | Red 1 is finely adjusted |
| 18 | 000-255 | Green 1 coarse tone |
| 19 | 000-255 | Green 1 is finely adjusted |
| 20 | 000-255 | Blue 1 is coarse |
| 21 | 000-255 | Blue 1 is fine-tuned |
| 22 | 000-255 | White 1 is coarse tone |
| 23 | 000-255 | White 1 is finely adjusted |
| 24 | 000-255 | Leng Bai 1-2-3-70-71-72 |
| 25 | 000-255 | Warm white 1-2-3-70-71-72 |
| 26 | 000-255 | Y2 coarse tuning |
| 27 | 000-255 | Y2 fine-tuning |
| 28 | 000-255 | Zoom in 2 |
| 29 | 000-255 | Red 2 is coarse |
| 30 | 000-255 | Red 2 is fine-tuned |
| 31 | 000-255 | Green 2 is coarse |
| 32 | 000-255 | Green 2 is finely tuned |
| 33 | 000-255 | Blue 2 is coarse |
| 34 | 000-255 | Blue 2 is fine-tuned |
| 35 | 000-255 | White 2 is coarse tone |
| 36 | 000-255 | White 2 is finely adjusted |
| 37 | 000-255 | Cold white 4-5-6-67-68-69 |
| 38 | 000-255 | Warm white 4-5-6-67-68-69 |
| 143 | 000-255 | Y11 coarse tuning |
| 144 | 000-255 | Y11 is fine-tuned |
| 145 | 000-255 | Magnify 11 |
| 146 | 000-255 | Red 11 is coarse |
| 147 | 000-255 | Red 11 is fine-tuned |
| 148 | 000-255 | Green 11 is coarse in tone |
| 149 | 000-255 | Green 11 fine-tuning |
| 150 | 000-255 | Blue 11 is in bold tones |
| 151 | 000-255 | Blue 11 is fine-tuned |
| 152 | 000-255 | White 11 is coarse |
| 153 | 000-255 | White 11 is finely tuned |
| 154 | 000-255 | Leng Bai 31-32-33-40-41-42 |
| 155 | 000-255 | Warm white: 31-32-33-40-41-42 |
| 156 | 000-255 | Y12 coarse tuning |
| 157 | 000-255 | Y12 fine-tuning |
| 158 | 000-255 | Magnification 12 |
| 159 | 000-255 | Red 12 is coarse |
| 160 | 000-255 | Red 12 fine-tuning |
| 161 | 000-255 | Green 12 is coarse |
| 162 | 000-255 | Green 12 fine-tuning |
| 163 | 000-255 | Blue 12 is coarse |
| 164 | 000-255 | Blue 12 fine-tuning |
| 165 | 000-255 | White 12 is coarse in color |
| 166 | 000-255 | White 12 is finely tuned |
| 167 | 000-255 | Leng Bai 34-35-36-37-38-39 |
| 168 | 000-255 | Warm white: 34-35-36-37-38-39 |
| 169 | 000-255 | Cold white always dimmed the light |
| 170 | 000-255 | Warm white always dimmed the lighting |

4.5 CH254 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Y1 rough adjustment (Y mode direction). |
| 2 | 000-255 | Y1 fine-tuning |
| 3 | 000-255 | Y2 coarse tuning (Y mode amplitude). |
| 4 | 000-255 | Y2 fine-tuning |
| 21 | 000-255 | Y11 coarse tuning |
| 22 | 000-255 | Y11 is fine-tuned |
| 23 | 000-255 | Y12 coarse tuning |
| 24 | 000-255 | Y12 fine-tuning |
| 25 | 000-255 | Y speed |
| 26 | 000-255 | Y mode (Y_Mode). |
| 27 | 000-255 | CTC |
| 28 | 000-255 | White light CTC |
| 29 | 000-255 | Color Chart |
| 30 | 000-255 | Pattern |
| 31 | 000-255 | White light pattern |
| 32 | 000-255 | LED built-in effects |
| 33 | 000-255 | LED built-in effect speed |
| 34 | 000-255 | LED built-in effect tail time |
| 35 | 000-255 | White light built-in effect |
| 36 | 000-255 | White light built-in effect speed |
| 37 | 000-255 | White light built-in effect drag time |
| 38 | 000-255 | LED background color |
| 39 | 000-255 | LED background tone light coarseness |
| 40 | 000-255 | Total dimming |
| 41 | 000-255 | White light is always dimmed |
| 42 | 000-255 | Shutter |
| 43 | 000-255 | White light flashing (Shutter). |
| 44 | 000-255 | Zoom in 1 |
| 55 | 000-255 | Magnification 12 |
| 56 | 000-255 | Function |
| 57 | 000-255 | Red (color selection for effects). |
| 58 | 000-255 | Green (color selection for effects). |
| 59 | 000-255 | Blue (color selection for effects). |
| 60 | 000-255 | White (color selection for effects). |
| 61 | 000-255 | Cold white always dimmed the light |
| 62 | 000-255 | Warm white always dimmed the lighting |
| 63 | 000-255 | Red 1 |
| 64 | 000-255 | Green 1 |
| 65 | 000-255 | Blue 1 |
| 66 | 000-255 | White 1 |
| 107 | 000-255 | Red 12 |
| 108 | 000-255 | Green 12 |
| 109 | 000-255 | Blue 12 |
| 110 | 000-255 | White 12 |
| 111 | 000-255 | Leng Bai 1 |
| 112 | 000-255 | Warm White 1 |
| 253 | 000-255 | Leng Bai 72 |
| 254 | 000-255 | Warm white 72 |

4.6 CH308 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Y1 rough adjustment (Y mode direction). |
| 2 | 000-255 | Y1 fine-tuning |
| 3 | 000-255 | Y2 coarse tuning (Y mode amplitude). |
| 4 | 000-255 | Y2 fine-tuning |
| 21 | 000-255 | Y11 coarse tuning |
| 22 | 000-255 | Y11 is fine-tuned |
| 23 | 000-255 | Y12 coarse tuning |
| 24 | 000-255 | Y12 fine-tuning |
| 25 | 000-255 | Y speed |
| 26 | 000-255 | Y mode (Y_Mode). |
| 27 | 000-255 | CTC |
| 28 | 000-255 | White light CTC |
| 29 | 000-255 | Color Chart |
| 30 | 000-255 | Pattern |
| 31 | 000-255 | White light pattern |
| 32 | 000-255 | LED built-in effects |
| 33 | 000-255 | LED built-in effect speed |
| 34 | 000-255 | LED built-in effect tail time |
| 35 | 000-255 | White light built-in effect |
| 36 | 000-255 | White light built-in effect speed |
| 37 | 000-255 | White light built-in effect drag time |
| 38 | 000-255 | LED background color |
| 39 | 000-255 | LED background tone light coarseness |
| 40 | 000-255 | LED background tone light fine-tuning |
| 41 | 000-255 | Total dimming |
| 42 | 000-255 | Total dimming and fine adjustment |
| 43 | 000-255 | White light is always dimmed |
| 44 | 000-255 | Shutter |
| 45 | 000-255 | White light flashing (Shutter). |
| 46 | 000-255 | Zoom in 1 |
| 57 | 000-255 | Magnification 12 |
| 58 | 000-255 | Function |
| 59 | 000-255 | Red (color selection for effects). |
| 60 | 000-255 | Fine-tune red (color selection for effects). |
| 61 | 000-255 | Green (color selection for effects). |
| 62 | 000-255 | Green fine-tuning (effect color selection). |
| 63 | 000-255 | Blue (color selection for effects). |
| 64 | 000-255 | Blue fine-tuning (effect color selection). |
| 65 | 000-255 | White (color selection for effects). |
| 66 | 000-255 | White fine-tuning (effect color selection). |
| 67 | 000-255 | Cold white always dimmed the light |
| 68 | 000-255 | Warm white always dimmed the lighting |
| 69 | 000-255 | Red 1 is coarse tone |
| 70 | 000-255 | Red 1 is finely adjusted |
| 71 | 000-255 | Green 1 coarse tone |
| 72 | 000-255 | Green 1 is finely adjusted |
| 73 | 000-255 | Blue 1 is coarse |
| 74 | 000-255 | Blue 1 is fine-tuned |
| 75 | 000-255 | White 1 is coarse tone |
| 76 | 000-255 | White 1 is finely adjusted |
| 157 | 000-255 | Red 12 is coarse |
| 158 | 000-255 | Red 12 fine-tuning |
| 159 | 000-255 | Green 12 is coarse |
| 160 | 000-255 | Green 12 fine-tuning |
| 161 | 000-255 | Blue 12 is coarse |
| 162 | 000-255 | Blue 12 fine-tuning |
| 163 | 000-255 | White 12 is coarse in color |
| 164 | 000-255 | White 12 is finely tuned |
| 165 | 000-255 | Leng Bai 1 |
| 166 | 000-255 | Warm White 1 |
| 307 | 000-255 | Leng Bai 72 |
| 308 | 000-255 | Warm white 72 |

4.7  CH332 Channel Table

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-255 | Y1 rough adjustment (Y mode direction). |
| 2 | 000-255 | Y1 fine-tuning |
| 3 | 000-255 | Y2 coarse tuning (Y mode amplitude). |
| 4 | 000-255 | Y2 fine-tuning |
| 21 | 000-255 | Y11 coarse tuning |
| 22 | 000-255 | Y11 is fine-tuned |
| 23 | 000-255 | Y12 coarse tuning |
| 24 | 000-255 | Y12 fine-tuning |
| 25 | 000-255 | Y speed |
| 26 | 000-255 | Y mode (Y_Mode). |
| 27 | 000-255 | CTC |
| 28 | 000-255 | White light CTC |
| 29 | 000-255 | Color Chart |
| 30 | 000-255 | Pattern |
| 31 | 000-255 | White light pattern |
| 32 | 000-255 | LED built-in effects |
| 33 | 000-255 | LED built-in effect speed |
| 34 | 000-255 | LED built-in effect tail time |
| 35 | 000-255 | White light built-in effect |
| 36 | 000-255 | White light built-in effect speed |
| 37 | 000-255 | White light built-in effect drag time |
| 38 | 000-255 | LED background color |
| 39 | 000-255 | LED background tone light coarseness |
| 40 | 000-255 | LED background tone light fine-tuning |
| 41 | 000-255 | Total dimming |
| 42 | 000-255 | Total dimming and fine adjustment |
| 43 | 000-255 | White light is always dimmed |
| 44 | 000-255 | Shutter |
| 45 | 000-255 | White light flashing (Shutter). |
| 46 | 000-255 | Zoom in 1 |
| 57 | 000-255 | Magnification 12 |
| 58 | 000-255 | Function |
| 59 | 000-255 | Red (color selection for effects). |
| 60 | 000-255 | Fine-tune red (color selection for effects). |
| 61 | 000-255 | Green (color selection for effects). |
| 62 | 000-255 | Green fine-tuning (effect color selection). |
| 63 | 000-255 | Blue (color selection for effects). |
| 64 | 000-255 | Blue fine-tuning (effect color selection). |
| 65 | 000-255 | White (color selection for effects). |
| 66 | 000-255 | White fine-tuning (effect color selection). |
| 67 | 000-255 | Cold white always dimmed the light |
| 68 | 000-255 | Warm white always dimmed the lighting |
| 69 | 000-255 | Dimming 1: Rough Mode |
| 70 | 000-255 | Dimming 1: Fine adjustment |
| 71 | 000-255 | Red 1 is coarse tone |
| 72 | 000-255 | Red 1 is finely adjusted |
| 73 | 000-255 | Green 1 coarse tone |
| 74 | 000-255 | Green 1 is finely adjusted |
| 75 | 000-255 | Blue 1 is coarse |
| 76 | 000-255 | Blue 1 is fine-tuned |
| 77 | 000-255 | White 1 is coarse tone |
| 78 | 000-255 | White 1 is finely adjusted |
| 179 | 000-255 | Dimming 12 is coarse adjustment |
| 180 | 000-255 | Dimming 12 with fine adjustments |
| 181 | 000-255 | Red 12 is coarse |
| 182 | 000-255 | Red 12 fine-tuning |
| 183 | 000-255 | Green 12 is coarse |
| 184 | 000-255 | Green 12 fine-tuning |
| 185 | 000-255 | Blue 12 is coarse |
| 186 | 000-255 | Blue 12 fine-tuning |
| 187 | 000-255 | White 12 is coarse in color |
| 188 | 000-255 | White 12 is finely tuned |
| 189 | 000-255 | Leng Bai 1 |
| 190 | 000-255 | Warm White 1 |
| 331 | 000-255 | Leng Bai 72 |
| 332 | 000-255 | Warm white 72 |

| Note 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| The menu Y mode amplitude is set to 0; use Y control for fine adjustment or Y2 coarse adjustment to control the amplitude of Y mode |  |  |  |  |  |
| The menu Y mode amplitude ranges from 1 to 255, and the function channel ranges from 205 to 255. Control the amplitude of Y mode Functional channels range from 0 to 204, controlled by the Y mode amplitude set by the menu |  |  |  |  |  |

Pattern

Y Mode

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-003 | No effect |
| 2 | 004-075 | Every 4 values creates one effect, for a total of 17 effects |
| 3 | 076-255 | Effect 18 |

Strobe

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-019 | The light valve closed |
| 2 | 020-024 | The light valve opened |
| 3 | 025-064 | Strobe ranges from fast to slow |
| 4 | 065-069 | The light valve opened |
| 5 | 070-084 | 100-0% flicker, from fast to slow |
| 6 | 085-089 | The light valve opened |
| 7 | 090-104 | 0-100% flicker, from fast to slow |
| 8 | 105-109 | The light valve opened |
| 9 | 110-124 | Random flicker, from fast to slow |
| 10 | 125-129 | The light valve opened |
| 11 | 130-144 | Random 100-0% flicker, from fast to slow |
| 12 | 145-149 | The light valve opened |
| 13 | 150-164 | Random 0-100% flicker, from fast to slow |
| 14 | 165-169 | The light valve opened |
| 15 | 170-184 | Pulse strobe 1, from fast to slow |
| 16 | 185-189 | The light valve opened |
| 17 | 190-204 | Random pulse strobe flashes, from fast to slow |
| 18 | 205-209 | The light valve opened |
| 19 | 210-224 | 0-100-0% flicker, from fast to slow |
| 20 | 225-229 | The light valve opened |
| 21 | 230-244 | 100-0-100% flicker, from fast to slow |
| 22 | 245-255 | The light valve opened |

Color Chart

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 0 | None |
| 2 | 001-002 | 2700k |
| 3 | 003-004 | 3200k |
| 4 | 005-006 | 4200k |
| 5 | 007-008 | 5600k |
| 6 | 009-010 | 8000k |
| 7 | 011 | Blue |
| 8 | 012-048 | Blue to cyan |
| 9 | 049 | Blue |
| 10 | 050-086 | From green to green |
| 11 | 087 | Green |
| 12 | 088-124 | Green to yellow |
| 13 | 125 | Yellow |
| 14 | 126-162 | Yellow to red |
| 15 | 163 | Red |
| 16 | 164-200 | Red to magenta |
| 17 | 201 | Magenta |
| 18 | 202-238 | Magenta to blue |
| 19 | 239 | Blue |
| 20 | 240-247 | Gradient effects, from fast to slow |
| 21 | 248-255 | The pulse change effect ranges from fast to slow |

Function

| Passage | Channel values | Basic functions |
| --- | --- | --- |
| 1 | 000-009 | Empty |
| 2 | 010-014 | Y-axis motion turns off the light |
| 3 | 015-019 | Y-axis motion turns off the light |
| 4 | 020-024 | rgbw |
| 5 | 025-029 | cmy |
| 6 | 030-034 | Y-axis control activated |
| 7 | 035-039 | Y-axis control is turned off |
| 8 | 040-044 | Reserve |
| 9 | 045-049 | Reserve |
| 10 | 050-054 | Reserve |
| 11 | 055-059 | Y-axis reset |
| 12 | 060-064 | Enlarge reset |
| 13 | 065-069 | Zoom in and quietly reset |
| 14 | 070-074 | Reset the whole machine (requires zeroing) |
| 15 | 075-079 | Lamp head sequence 1-12 |
| 16 | 080-084 | Lamp head sequence 12-1 |
| 17 | 085-089 | Reserve |
| 18 | 090-094 | Y reversal begins |
| 19 | 095-099 | Reserve |
| 20 | 100-104 | Y reversal closes |
| 21 | 105-109 | Reserve |
| 22 | 110-114 | Reserve |
| 23 | 115-119 | Reserve |
| 24 | 120-134 | Reserve |
| 25 | 135-139 | Quick dimming |
| 26 | 140-144 | Smooth dimming |
| 27 | 145-149 | Linear curve |
| 28 | 150-154 | Square ratio curve |
| 29 | 155-159 | Inverse square curve |
| 30 | 160-164 | S-curve |
| 31 | 165-169 | Reserve |
| 32 | 170-174 | Reserve |
| 33 | 175-179 | PWM frequency is 600HZ |
| 34 | 180-184 | PWM frequency is 1200Hz |
| 35 | 185-189 | PWM frequency is 2000HZ |
| 36 | 190-194 | PWM frequency is 4000HZ |
| 37 | 195-199 | PWM frequency 6000HZ |
| 38 | 200-204 | PWM frequency is 15,000HZ |
| 39 | 205-255 | Y mode amplitude |

- Common faults

Corresponding solutions have been proposed for some common faults. Any unsolvable problem should be handled by professionals. Before maintaining the lighting fixtures, please disconnect the power first.

- The lamp beads did not light up

- Check whether the voltage matching the fixture is installed;

- Check whether the connection to the lamp's power supply or the control switch is not properly connected;

- Check if the power supply is insufficient;

- Check whether the DMX512 controller has sent the command.

2. After the lamp resets normally, it is not subject to control by the console

- Check whether the lamp's digital start address values and function options are correct;

- Check whether the communication control line connections are correct, whether the communication line is too long or has been interrupted;

- Check whether the control equipment has failed, and whether the signal amplifier connected in series is failing;

- Check whether communication lines are too long or if other devices are interfering with each other;

- Optimized cabling by shortening control signal line lengths and separating high-voltage and low-voltage lines;

- Add signal amplifiers;

- The signal cable uses high-quality shielded twisted pair cables;

- Connect a signal terminal resistor (120 ohms) at the end of the fixture.

3. The lamp cannot be started

- Check whether the power supply parameters match the fixtures;

- Check that during long-distance transportation, the fixture may have poor contact or fall off due to compression deformation, internal parts vibration, moisture, and other issues.

- Please check whether the internal wires and connectors of the fixture are loose or detached.

- Check whether the electronic components of the lighting fixture (such as electronic transformers, PCB boards, motor control boards, etc.) are loose, short-circuited, or burnt.