| Product Technical Specification (Moving Head Light Series) |  |  |  |  |
| --- | --- | --- | --- | --- |
| Product Image |  |  | Product Dimension Drawing |  |
| Lighting Specifications |  |  |  | Parameters |
| Product Name | Waterproof Moving Head Strobe Light | Light Color | Black | Input Voltage: AC100V-240V 50/60HZ Power Consumption: 1400W RGB color washing light source： 784 pcs×1.5W×3-color 5050 LED White light strobe light source： 392 pcs×2W Cool White LED RGB Zones：14 independently controllable zones White Light Zones：28 independently controllable zones CCT：Approx. 6000K (Daylight) Beam Angle：Combined light 98°×57°, strobe light 95°× 50° Luminous Flux：Up to 71620 lumens Tilt Range：180°electronic tilt Operating Temperature: -10℃～+40℃ Dimming：16-bit high-precision dimming for ultra-smooth transitions Strobe：Electronic strobe, rate 0–30 Hz Other Features：Adjustable PWM frequency (up to 25kHz) for flicker-free filming Control Mode: DMX、Art-Net、sACN、RDM Channel：8CH /11CH /13CH /14CH /24CH /30CH /47CH /74CH /97CH IP Rating:IP65 Product Dimensions: 406*339.4*202mm Net Weight: 11.5kg in-1 strobe & wash effect, 180° electric pan, rich varied effects. - High brightness output, pixel-level precise control, ideal for large stages and outdoor performances. |
| Net Weight | 11.5kg | Gross Weight | 14.5kg |  |
| Product Dimensions | 406*339.4*202mm | Lighting Material | All-magnesium alloy body |  |
| Optical System/Effect System |  |  |  |  |
| RGB color washing light source | 784 pcs×1.5W×3-color 5050 LED | White light strobe light source | 392 pcs×2W Cool White LED |  |
| RGB Zones | 14 independently controllable zones | White Light Zones | 28 independently controllable zones |  |
| Beam Angle | Combined light 98°×57°, strobe light 95°× 50° | CCT | Approx. 6000K (Daylight) |  |
| Tilt Range | 180°electronic tilt | Dimming | 16-bit high-precision dimming for ultra-smooth gradient effect |  |
| Luminous Flux | Up to 71620 lumens | Strobe | Electronic strobe, rate 0–30 Hz |  |
| Effect Light | - in-1 strobe & wash effect, 180° electric pan, rich varied effects. | Other Features | Adjustable PWM frequency (up to 25kHz) for flicker-free filming |  |
| Electrical Connection |  | Control System |  |  |
| Input Voltage | AC100V-240V 50/60HZ | Mechanical Movement | Vertical Scanning Angle 221°, Automatic Return Correction and 16bit Precision Fine-Tuning Function |  |
| Total Lamp Power | 1400W | Communication Design | DMX Wired/Wireless Transmission, Convenient and Fast Software Upgrade via DMX Data Cable |  |
| Signal Connection | 3-pin / 5-pin waterproof signal cable & XLR socket & RJ45 interface | Heat Dissipation System | Fan Heat Dissipation Structure and Temperature Intelligent Monitoring Technology, Automatically Adjusts Heat Dissipation System to Effectively Control Lamp Temperature |  |
| Power Connection | Waterproof Power I/O & Xishan Connector I/O | Control Protocol | DMX、Art-Net、sACN、RDM |  |
| Display Screen | Fixture and lamp usage time can be queried for easy maintenance | Channel Mode | 8CH/11CH/13CH/14CH/24CH/30CH/47CH/74CH/97CH |  |
| Photometric Diagram |  |  |  |  |