User's manual

810+468+A631+A628-8 Thousand-armed Guanyin with auxiliary light manual

Read the instructions carefully before installing and using the product

catalogue

1. Precautions and installation precautions and installation	3

1.1 statement	3

1.2 Maintenance	3

1.3 Product Precautions	3

1.4 Product Introduction	4

2. Display panel and button definition	4

2.1 Control panel	4

2.2 Menu function	5

3. Master and slave control	8

4.DMX512 control	8

4.1, address code	8

4.2 Channel Table for CH18 (Manual Page Channel)	8

4.3 CH38 channel table	9

4.4 CH87 Channel Table	10

5. Common faults	12

# 1. Precautions and installation precautions and installation

## 1.1 statement

Thank you for choosing our company's products! Our products are shipped in perfect condition with complete packaging. To ensure your safe and effective use of our products, please read this user manual carefully and thoroughly before using the product. This manual provides essential information on installation and usage. Please follow the instructions for installation and operation, and keep this manual handy for future reference.

No further notice will be given if any technical changes are made to this manual.

## 1.2 Maintenance

 This lamp is for professional use only.

 If the external soft cable or wire of this lamp is damaged, replace it with special wires or wires provided by the manufacturer or its service agent.

 The light source contained in this lamp can only be replaced by the manufacturer or its service agent or similar qualified personnel.

 Always connect the product to a protected circuit (circuit breaker or fuse). Ensure that the product has proper electrical grounding to avoid the risk of electric shock or fire.

 In order to avoid unnecessary wear and prolong its service life, when not in use, the power should be completely cut off through the circuit breaker or unplugging the plug.

 The shell of the product may heat up when running. The product should be installed in a well-ventilated position.

 When transferring a product from an extreme temperature environment (e.g., from a cold truck to a warm and humid environment), the internal electronics of the product may condense. To avoid causing failure, allow the product to fully adapt to the surrounding environment before turning on the power.

 It is well known that flash can trigger seizures. Users must comply with local laws on the use of flash.

## 1.3 Product Precautions

 Before cleaning the product or replacing the fuse, disconnect the power supply.

 When not in use, replace and fix all power, data, USB or other port covers to achieve IP protection level.

 When installing this product above the head, please use a safety rope.

 Connect this product to a grounded and protected circuit.

 Check the light source when the product is turned on.

 Do not place any flammable materials within 20 cm of this product when operating or connecting the power supply.

 Do not operate this product if the shell, lens or cable is damaged.

 This product can only be handled by hand or hoisting/installation bracket.

 In order to avoid unnecessary wear and prolong its service life, when not in use, the power should be completely cut off through the circuit breaker or unplugging the plug.

 In case of serious operation problems, it should be stopped immediately

## 1.4 Product Introduction

 Voltage: DC24V+12V

 Power: Total power 314W

 Light beads: 15060 +165050

 Control mode: DMX512, self-walking, master and slave, with RDM function, sound control

 Channel: CH18, CH38, CH87

 Light dimming: 32bit linear light dimming from 0 to 100%

 Features: shake head + color + focus

 Working temperature: -30 ~50 degrees

 Frequency of strobe: 1~30HZ

 Appearance: metal, black

 Connection mode: DMX512 input/output / power input/output.

 IP class: IP20

 size ：

 weight ：

# 2. Display panel and button definition

## 2.1 Control panel

Menu key: Enter/return to the main menu

Up arrow: menu function up selection / parameter increment

Down arrow: Menu function down selection / parameter decrement

Confirmation key: Select the menu function, and confirm and save when modifying the parameters of the menu function.

## 2.2 Menu function

After powering on and resetting the entire unit, press the menu key to enter the main menu (if there is no operation in the main menu interface for 30 seconds, exit). Refer to the main menu function table (which includes 5 main menu options). Use the up or down keys to navigate through the main menu, and the confirm key to enter the settings for modifying each menu. After making changes, press the confirm key to save the current function parameters (saving will retain the memory of power-off settings).

| primary menu | submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System Settings | 1. DMX address | Up or down modify the DMX address code (address code is optional range 001-512), confirm the key is saved, default 001. |  |
|  | 2. Y reversal | Up or down modify Y to turn on/off, confirm key save, default off. |  |
|  | 3. No signal | Up or down modify the signal reset/hold, confirm the key is saved, default reset. |  |
|  | 4. Voice control sensitivity | Modify the voice control sensitivity up or down (000-099), default 009. |  |
|  | 5. Reset | Up or down modification reset is yes/no, default is no. |  |
|  | 6. Restore factory Settings | Up or down modify restore factory Settings is / is not, default is no. |  |
|  | 7. Advanced Settings |  |  |
| 2. Operation mode | 1. Operation mode | Select the running mode of up or down: DMX, voice control and automatic. Confirm the key to save, and the default is automatic. |  |
|  | 2. Channel selection | Select channel 18CH, 38CH, or 87CH up or down to confirm the key save. The default is 18CH. |  |
| 3. Manual control | 1 、Y | Up or down modify the Y motor operation (adjustable range 000-255), confirm the key is saved, default 000. |  |
|  | 2、Y fine-tuning | Up or down modify the Y motor fine-tuning (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 3. Y velocity | Up or down modify the Y speed parameter value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 4、Y pattern | Up or down modify the Y motor mode (adjustable range 000-255), confirm the key is saved, default 000. |  |
|  | 5. Dimming | Up or down modify the dimming parameter value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 6. Strobe | Up or down modify the flicker parameter value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 7. Red | Up or down modify the red current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | Green | Up or down modify the green current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 9. Blue | Up or down modify the blue current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 10. White | Up or down modify the white current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 11、RGBW pattern | Up or down modify the RGBW mode (adjustable range 000-255), confirm key save, default 000. |  |
|  | 12. RGBW speed | Up or down modify the RGBW speed (adjustable range 000-255), confirm key save, default 000. |  |
|  | 13. Red | Up or down modify the red current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 14. Green | Up or down modify the value of green current (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 15. Blue | Up or down modify the blue current value (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 16、RGB pattern | Up or down modify the RGB mode (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 17. RGB speed | Up or down modify the RGB speed (adjustable range 000-255), confirm key save, default 000. |  |
|  | 18. Reset | Up or down modify the reset (adjustable range 000-255), confirm the key is saved, default 000. |  |
| 4. Display Settings | 1. Language selection | Select Chinese /EN (English) up or down, confirm the key to save, default Chinese. |  |
|  | 2. Inverted display Settings | Select the on/off reverse display setting up or down, confirm the key to save, default off. |  |
|  | 3. Screen saver Settings | Select the on/off backlight setting up or down, and confirm that the key is saved. The default is on. |  |
|  | 4. Software version | A631 |  |
| 5. Information Settings | sensor | Used to view the Hall state and temperature communication status |  |
|  | error message | Used to display error messages |  |
|  | Channel information | Used to display DMX channel data |  |
| 6. Factory setup | Display to enter the password! Up or down modify to 138, and press the confirm key to enter the factory Settings. | 1. Half step on the Y axis | Modify the half step of Y1 axis up/down (000-255), confirm and save the key, default is 128. Modify the half step of Y2 axis up/down (000-255), confirm and save the key, default is 146. Modify the Y3 axis half-step up/down (000-255), confirm the key to save, default 080. Modify the Y4 axis half-step up/down (000-255), confirm the key to save, default 139. Modify the Y5 axis half-step up/down (000-255), confirm the key to save, default 000. Modify the half step of Y6 axis up/down (000-255), confirm and save the key, default is 161. Modify the Y7 axis half-step up/down (000-255), confirm the key to save, default 118. Modify the Y8 axis half-step up/down (000-255), confirm the key to save, default 029. |
|  |  | 2. Y axis calibration | Y1 axis calibration is modified upward/downward (000-255), and the confirm key is saved, default 032. Y2 axis calibration is modified upward/downward (000-255), and the confirm key is saved, default 034. Y3 axis calibration is modified upward/downward (000-255), and the confirm key is saved, default 034. Y4 axis calibration is modified upward/downward (000-255), and the confirm key is saved, default 041. Y5 axis calibration up/down modification (000-255), confirm the key to save, default 000. Y6 axis calibration up/down modification (000-255), confirm key save, default 030. Y7 axis calibration up/down modification (000-255), confirm the key save, default 030. Y8 axis calibration up/down modification (000-255), confirm the key save, default 031. |
|  |  | 3. White balance | Red 1 up/down modify (000-255), confirm key save, default 252. Green 1 Up/down modification (000-255), confirm key save, default 252. Blue 1 Up/down modification (000-255), confirm key save, default 252. White 1 up/down modification (000-255), confirm key save, default 252. Red 2 up/down modify (000-255), confirm key save, default 252. Green 2 Up/down modification (000-255), confirm key save, default 252. Blue 2 up/down modify (000-255), confirm key save, default 252. White 2 up/down modification (000-255), confirm the key to save, default 252. Red 3 up/down modify (000-255), confirm key save, default 252. Green 3 Up/down modification (000-255), confirm the key to save, default 252. Blue 3 up/down modify (000-255), confirm key save, default 252. White 3 up/down modify (000-255), confirm key save, default 252. Red 4 is modified up/down (000-255), and the confirm key is saved by default 252. Green 4 Up/down modification (000-255), confirm key save, default 252. Blue 4 up/down modify (000-255), confirm key save, default 252. White 4 up/down modification (000-255), confirm the key to save, default 252. Red 5 is modified up/down (000-255), and the confirm key is saved by default 252. Green 5 Up/down modification (000-255), confirm the key to save, default 252. Blue 5 up/down modify (000-255), confirm key save, default 252. White 5 up/down modification (000-255), confirm the key to save, default 252. Red 6 is modified up/down (000-255), and the confirm key is saved by default 252. Green 6 Up/down modification (000-255), confirm the key to save, default 252. Blue 6 up/down modify (000-255), confirm key save, default 252. White 6 up/down modification (000-255), confirm the key to save, default 252. Red 7 is modified up/down (000-255), and the confirm key is saved by default 252. Green 7 Up/down modification (000-255), confirm the key to save, default 252. Blue 7 Up/down modification (000-255), confirm key save, default 252. White 7 up/down modification (000-255), confirm the key to save, default 252. Red 8 is modified up/down (000-255), and the confirm key is saved by default 252. Green 8 Up/down modification (000-255), confirm the key to save, default 252. Blue 8 up/down modify (000-255), confirm key save, default 252. White 8 up/down modification (000-255), confirm the key to save, default 252. |
|  |  | 4. LEDY direction | Up or down modify the current of blue LED 000 /001, confirm the key to save, default 000. |
|  |  | 5. Power | Modify the current of white LED (064-255) up or down, confirm the key to save, default is 240. |
|  |  | 6 、LED | Modify LEDA/LEDB up or down, confirm the key is saved, and default LEDA. |
|  |  | 7 、Logo | Select 000 /001 up or down, confirm key save, default 000. |
|  |  | 8. Fans | Select the fan control to turn off or on up or down. Confirm the key to save, and the default is on. |
|  |  | 9. On value | Up or down modify the temperature parameter (range 000-255), confirm the key to save, default 032. |
|  |  | 10. Time reset | Select yes or no for time reset control. Confirm key is saved, default is no. |

# 3. Master and slave control

Two or more identical lamps are connected using DMX three-core signal cables. All lamps are assigned any address code from 001 to 512, with one lamp set as the master and the others as slaves. The slave displays do not flicker. When the master adjusts the effects such as gradient, pulse, jump, sound control, and self-walking, all slaves will synchronize these effects.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash randomly and out of sync.

2. All lamps must be turned off when the DMX512 console is closed for the master and slave machines to function.

# 4.DMX512 control

## 4.1, address code

After setting the address code of all lamps, connect all lamps in parallel with DMX three-core signal lines to DMX512 control console. The address code stops flashing, indicating that the signal of DMX512 control console has been sent to the lamps. Control relevant functions according to channel instructions with DMX512 control console.

## 4.2 Channel Table for CH18 (Manual Page Channel)

| channel | Channel values | basic function |
| --- | --- | --- |
| 1 | 000-255 | Overall Y axis (direction of motor effect) |
| 2 | 000-255 | Overall Y-axis fine-tuning (motor effect amplitude) |
| 3 | 000-255 | Y-axis velocity |
| 4 | 000-009 010-255 | of no avail Motor Effect(Each of 10 values has one effect， for a total of 24 effects） |
| 5 | 000-255 | aiming |
| 6 | 000-255 | stroboflash |
| 7 | 000-255 | red |
| 8 | 000-255 | green |
| 9 | 000-255 | blue |
| 10 | 000-255 | white |
| 11 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 12 | 000-255 | velocity |
| 13 | 000-255 | The halo glows red |
| 14 | 000-255 | The halo green light |
| 15 | 000-255 | Halo blue |
| 16 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 17 | 000-255 | Halo mode speed |
| 18 | 000-255 | reset |

## 4.3 CH38 channel table

| channel | Channel values | basic function |
| --- | --- | --- |
| 1 | 000-255 | Y axis 1 (direction of motor effect) |
| 2 | 000-255 | Y-axis 1 fine-tuning |
| 3 | 000-255 | Y axis 2 (motor effect amplitude) |
| 4 | 000-255 | Y axis 2 fine-tuning |
| 5 | 000-255 | Axis Y 3 |
| 6 | 000-255 | Y axis 3 fine adjustment |
| 7 | 000-255 | Axis Y 4 |
| 8 | 000-255 | Y axis 4 fine-tuning |
| 9 | 000-255 | Y axis 5 |
| 10 | 000-255 | Y axis 5 fine adjustment |
| 11 | 000-255 | Y axis 6 |
| 12 | 000-255 | Y axis 6 fine adjustment |
| 13 | 000-255 | Axis Y 7 |
| 14 | 000-255 | Y axis 7 fine adjustment |
| 15 | 000-255 | Axis Y 8 |
| 16 | 000-255 | Y axis 8 fine adjustment |
| 17 | 000-255 | Y-axis velocity |
| 18 | 000-009 010-255 | of no avail Motor Effect(Each of 10 values has one effect， for a total of 24 effects） |
| 19 | 000-255 | aiming |
| 20 | 000-255 | stroboflash |
| 21 | 000-255 | red |
| 22 | 000-255 | green |
| 23 | 000-255 | blue |
| 24 | 000-255 | white |
| 25 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 26 | 000-255 | velocity |
| 27 | 000-255 | background colour |
| 28 | 000-255 | Background tone light |
| 29 | 000-255 | Hue dimming |
| 30 | 000-255 | Hertzian flicker |
| 31 | 000-255 | The halo glows red |
| 32 | 000-255 | The halo green light |
| 33 | 000-255 | Halo blue light |
| 34 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 35 | 000-255 | Halo mode speed |
| 36 | 000-255 | The halo background color |
| 37 | 000-255 | The halo has a background tone light |
| 38 | 000-255 | reset |

## 4.4 CH87 Channel Table

| channel | Channel values | basic function |
| --- | --- | --- |
| 1 | 000-255 | Y axis 1 (direction of motor effect) |
| 2 | 000-255 | Y-axis 1 fine-tuning |
| 3 | 000-255 | Y axis 2 (motor effect amplitude) |
| 4 | 000-255 | Y axis 2 fine adjustment |
| 5 | 000-255 | Axis Y 3 |
| 6 | 000-255 | Y axis 3 fine adjustment |
| 7 | 000-255 | Axis Y 4 |
| 8 | 000-255 | Y axis 4 fine tuning |
| 9 | 000-255 | Y axis 5 |
| 10 | 000-255 | Y axis 5 fine adjustment |
| 11 | 000-255 | Axis Y 6 |
| 12 | 000-255 | Y axis 6 fine adjustment |
| 13 | 000-255 | Axis Y 7 |
| 14 | 000-255 | Y axis 7 fine adjustment |
| 15 | 000-255 | Axis Y 8 |
| 16 | 000-255 | Y axis 8 fine adjustment |
| 17 | 000-255 | Y-axis velocity |
| 18 | 000-009 010-255 | of no avail Motor Effect(Each of 10 values has one effect， for a total of 24 effects） |
| 19 | 000-255 | aiming |
| 20 | 000-255 | stroboflash |
| 21 | 000-255 | Red 1 |
| 22 | 000-255 | Green 1 |
| 23 | 000-255 | Blue 1 |
| 24 | 000-255 | White 1 |
| … | … | … … |
| 49 | 000-255 | Red 8 |
| 50 | 000-255 | Green 8 |
| 51 | 000-255 | Blue 8 |
| 52 | 000-255 | White 8 |
| 53 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 54 | 000-255 | velocity |
| 55 | 000-255 | background colour |
| 56 | 000-255 | Background tone light |
| 57 | 000-255 | Hue dimming |
| 58 | 000-255 | Hertzian flicker |
| 59 | 000-255 | Ring of red light 1 |
| 60 | 000-255 | Halo green 1 |
| 61 | 000-255 | Halo Blue 1 |
| … | … | … … |
| 80 | 000-255 | Ring of Red Light 8 |
| 81 | 000-255 | Halo green light 8 |
| 82 | 000-255 | Halo Blu-ray 8 |
| 83 | 000-003 004-255 | of no avail Mode(Each of the four values has one effect， for a total of 62 effects） |
| 84 | 000-255 | Halo mode speed |
| 85 | 000-255 | The halo background color |
| 86 | 000-255 | The halo has a background tone light |
| 87 | 000-255 | reset |

# 5. Common faults

For some common faults, corresponding solutions are proposed. Any problems that cannot be solved should be handled by professionals. Before maintaining the lamp, disconnect the power supply first.

1. The lamp is not bright

 Check whether the voltage matching the lamp is installed;

 Check whether the power supply connection of the lamp or the control switch is in poor contact;

 Check whether the power supply is insufficient;

 Check whether the DMX512 controller has sent a command.

2. The lamp is not controlled by the console after normal reset

-  Check whether the digital start address value and function options of the lamp are correct;

-  Check whether the connection of the communication control line is correct, whether the communication line is too long or has been interrupted;

-  Check whether the control equipment is invalid, and check whether the signal amplifier connected in series is invalid;

-  Check whether the communication line is too long or there is interference from other equipment;

-  Optimize the wiring, shorten the length of control signal lines, and separate high voltage and low voltage lines;

-  Add signal amplifier;

-  Signal lines use high quality shielded twisted pair;

-  Connect a signal terminal resistor (120 ohms) at the end of the lamp.

3. The lamp cannot be started

 Check whether the power supply parameters are consistent with the lamp;

 Check whether the contact is poor or the lamp falls off due to compression deformation, internal parts vibration and moisture during long-distance transportation.

 Please check whether the internal wires and connectors of the lamp are loose.

 Check whether the electronic components of the lamp (such as electronic transformer, PCB board, motor control board, etc.) are loose, short circuit and burned.