1940 Outdoor Waterproof Bee Eye Shake Head  Dyeing Light

(LED Strip)

User Manual

Read the instructions carefully before use

1. Remove the packaging

To ensure proper and safe use of this product, please read the instruction manual carefully before installation. This manual contains essential installation and application information. Please strictly follow the operational steps outlined in the manual during installation and operation. Additionally, please keep this manual properly.

The 19-channel focus system is engineered and manufactured to CE standards, featuring the internationally compliant DMX512 signal protocol. It supports both standalone and networked control, delivering rapid rotation, low noise, and robust functionality. Ideal for small to medium-sized venues including concerts, theaters, studios, nightclubs, and bars.

This product features 19 high-brightness, stable 40W 4-in-1 LEDs. Handle the packaging carefully when removing it. After opening, inspect the product for any damage during transportation and verify that all components are intact.

19 staining lines---------1 unit                                   signal line--------1 unit

Manual------------1 copy                                           Power cord--------------1 piece

2. Remove the packaging

Specifications

Voltage: AC 90–240 V, 50/60 Hz

Total Power: 800 W

Light Source: 19 pcs 4-in-1 40 W high-brightness LEDs

Beam Angle: 4–60°

Strobe: 1–25 Hz, with multiple strobe effects

Dimming: 0–100% linear dimming

Channels: 4 built-in channel modes: 21 CH / 29 CH / 53 CH / 57 CH

Control: DMX512, Auto Run, Sound Active

Pan: 540° with 16-bit fine adjustment

Tilt: 270° with 16-bit fine adjustment

Display: Chinese / English

Product Size: 328 × 252 × 419 mm

N.W.: 14 kg

Carton Size: 428 × 352 × 519 mm

- Security Notice

pay attention to ！

Handle with caution. This high-voltage product may cause electric shock if it comes into contact with live wires.

This product is in perfect condition before leaving the factory. To maintain its integrity and ensure safe operation, users must follow the safety precautions and warnings in this manual. Important: Damages caused by non-compliance with this manual are not covered by warranty. The supplier shall not be liable for any product issues arising from such non-compliance. If the product has been exposed to extreme temperature fluctuations (e.g., during transportation), do not power it on immediately, as condensation from temperature changes may damage the device. Wait until the product reaches normal temperature before use.

This product is designed for indoor use and operates within a voltage range of 90-240V. Ensure the ground voltage does not exceed the product's rated tolerance. The power plug must be inserted into a Class I socket with proper protection. The green or tea-green conductor must be properly grounded.

Regularly inspect this product's power cord. Ensure it is not folded, damaged, or scratched, and avoid connecting it to other wires. Exercise special caution when connecting the power cord or related cables. Always unplug the power supply when not in use or before cleaning.

Before using the product, familiarize yourself with its operation. Do not let children or non-professionals handle the product.

Do not shake this product. Avoid using excessive force during installation or operation. Do not let unqualified personnel operate the product. Most damages are caused by improper operation.

The product is not equipped with relevant repair parts. Maintenance and repair of the product can only be performed by professionals!

Do not modify this product without authorization, as it may cause damage that is not covered by the warranty. Additionally, improper operation may lead to short circuits, burns, or electric shocks.

4. Operation Instructions

This product is a shaking dyeing beam lamp designed to create dynamic dyeing effects.

Do not start the product immediately after significant temperature changes (e.g., post-transportation), as thermal expansion and contraction may cause damage. Wait until the equipment reaches normal room temperature before initiating operation.

--Be cautious of vibrations. Avoid strong impacts during installation.

Do not lift the entire product by the lamp head, as this may cause mechanical damage to the device.

When selecting an installation location, ensure the product is not exposed to overheating, excessive moisture, or dust. Do not place any wires on the ground, as this may pose an electric shock hazard.

--Before installing the product, ensure the installation site is secure

--Please secure the product with a safety rope and check all screws during installation

--Ensure the lens is in good condition. Replace the lens if it is damaged or scratched.

--It is recommended that this product be operated by a technician familiar with it. Non-technical personnel are prohibited from operating this device, as many incidents of loss are caused by improper operation.

Store the packaging materials properly for potential secondary transportation.

Do not modify the product without the manufacturer or distributor's guidance.

Any equipment malfunction caused by improper operation of this device as specified in the instruction manual shall not be covered by warranty. Additionally, any incidents resulting from short circuits, injuries, electric shocks, ultraviolet radiation-induced harm, or bulb explosions shall not be covered by warranty.

5. Product Hanging and Installation

Note: For safety, hang and install this product in a location away from walkways, seating areas, or within easy reach.

Before hanging this product, ensure the mounting point can support up to 10 times its weight.

The product must be equipped with a double protection device, such as a safety rope.

Do not stand under the installation point when hanging, removing, or repairing this product.

Ensure this product is installed at least 0.5 meters away from flammable materials.

Always use safety ropes as a safety measure to prevent accidents caused by loose fasteners.

Safety Tip: Top-mounted installations require highly experienced installers who can calculate load-bearing requirements, select appropriate materials, and conduct periodic safety inspections. If you lack this expertise, never attempt installation yourself. Improper handling may lead to serious consequences, including personal injury.

Before powering on the product, make sure all required hanging and installation steps are completed.

Quick-lock hanging system: This product features a specially designed professional hanging mechanism at the base, including a quick-lock hanging component and a safety rope attachment point (see diagram below).

When installing this product on the truss, use the appropriate quick-release lock to secure it at the designated quick-release lock mounting point, and fasten it with M10 screws. For added safety, attach the safety rope to the product's bottom safety rope mounting point and secure it to the truss.

Regardless of the hanging method you choose, always use the safety rope. The product features a dedicated safety rope attachment point at the bottom (see the image above). Never attach the safety rope to the handle.

6.DMX-512 linkage

Connect the male end of the XLR signal cable to the controller's signal output port and the female end to the signal input port on the rear panel of this product. Multiple units can be connected in series. The cable must be three-core, shielded, and equipped with both XLR input and output connectors. Refer to the diagram below.

The DMX-512 connection requires a terminal (loop connection)

During installation, if signal cables are laid over long distances or in noisy environments such as dance halls, using a DMX terminal (circuit connection) is recommended to prevent digital control signal interference caused by electrical noise. The DMX terminal consists of a simple XLR connector with 2 or 3 cores and a 120-ohm resistor. Insert the terminal into the XLR output port of the last product, as shown in the figure below.

7. Menu

| main interface | description | explain |
| --- | --- | --- |
| temperature | 32℃ | The temperature of the light plate is 32°C. Temperature effect on power: The power is reduced from 70 degrees. The power increase only begins when the temperature drops below 68 degrees (excluding 68 degrees). The power remains constant between 68 and 69 degrees. Turn off the lamp directly at 85 degrees. |
|  | **℃ | Temperature detection abnormal |
| electric fan | whack | 1) The fan runs for 15 seconds when turned on 2) The fan activates when the light panel temperature reaches 48°C and stops only when it drops below 45°C. |
|  | Keep turning | 1) Temperature not detected 2) Communication error |
| address | 001 - 512 | Increment or decrement the address code by 1 or 1 |
| signal lamp | blue | DMX signal |
|  | red | report an error |

| primary menu | Sub-menu | Three-level menu/parameters |
| --- | --- | --- |
| System Settings | running mode | DMX/Voice/Trailer 1/Trailer 2 |
|  | channel pattern | 21CH/29CH/53CH/57CH |
|  | Horizontal Inversion | On/Off |
|  | Vertical Reversal | On/Off |
|  | Reserve feature |  |
|  | optical coupling error correction | On/Off |
|  | signal holding | On/Off |
|  | Screen Protector | On/Off |
|  | Flip screen | On/Off |
|  | Sync updates | On/Off This feature requires the DMX cable to be unplugged. |
|  | language | centre /EN |
|  | factory data reset |  |
| manual mode | Manual control of channel values | 000-255 |
| system calibration | enter password | lamp calibration |
| system reset | actuator motor reset | Reset the effect panel except XY |
|  | scan motor reset | Reset only the XY axis |
|  | Reset all motors | Lamp reset |
| system info | Reduction information | Display specific information |
|  | DMX data detection | Display specific DMX detection content |
|  | sensor information | Display specific information about optocouplers and Hall effect sensors |
|  | Hardware version | Display hardware version |
|  | software release | Display software version |

◆System Settings

| option | explain |  |
| --- | --- | --- |
| running mode | The lamp supports three operating modes: DMX mode, self-driving mode, and voice-controlled mode. |  |
|  | DMX pattern | Console mode, receiving DMX signals, RDM signals |
|  | Autopilot Mode 1 | The lamp operates automatically according to built-in program 1 |
|  | Autopilot Mode 2 | The lamp operates automatically according to built-in program 2 |
|  | Voice Mode | When the lamp detects a loud sound, it automatically runs a scene according to the built-in program; otherwise, it maintains the last scene. |
| channel pattern | Available channel values: 21CH/29CH/53CH/57CH |  |
| Horizontal Inversion | Set X-axis rotation direction |  |
|  | close | Not reverse |
|  | open | opposite direction |
| Vertical Reversal | Set Y-axis rotation direction |  |
|  | close | Not reverse |
|  | open | opposite direction |
| Reserve feature |  |  |
| optical coupling error correction | Check if XY is out of sync and correct |  |
|  | close | Failure to correct position after losing track |
|  | open | Automatic position correction after loss of step |
| signal holding | The output status of the lamp when there is no DMX signal |  |
|  | close | No signal detected, so the motor and light source reset to their initial positions and states. |
|  | open | No signal detected. Maintain the last frame of DMX data output. |
| Screen Protector | Screen on time |  |
|  | close | Always on |
|  | open | Turn off the screen automatically in static mode |
| Flip screen | Set the screen orientation |  |
|  | close | Display in reverse |
|  | open | Reverse display |
| Sync updates | Sync the setup or calibration parameters of multiple lights |  |
|  | close | Sync update is disabled. |
|  | open | Connect multiple lamps with DMX cables to synchronize information on the calibration interface. (Note: Unplug the DMX signal cable) |
| language | Switch between Chinese and English menus |  |
|  | centre | the Chinese language |
|  | EN | English menu |
| factory data reset | Lamp parameters return to factory settings |  |
|  | cancel | res sic stantibus |
|  | affirm | Restore the lamp to factory settings |

◆ manual control

This interface controls the current light. See the channel table for details.

Press OK to enter edit mode. Use the up and down keys to change channel values. Press OK again to save changes and exit edit. Press Exit to exit edit without saving changes.

◆ system calibration

A password "8888" is set here to prevent unauthorized access. Press the "OK" key to verify the password.

| option | explain |
| --- | --- |
| initial position calibration | After entering the sub-interface, you can adjust the reset positions of motors for X-axis, Y-axis, focus adjustment, and rotation to compensate for hardware installation errors. The adjustment range is 0 to 255, with 127 indicating no adjustment. |
| white balance CAL | Adjust the power of individual LEDs (1-19) and the total LED output, with a range of 0 to 255, where 255 indicates no adjustment. |
| calibration of power | Adjust the maximum power of the entire lamp. The range is 0 to 255, with 200 indicating no adjustment. |
| MIC sensitivity calibration | Adjust the voice sensitivity. |
| change password | Set the system calibration password. |

◆ reset

Press the up or down key to switch between reset modes. Press OK to reset directly.

| option | explain |
| --- | --- |
| actuator motor reset | focus adjustment, motor reset |
| scan motor reset | Reset only the XY axis |
| Reset all motors | Lamp reset |

◆ system info

| option | explain |
| --- | --- |
| Reduction error message | If the red ERR indicator light turns on, the lamp is malfunctioning. 1) MCU1 communication failure (motor and display board communication failure) 2) MCU2 communication failure (communication between motor board and light board failed) 3) X/Y optical coupler error 4) Failed rotation and focus reset |
| DMX data monitoring | Enter the sub-interface to view channel values numerically. You can navigate through pages. |
| sensor information | 1) Hall: - sensor ： - X/Y optical coupler: XXXX--XXXX |
| Hardware version | Hardware information of the lamp XX . XX . XX .XX Display board version. Motor version. Light board version. |
| Software version | Lamp software version XX . XX . XX .XX Display board version. Motor version. Light board version. |

8. Channel Table

Channel table reference to original BQ channel

| 21CH | function | DMX numeric value | function |
| --- | --- | --- | --- |
| 1 | stroboflash | 0-3 | photopigment |
|  |  | 4-103 | Synchronous Stroboscopic Lighting from Slow to Fast |
|  |  | 104-107 | opening the light |
|  |  | 108-207 | Split Frequency Strobismus from Slow to Fast |
|  |  | 208-212 | opening the light |
|  |  | 213-225 | slow random stroboscopic |
|  |  | 226-238 | medium speed random strobe |
|  |  | 239-255 | fast random strobe |
| 2 | aiming | 0-255 | Linear dimming from dark to bright 0-100% |
| 3 | amplify | 0-255 | Focus range from large to small |
| 4 | X axle | 0-255 | horizontal control |
| 5 | X-axis fine-tuning | 0-255 | level control fine adjustment |
| 6 | Y axle | 0-255 | vertical control |
| 7 | Y-axis fine-tuning | 0-255 | vertical fine-tuning control |
| 8 | macro | 0-9 | NF |
|  |  | 10-14 | All reset valid for 5 seconds |
| 9 | Color Macro | 0-9 | NF |
|  |  | 9-255 | Color Macro |
| 10 | red | 0-255 | Red linear dimming from dark to bright 0-100% |
| 11 | green | 0-255 | Linear dimming from dark to bright (0-100%) |
| 12 | blue | 0-255 | Blue linear dimming from dark to light 0-100% |
| 13 | white | 0-255 | White linear dimming from dark to bright 0-100% |
| 14 | colour temperature | 0-9 | NF |
|  |  | 10-255 | 8000K-2500K |
| 15 | halo strobismus | 0-4 | NF |
|  |  | 5-250 | The frequency of the supplementary light flicker increases from slow to fast. |
|  |  | 251-255 | NF |
| 16 | halo light dimming | 0-255 | Linear dimming from dark to bright 0-100% |
| 17 | Aurora Red | 0-255 | The halo red gradually dims from dark to bright with linear dimming 0-100% |
| 18 | Aurora Green | 0-255 | The halo green gradually dims from dark to bright with linear dimming 0-100% |
| 19 | Aurora Blue | 0-255 | The halo blue transitions linearly from dark to bright at 0-100% |
| 20 | Halo effect | 0-51 | NF |
|  |  | 52-255 | Effect 1-47 (one effect per 4 numbers) |
| 21 | halo effect speed | 0-255 | Halo effect speed from slow to fast |

| 29CH | function | DMX numeric value | function |
| --- | --- | --- | --- |
| 1 | stroboflash | 0-3 | photopigment |
|  |  | 4-103 | Synchronous Stroboscopic Lighting from Slow to Fast |
|  |  | 104-107 | opening the light |
|  |  | 108-207 | Split Frequency Strobismus from Slow to Fast |
|  |  | 208-212 | opening the light |
|  |  | 213-225 | slow random stroboscopic |
|  |  | 226-238 | medium speed random strobe |
|  |  | 239-255 | fast random strobe |
| 2 | aiming | 0-255 | Linear dimming from dark to bright 0-100% |
| 3 | dimming fine adjustment | 0-255 | Linear dimming from dark to bright 0-1% |
| 4 | amplify | 0-255 | Focus range from large to small |
| 5 | X axle | 0-255 | horizontal control |
| 6 | X-axis fine-tuning | 0-255 | level control fine adjustment |
| 7 | Y axle | 0-255 | vertical control |
| 8 | Y-axis fine-tuning | 0-255 | vertical fine-tuning control |
| 9 | macro | 0-9 | NF |
|  |  | 10-14 | All reset valid for 5 seconds |
| 10 | Color Macro | 0-9 | NF |
|  |  | 9-255 | Color Macro |
| 11 | red | 0-255 | Red linear dimming from dark to bright 0-100% |
| 12 | Red fine-tuning | 0-255 | Red fine-tuning: linear dimming from dark to bright (0-1%) |
| 13 | green | 0-255 | Linear dimming from dark to bright (0-100%) |
| 14 | Green fine-tuning | 0-255 | Green fine-tuning for linear dimming from dark to bright (0-1%) |
| 15 | blue | 0-255 | Blue linear dimming from dark to light 0-100% |
| 16 | Blue fine-tuning | 0-255 | Blue fine-tuning: linear dimming from dark to bright (0-1%) |
| 17 | white | 0-255 | White linear dimming from dark to bright 0-100% |
| 18 | White fine-tuning | 0-255 | White fine-tuning: linear dimming from dark to bright (0-1%) |
| 19 | colour temperature | 0-9 | NF |
|  |  | 10-255 | 8000K-2500K |
| 20 | effect | 0-9 | NF |
|  |  | 10-255 | Effect 1-5 (one effect per 50 numbers) |
| 21 | Effect speed | 0-63 | Static effect |
|  |  | 64-158 | From fast to slow in the forward direction |
|  |  | 159-160 | static |
|  |  | 161-255 | Reverse from slow to fast |
| 22 | Effect fade | 0-255 | Effect duration from longest to shortest |
| 23 | halo strobismus | 0-4 | NF |
|  |  | 5-250 | The frequency of the supplementary light flicker increases from slow to fast. |
|  |  | 251-255 | NF |
| 24 | halo light dimming | 0-255 | Linear dimming from dark to bright 0-100% |
| 25 | Aurora Red | 0-255 | The halo red gradually dims from dark to bright with linear dimming 0-100% |
| 26 | Aurora Green | 0-255 | The halo green gradually dims from dark to bright with linear dimming 0-100% |
| 27 | Aurora Blue | 0-255 | The halo blue transitions linearly from dark to bright at 0-100% |
| 28 | Halo effect | 0-51 | NF |
|  |  | 52-255 | Effect 1-47 (one effect per 4 numbers) |
| 29 | halo effect speed | 0-255 | Halo effect speed from slow to fast |

| 53CH | function | DMX numeric value | function |
| --- | --- | --- | --- |
| 1 | stroboflash | 0-3 | photopigment |
|  |  | 4-103 | Synchronous Stroboscopic Lighting from Slow to Fast |
|  |  | 104-107 | opening the light |
|  |  | 108-207 | Split Frequency Strobismus from Slow to Fast |
|  |  | 208-212 | opening the light |
|  |  | 213-225 | slow random stroboscopic |
|  |  | 226-238 | medium speed random strobe |
|  |  | 239-255 | fast random strobe |
| 2 | aiming | 0-255 | Linear dimming from dark to bright 0-100% |
| 3 | dimming fine adjustment | 0-255 | Linear dimming from dark to bright 0-1% |
| 4 | amplify | 0-255 | Focus range from large to small |
| 5 | X axle | 0-255 | horizontal control |
| 6 | X-axis fine-tuning | 0-255 | level control fine adjustment |
| 7 | Y axle | 0-255 | vertical control |
| 8 | Y-axis fine-tuning | 0-255 | vertical fine-tuning control |
| 9 | macro | 0-9 | NF |
|  |  | 10-14 | All reset valid for 5 seconds |
| 10 | Color Macro | 0-9 | NF |
|  |  | 9-255 | Color Macro |
| 11 | red | 0-255 | Red linear dimming from dark to bright 0-100% |
| 12 | Red fine-tuning | 0-255 | Red fine-tuning: linear dimming from dark to bright (0-1%) |
| 13 | green | 0-255 | Linear dimming from dark to bright (0-100%) |
| 14 | Green fine-tuning | 0-255 | Green fine-tuning for linear dimming from dark to bright (0-1%) |
| 15 | blue | 0-255 | Blue linear dimming from dark to light 0-100% |
| 16 | Blue fine-tuning | 0-255 | Blue fine-tuning: linear dimming from dark to bright (0-1%) |
| 17 | white | 0-255 | White linear dimming from dark to bright 0-100% |
| 18 | White fine-tuning | 0-255 | White fine-tuning: linear dimming from dark to bright (0-1%) |
| 19 | Red 1 | 0-255 | Red 1: Linear dimming from dark to bright (0-100%) |
| 20 | Red 1 fine-tuning | 0-255 | Red 1: Linear dimming from dark to bright (0-1%) |
| 21 | Green 1 | 0-255 | Green 1: Linear dimming from dark to bright (0-100%) |
| 22 | Green 1 fine-tune | 0-255 | Green 1: Linear dimming from dark to bright (0-1%) |
| 23 | Blue 1 | 0-255 | Blue 1: Linear dimming from dark to light (0-100%) |
| 24 | Blue 1 Fine Adjustment | 0-255 | Blue 1: Linear dimming from dark to bright (0-1%) |
| 25 | White 1 | 0-255 | White 1: Linear dimming from dark to bright (0-100%) |
| 26 | White 1 fine-tuning | 0-255 | White 1: Linear dimming from dark to bright (0-1%) |
| 27 | Red 2 | 0-255 | Red 2: Linear dimming from dark to bright (0-100%) |
| 28 | Red 2 fine-tuning | 0-255 | Red 2: Linear dimming from dark to bright (0-1%) |
| 29 | Green 2 | 0-255 | Green 2: Linear dimming from dark to bright (0-100%) |
| 30 | Green 2 Fine-tuning | 0-255 | Green 2: Linear dimming from dark to bright (0-1%) |
| 31 | Blue 2 | 0-255 | Blue 2: Linear dimming from dark to bright (0-100%) |
| 32 | Blue 2 Fine-Tune | 0-255 | Blue 2: Linear dimming from dark to bright (0-1%) |
| 33 | White 2 | 0-255 | White 2: Linear dimming from dark to bright (0-100%) |
| 34 | White 2 fine-tuning | 0-255 | Adjust the white 2 from dark to bright with linear dimming 0-1% |
| 35 | Red 3 | 0-255 | Red 3: Linear dimming from dark to bright (0-100%) |
| 36 | Red 3 fine-tuning | 0-255 | Red 3: Linear dimming from dark to bright (0-1%) |
| 37 | Green 3 | 0-255 | Green 3: Linear dimming from dark to bright (0-100%) |
| 38 | Green 3 Fine-tuning | 0-255 | Green 3: Linear dimming from dark to bright (0-1%) |
| 39 | Blue 3 | 0-255 | Blue 3: Linear dimming from dark to bright (0-100%) |
| 40 | Blue 3 Fine Adjustment | 0-255 | Blue 3: Linear dimming from dark to bright (0-1%) |
| 41 | White 3 | 0-255 | White 3: Linear dimming from dark to bright (0-100%) |
| 42 | White 3 fine-tuning | 0-255 | White 3: Linear dimming from dark to bright (0-1%) |
| 43 | colour temperature | 0-9 | NF |
|  |  | 10-255 | 8000K-2500K |
| 44 | effect | 0-9 | NF |
|  |  | 10-255 | Effect 1-5 (one effect per 50 numbers) |
| 45 | Effect speed | 0-63 | Static effect |
|  |  | 64-158 | From fast to slow in the forward direction |
|  |  | 159-160 | static |
|  |  | 161-255 | Reverse from slow to fast |
| 46 | Effect fade | 0-255 | Effect duration from longest to shortest |
| 47 | halo strobismus | 0-4 | NF |
|  |  | 5-250 | The frequency of the supplementary light flicker increases from slow to fast. |
|  |  | 251-255 | NF |
| 48 | halo light dimming | 0-255 | Linear dimming from dark to bright 0-100% |
| 49 | Aurora Red | 0-255 | The halo red gradually dims from dark to bright with linear dimming 0-100% |
| 50 | Aurora Green | 0-255 | The halo green gradually dims from dark to bright with linear dimming 0-100% |
| 51 | Aurora Blue | 0-255 | The halo blue transitions linearly from dark to bright at 0-100% |
| 52 | Halo effect | 0-51 | NF |
|  |  | 52-255 | Effect 1-47 (one effect per 4 numbers) |
| 53 | halo effect speed | 0-255 | Halo effect speed from slow to fast |

| 57CH | function | DMX numeric value | function |
| --- | --- | --- | --- |
| 1 | stroboflash | 0-3 | photopigment |
|  |  | 4-103 | Synchronous Stroboscopic Lighting from Slow to Fast |
|  |  | 104-107 | opening the light |
|  |  | 108-207 | Split Frequency Strobismus from Slow to Fast |
|  |  | 208-212 | opening the light |
|  |  | 213-225 | slow random stroboscopic |
|  |  | 226-238 | medium speed random strobe |
|  |  | 239-255 | fast random strobe |
| 2 | aiming | 0-255 | Linear dimming from dark to bright 0-100% |
| 3 | dimming fine adjustment | 0-255 | Linear dimming from dark to bright 0-1% |
| 4 | amplify | 0-255 | Focus range from large to small |
| 5 | X axle | 0-255 | horizontal control |
| 6 | X-axis fine-tuning | 0-255 | level control fine adjustment |
| 7 | Y axle | 0-255 | vertical control |
| 8 | Y-axis fine-tuning | 0-255 | vertical fine-tuning control |
| 9 | macro | 0-9 | NF |
|  |  | 10-14 | All reset valid for 5 seconds |
| 10 | Color Macro | 0-9 | NF |
|  |  | 9-255 | Color Macro |
| 11 | red | 0-255 | Red linear dimming from dark to bright 0-100% |
| 12 | Red fine-tuning | 0-255 | Red fine-tuning: linear dimming from dark to bright (0-1%) |
| 13 | green | 0-255 | Linear dimming from dark to bright (0-100%) |
| 14 | Green fine-tuning | 0-255 | Green fine-tuning for linear dimming from dark to bright (0-1%) |
| 15 | blue | 0-255 | Blue linear dimming from dark to light 0-100% |
| 16 | Blue fine-tuning | 0-255 | Blue fine-tuning: linear dimming from dark to bright (0-1%) |
| 17 | white | 0-255 | White linear dimming from dark to bright 0-100% |
| 18 | White fine-tuning | 0-255 | White fine-tuning: linear dimming from dark to bright (0-1%) |
| 19 | Red 1 | 0-255 | Red 1: Linear dimming from dark to bright (0-100%) |
| 20 | Red 1 fine-tuning | 0-255 | Red 1: Linear dimming from dark to bright (0-1%) |
| 21 | Green 1 | 0-255 | Green 1: Linear dimming from dark to bright (0-100%) |
| 22 | Green 1 fine-tune | 0-255 | Green 1: Linear dimming from dark to bright (0-1%) |
| 23 | Blue 1 | 0-255 | Blue 1: Linear dimming from dark to light (0-100%) |
| 24 | Blue 1 Fine Adjustment | 0-255 | Blue 1: Linear dimming from dark to bright (0-1%) |
| 25 | White 1 | 0-255 | White 1: Linear dimming from dark to bright (0-100%) |
| 26 | White 1 fine-tuning | 0-255 | White 1: Linear dimming from dark to bright (0-1%) |
| 27 | Red 2 | 0-255 | Red 2: Linear dimming from dark to bright (0-100%) |
| 28 | Red 2 fine-tuning | 0-255 | Red 2: Linear dimming from dark to bright (0-1%) |
| 29 | Green 2 | 0-255 | Green 2: Linear dimming from dark to bright (0-100%) |
| 30 | Green 2 Fine-tuning | 0-255 | Green 2: Linear dimming from dark to bright (0-1%) |
| 31 | Blue 2 | 0-255 | Blue 2: Linear dimming from dark to bright (0-100%) |
| 32 | Blue 2 Fine-Tune | 0-255 | Blue 2: Linear dimming from dark to bright (0-1%) |
| 33 | White 2 | 0-255 | White 2: Linear dimming from dark to bright (0-100%) |
| 34 | White 2 fine-tuning | 0-255 | Adjust the white 2 from dark to bright with linear dimming 0-1% |
| 35 | Red 3 | 0-255 | Red 3: Linear dimming from dark to bright (0-100%) |
| 36 | Red 3 fine-tuning | 0-255 | Red 3: Linear dimming from dark to bright (0-1%) |
| 37 | Green 3 | 0-255 | Green 3: Linear dimming from dark to bright (0-100%) |
| 38 | Green 3 Fine-tuning | 0-255 | Green 3: Linear dimming from dark to bright (0-1%) |
| 39 | Blue 3 | 0-255 | Blue 3: Linear dimming from dark to bright (0-100%) |
| 40 | Blue 3 Fine Adjustment | 0-255 | Blue 3: Linear dimming from dark to bright (0-1%) |
| 41 | White 3 | 0-255 | White 3: Linear dimming from dark to bright (0-100%) |
| 42 | White 3 fine-tuning | 0-255 | White 3: Linear dimming from dark to bright (0-1%) |
| 43 | colour temperature | 0-9 | NF |
|  |  | 10-255 | 8000K-2500K |
| 44 | effect | 0-9 | NF |
|  |  | 10-255 | Effect 1-5 (one effect per 50 numbers) |
| 45 | Effect speed | 0-63 | Static effect |
|  |  | 64-158 | From fast to slow in the forward direction |
|  |  | 159-160 | static |
|  |  | 161-255 | Reverse from slow to fast |
| 46 | Effect fade | 0-255 | Effect duration from longest to shortest |
| 47 | Effect Background Color | 0-255 | Effect background color: Linear dimming from dark to light 0-100% |
| 48 | Background color | 0-255 | Effect background color: Linear dimming from dark to light 0-100% |
| 49 | Effect Background Color | 0-255 | Effect background color: Linear dimming from dark to light 0-100% |
| 50 | Effect Background White | 0-255 | Effect: Background white linearly dims from dark to light 0-100% |
| 51 | halo strobismus | 0-4 | NF |
|  |  | 5-250 | The frequency of the supplementary light flicker increases from slow to fast. |
|  |  | 251-255 | NF |
| 52 | halo light dimming | 0-255 | Linear dimming from dark to bright 0-100% |
| 53 | Aurora Red | 0-255 | The halo red gradually dims from dark to bright with linear dimming 0-100% |
| 54 | Aurora Green | 0-255 | The halo green gradually dims from dark to bright with linear dimming 0-100% |
| 55 | Aurora Blue | 0-255 | The halo blue transitions linearly from dark to bright at 0-100% |
| 56 | Halo effect | 0-51 | NF |
|  |  | 52-255 | Effect 1-47 (one effect per 4 numbers) |
| 57 | halo effect speed | 0-255 | Halo effect speed from slow to fast |

9. Product Maintenance and Cleaning

When testing this product, please note the following points:

- All screws and components of installed products must be securely fastened and rust-free.

- The housing, lenses, fasteners, and mounting locations (e.g., ceilings, brackets, trusses, etc.) must remain undistorted.

- The moving parts of the machine should not be worn out or rotate unbalance.

- The power cord must be free from any damage, including material peeling or scratches.

The installation and use of the product must be performed by trained professional technicians to ensure the elimination of safety hazards.

Note: Please unplug the power cord before product maintenance!!

To maintain optimal product condition and prolong its service life, we recommend routine cleaning.

1. Clean both the external and internal lenses weekly to prevent dust buildup that could reduce the product's light output.

2. Clean the fan every week.

3. Every three months, a certified electrical engineer conducts a thorough circuit inspection to ensure proper connections, preventing aging caused by overheating.

We recommend regular cleaning of the product. Please note that do not use damp or linen cloths, and absolutely avoid using alcohol or solvents for cleaning.

This product does not include replacement parts. Refer to the "Installation Instructions" section.

If you need any spare parts, purchase them from your local dealer.