- 4-color DMX channel menu

- AH40 DH41

8-channel mode (A001 state)

| passage | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | R dimming | Red dimming, from dark to bright. |
| CH3 | G dimming | Green dimming, from dark to bright |
| CH4 | B dimming | Blue dimming, from dark to bright |
| CH5 | W dimming | White dimming, from dark to bright |
| CH6 | Total stroboscopic | Global stroboscopic, from slow to fast |
| CH7 | FS | 0: Dimming mode; 1-62: color selection (selected by CH8); 63-124: saltus step; 125-186: gradual change; 187-249: pulse change; 250-255: Voice control (CH8 selects voice control 1 and 2) |
| CH8 | Functional speed | Function speed, from slow to fast (color selection for color output, sound control strobe and jump for voice control) |

8-channel mode (d001)

| passage | function | explain |
| --- | --- | --- |
| CH1 | Total dimming | Total dimming, from dark to bright |
| CH2 | Total stroboscopic | Global stroboscopic, from slow to fast |
| CH3 | FS | 0: Dimming mode; 1-62: color selection (selected by CH4); 63-124: Jump; 125-186: gradual change; 187-249 pulse change; 250-255: Voice control (CH4 selects voice control 1 and 2) |
| CH4 | Functional speed | Function speed, from slow to fast (color selection for color output, sound control strobe and jump for voice control) |
| CH5 | R dimming | Red dimming, from dark to bright. |
| CH6 | G dimming | Green dimming, from dark to bright |
| CH7 | B dimming | Blue light, from dark to bright |
| CH8 | W dimming | White dimming, from dark to bright |

Second, the digital tube display menu description

| serial number | content | scale | Function and description |
| --- | --- | --- | --- |
| one | A001 | 001- 512 | DMX512 address code setting -8-channel mode |
| 2 | d001 | 001- 512 | DMX512 address code setting -8-channel mode |
| three | r255 | 000 - 255 | Red dimming, from dark to bright. |
| four | G255 | 000 - 255 | Green dimming, from dark to bright. |
| five | b255 | 000 - 255 | Blue dimming, from dark to bright. |
| six | u255 | 000 - 255 | White dimming, from dark to bright. |
| seven | FH99 | 01 - 99 | Strobe, speed from slow to fast. |
| eight | CL01 | 01 - 15 | Manual color selection mode. |
| nine | CC99 | 01 - 99 | Jump from slow to fast. |
| 10 | DE99 | 01 - 99 | Gradient, from slow to fast. |
| 11 | CP99 | 01 - 99 | Pulse changes from slow to fast. |
| 12 | SU01 | 01 - 02 | Mode: 1 is stroboscopic jump, 2 is jump. |

- Third, the master-slave machine description

Automatically receive the slave signal in d001 or A001 state, without setting it separately.

- Types of applicable lamps and lanterns

- 1,4-eye COB surface light lamp ()

- 2, 18 24 4-in-1 color surface lights ()

- 3,2-eye bicolor COB surface light lamp ()