LED operating instructions for stage lighting equipment

(Please read the instructions carefully and carefully before using the product)

For proper and safe use, read the instructions carefully before installing and using the product. This specification contains important installation and application information, please strictly follow the instructions when installing and operating the product. At the same time, please keep this instruction manual properly.

Our 230W beam lamp uses a new and beautiful high-temperature resistant metal body. This product strictly follows the CE standard for design and production, in line with the international standard DMX512 signal protocol, can be used separately for control, can also be used online, with fast rotation, low noise, powerful features, suitable for small and medium-sized concerts, theaters, studios, nightclubs and bars and other places,

Please remove the package carefully, check whether the product is damaged during transportation, and check whether the following contents are complete.

Beam lamp- - - - - - - - -1 set             Instruction manual- - - - - - - -1 version

Power cord- - - - - - - - -1 piece             Signal line- - - - - - - - -1 bar

This product is a good product before delivery. In order to keep the product in good condition and ensure safe operation, users should follow the safety matters and the instructions.

Important: Damage caused by not following this instruction is not covered by the warranty. The supplier is not responsible for the product problems arising from this.

If the product has been exposed to extreme unstable temperature (e. g. after transportation), do not power the product immediately because water drops due to temperature changes may damage the product. Please use it after the product has returned to the normal temperature.

This product can be used in the voltage range of 90-240V, and is used for indoor products. Please confirm that the voltage used is not above the tolerable range of the product!! The power plug must be plugged into a protective class I socket. Green or tea-cyan conductors must be grounded.

Please check the product and the power cord frequently. Ensure that the power cord is not folded or damaged and not connected to other wires! Special attention is required when connecting the power cord or the related wiring. Unplug the power supply without using the product or before cleaning.

Before using the product, please be familiar with the operation function of the product. Please do not expose children or no professionals to the product. Please do not shake this product. Do not use brute force when installing or operating the product. Do not allow unprofessionals to operate the product. Most of the damage is caused by an unprofessional operation.

The product is not equipped with relevant repair accessories. The maintenance and repair of the products can only be carried out by professionals!

Please do not change this product without authorization, otherwise the product may be damaged, so the damage caused is not covered by the warranty. Also, unprofessional operations can lead to short circuits, burns or electric shocks, and so on.

DMX512 Connection of the signal:

This lamp uses DMX512 signal control mode, and the control signal of each lamp is in parallel. When connecting the signals of multiple lamps, it is best to use dual-core shielding cable. When connecting, each lamp is connected through the DMX signal jack (Canon seat) INPUT (input) and OUTPUT (output). The 3-core XLR plug terminal of the signal line connecting the lamp must correspond to each other. When connecting the lamp signal, it is recommended to use the DMX signal terminal. It is possible to destroy the control signal due to electrical noise. The DMX signal terminal is a 120 ohm 1W resistance between the 2 and 3 feet of an XLR plug and connects it to the OUTPUT (output) jack of the last lamp.

Calculation method of lamp starting address code:

The starting address code of the current lamp is equal to (the starting address code of the previous lamp) + (number of channels of the lamp) description:

1: Start address code value of the first lamp A001.

2: The basic number of controller should be greater than or equal to the total number of lighting channels.

3: Note: when any controller is used, each lamp must have its own starting address code. If the starting address code of the first lamp is set A001, the pass number of the lamp is 16 CH; then the starting address code of the second lamp is set to A017; the starting address code of the third lamp is set to A033; and so on, (this setting mode should be determined by different console).

Product Parameter:

Input voltage: AC100V-220V 50 / 60Hz

Power: 450W

Color plate: 12 colors + white light

Colorful color: six-color plate

Fixed pattern: 13 patterns + white light

Prism: a single prism

Aization: six color rainbow + atomization integration

Horizontal / vertical: 540° horizontal scan, 270° vertical scan

Display interface: color screen 1.8 inch button, 180° reverse display can be switched in Chinese and English

Protection level: IP20

| Menu Action Settings |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| primary menu |  | Submenu one |  | Function Settings |  | Functional profile |
| DMX address |  |  |  | 1-512 |  |  |
| System Settings |  | Show upside down |  | normal |  | . |
|  |  |  |  | overturn |  |  |
|  |  | Language setting |  | the Chinese language |  |  |
|  |  |  |  | English |  |  |
|  |  | channel pattern |  | 20CH |  |  |
|  |  | running mode |  | DMX |  |  |
|  |  |  |  | sound control |  |  |
|  |  |  |  | voluntarily |  |  |
|  |  | Electronic lighting |  | close |  |  |
|  |  |  |  | open |  |  |
|  |  | The bulb default |  | Turn on _ Close |  |  |
|  |  | Bulb control |  | illume |  |  |
|  |  | optimal path |  | Turn on _ Close |  |  |
|  |  | Horizontal reverse |  | close |  |  |
|  |  |  |  | open |  |  |
|  |  | Vertical reverse |  | close |  |  |
|  |  |  |  | open |  |  |
|  |  | XY velocity |  | low speed |  |  |
|  |  |  |  | intermediate speed |  |  |
|  |  |  |  | fast |  |  |
|  |  | Optical coupling detection |  | close |  |  |
|  |  |  |  | open |  |  |
|  |  | Return to the superior |  | ▲ |  |  |
| information services |  | total time |  | 0-9999 |  |  |
|  |  | Light bulb time |  | 0-9999 |  |  |
|  |  | binding hours |  | 0-9999 |  |  |
|  |  | display software |  | Vx001 |  |  |
|  |  | XY software |  | Vx001 |  |  |
|  |  | Small motor software |  | Vx001 |  |  |
|  |  | Return to the superior |  | ▲ |  |  |
| Channel control |  | Color hovering |  | 0-255 |  |  |
|  |  | stroboflash |  | 0-255 |  |  |
|  |  | aiming" |  | 0-255 |  |  |
|  |  | Fixed pattern |  | 0-255 |  |  |
|  |  | prism |  | 0-255 |  |  |
|  |  | Prism rotation |  | 0-255 |  |  |
|  |  | atomization |  | 0-255 |  |  |
|  |  | focus |  | 0-255 |  |  |
|  |  | Horizontal rotation |  | 0-255 |  |  |
|  |  | Horizontal fine-tuning |  | 0-255 |  |  |
|  |  | vertical turn |  | 0-255 |  |  |
|  |  | Vertical fine-tuning |  | 0-255 |  |  |
|  |  | reset |  | 0-255 |  |  |
|  |  | light up |  | 0-255 |  |  |
|  |  | XY velocity |  | 0-255 |  |  |
|  |  | Color disk speed |  | 0-255 |  |  |
|  |  | Zoom speed |  | 0-255 |  |  |
|  |  | pattern velocity |  | 0-255 |  |  |
|  |  | Return to the superior |  | 0-255 |  |  |
| operative mode |  | XY communication |  | normal |  | current state |
|  |  |  |  | wrong |  |  |
|  |  | Small motor communication |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | Focus hall |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | Color plate hall |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | Pattern hall |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | X optocoupler |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | Y optocoupler |  | normal |  |  |
|  |  |  |  | wrong |  |  |
|  |  | Fan speed |  | 6500 |  |  |
|  |  | Return to the superior |  | ▲ |  |  |
| auxiliary neans |  | Calibration code 1 |  | 0-9999 |  |  |
|  |  | total time |  | zero clearing |  |  |
|  |  |  |  | carry out |  |  |
|  |  | Light gun time |  | zero clearing |  |  |
|  |  |  |  | carry out |  |  |
|  |  | Restore the default |  | give tacit consent to |  |  |
|  |  |  |  | recover |  |  |
|  |  |  |  | preserve |  |  |
|  |  | Horizontal rotation |  | 0-255 |  | Zero-bit initial bits |
|  |  | vertical turn |  | 0-255 |  |  |
|  |  | focus |  | 0-255 |  |  |
|  |  | Prism # # 1 |  | 0-255 |  |  |
|  |  | Prism mirror 2 |  | 0-255 |  |  |
|  |  | Prism rotation 1 |  | 0-255 |  |  |
|  |  | Prism rotation 2 |  | 0-255 |  |  |
|  |  | atomization |  | 0-255 |  |  |
|  |  | Color combination |  | 0-255 |  |  |
|  |  | Fixed pattern |  | 0-255 |  |  |
|  |  | colour disk |  | 0-255 |  |  |
|  |  | stroboflash |  | 0-255 |  |  |
|  |  | Flash midpoint |  | 0-255 |  |  |
|  |  | Prism a point |  | 0-255 |  |  |
|  |  | Prism two points |  | 0-255 |  |  |
|  |  | Fog midpoint |  | 0-255 |  |  |
|  |  | Colorful midpoint |  | 0-255 |  |  |
|  |  | Return to the superior |  | ▲ |  |  |
| advanced setup |  | Check code 2 |  | 0-9999 |  |  |
|  |  | Signal encryption |  | open |  |  |
|  |  |  |  | close |  |  |
|  |  | DMX velocity |  | 0-255 |  |  |
|  |  | DMX data packet |  | 0-255 |  |  |
|  |  | DMX type |  | 0-055 |  |  |
|  |  | DMX insulate |  | 0-65535 |  | A smaller value would indicate a more stable signal |
|  |  | Drive plate temperature |  | <110 Degrees> |  | Red appears beyond this range |
|  |  | Drive board is 3.3V |  | 3.0V - 3.6V |  | Red appears beyond this range |
|  |  | Drive board: 5.0V |  | 4.5V - 5.5V |  | Red appears beyond this range |
|  |  | Drive board 12V |  | 10.0V - 14.0V |  | Red appears beyond this range |
|  |  | Drive board 36V |  | 20.0V - 42.0V |  | Red appears beyond this range |
|  |  | X state |  | static |  |  |
|  |  |  |  | turn |  |  |
|  |  | X Hoare |  | normal |  |  |
|  |  |  |  | wrong |  | Show red |
|  |  | X-light coupling phase |  | 0-3 |  |  |
|  |  | X optocoupler A |  | 40-160 |  | Standard setting 100 shows red beyond the required range, but the panel red light does not alarm |
|  |  | X optocoupler B |  | 40-160 |  | Standard setting 100 shows red beyond the required range, but the panel red light does not alarm |
|  |  | X optocoupler C |  | 40-160 |  | Standard setting 100 shows red beyond the required range, but the panel red light does not alarm |
|  |  | X optocoupler D |  | 40-160 |  | Standard setting 100 shows red beyond the required range, but the panel red light does not alarm |
|  |  | X Current Angle |  | 000-540 |  |  |
|  |  | X drive state |  | normal |  |  |
|  |  |  |  | wrong |  | Show red |
|  |  | Y state |  | static |  |  |
|  |  |  |  | turn |  |  |
|  |  | Y optocoupler |  | normal |  |  |
|  |  |  |  | wrong |  | Show red |
|  |  | Y Hoare |  | normal |  |  |
|  |  |  |  | wrong |  | Show red |
|  |  | Y light coupling phase |  | 0-3 |  |  |
|  |  | Y optocoupler A |  | 40-160 |  | Red appears beyond this range |
|  |  | Y optocoupler B |  | 40-160 |  | Red appears beyond this range |
|  |  | Y optocoupler C |  | 40-160 |  | Red appears beyond this range |
|  |  | Y optocoupler D |  | 40-160 |  | Red appears beyond this range |
|  |  | Y Current Angle |  | 000-250 |  |  |
|  |  | Y drive state |  | normal |  |  |
|  |  |  |  | wrong |  | Show red |
|  |  | Return to the superior |  | ▲ |  |  |
| Channel level |  | Color hovering |  | 0-255 |  | Current DMX channel value |
|  |  | stroboflash |  | 0-255 |  |  |
|  |  | aiming |  | 0-255 |  |  |
|  |  | Fixed pattern |  | 0-255 |  |  |
|  |  | prism |  | 0-255 |  |  |
|  |  | Prism rotation |  | 0-255 |  |  |
|  |  | atomization |  | 0-255 |  |  |
|  |  | focus |  | 0-255 |  |  |
|  |  | Horizontal rotation |  | 0-255 |  |  |
|  |  | Horizontal fine-tuning |  | 0-255 |  |  |
|  |  | vertical turn |  | 0-255 |  |  |
|  |  | Vertical fine-tuning |  | 0-255 |  |  |
|  |  | reset |  | 0-255 |  |  |
|  |  | light up |  | 0-288 |  |  |
|  |  | XY velocity |  | 0-255 |  |  |
|  |  | Color disk speed |  | 0-255 |  |  |
|  |  | Zoom speed |  | 0-255 |  |  |
|  |  | pattern velocity |  | 0-255 |  |  |
|  |  | Return to the superior |  |  |  |  |
| reset |  | Full reset |  | cancel |  | manual reset |
|  |  |  |  | carry out |  |  |
|  |  | XY reset |  | cancel |  |  |
|  |  |  |  | carry out |  |  |
|  |  | Effect reset" |  | cancel |  |  |
|  |  |  |  | carry out |  |  |
|  |  | Return to the superior |  |  |  |  |
| Exit menu |  |  |  |  |  |  |
| 16 Channel table |  |  |  |  |  |  |
| CH1 | pigment |  | 0--4 |  | white light |  |
|  |  |  | 5--9 |  | White light + color 1 |  |
|  |  |  | 10--14 |  | Color 1 |  |
|  |  |  | 15--19 |  | Color # 1 + color # 2 |  |
|  |  |  | 20--24 |  | Color 2 |  |
|  |  |  | 25--29 |  | Color # 2 + color # # 3 |  |
|  |  |  | 30--34 |  | Color 3 |  |
|  |  |  | 35--39 |  | Color # 3 + color # # 4 |  |
|  |  |  | 40--44 |  | Color 4 |  |
|  |  |  | 45-49 |  | Color # 4 + color # # 5 |  |
|  |  |  | 50--54 |  | Color 5 |  |
|  |  |  | 55--59 |  | Color # 5 + color # # 6 |  |
|  |  |  | 60--64 |  | Color 6 |  |
|  |  |  | 65--69 |  | Color # 6 + color # # 7 |  |
|  |  |  | 70--74 |  | Color 7 |  |
|  |  |  | 75--79 |  | Color # 7 + color # # 8 |  |
|  |  |  | 80--84 |  | Color 8 |  |
|  |  |  | 85--89 |  | Color # 8 + color # # 9 |  |
|  |  |  | 90--94 |  | Color 9 |  |
|  |  |  | 95--99 |  | Colour 9 + color 10 |  |
|  |  |  | 100--104 |  | Color 10 |  |
|  |  |  | 105--109 |  | Colour 10 + color 11 |  |
|  |  |  | 110--114 |  | Color 11 |  |
|  |  |  | 115--119 |  | Colour 11 + color 12 |  |
|  |  |  | 120--124 |  | Color 12 |  |
|  |  |  | 125--129 |  | Colour 12 + color 13 |  |
|  |  |  | 130--135 |  | white light |  |
|  |  |  | 136--195 |  | From fast to slow forward flowing water |  |
|  |  |  | 196--255 |  | From fast to slow reverse flow water |  |
| CH2 | stroboflash |  | 0-5 |  | The light switch is closed |  |
|  |  |  | 6--20 |  | Open the flash |  |
|  |  |  | 21-70 |  | Synchronize the strobe slow to fast |  |
|  |  |  | 71-100 |  | Random strobe is slow to fast |  |
|  |  |  | 101-150 |  | Pulse strobe is slow to fast |  |
|  |  |  | 151-190 |  | The open pulse is slow to fast |  |
|  |  |  | 191-230 |  | Turn-off pulse frequency strobe is slow to fast |  |
|  |  |  | 231-255 |  | The light switch is opened |  |
| CH3 | aiming |  | 0-255 |  | With 0-100% dimming |  |
| CH4 | pattern |  | 0--4 |  | White aperture |  |
|  |  |  | 5--9 |  | Pattern 1 |  |
|  |  |  | 10--14 |  | Pattern 2 |  |
|  |  |  | 15--19 |  | Pattern 3 |  |
|  |  |  | 20--24 |  | Pattern 4 |  |
|  |  |  | 25--29 |  | Pattern 5 |  |
|  |  |  | 30--34 |  | Pattern 6 |  |
|  |  |  | 35--39 |  | Pattern 7 |  |
|  |  |  | 40--44 |  | Pattern 8 |  |
|  |  |  | 45-49 |  | Pattern 9 |  |
|  |  |  | 50--54 |  | Pattern 10 |  |
|  |  |  | 55--59 |  | Pattern 11 |  |
|  |  |  | 60--64 |  | Pattern 12 |  |
|  |  |  | 65--69 |  | Pattern 13 |  |
|  |  |  | 70-125 |  | From fast to slow counterclockwise flow water |  |
|  |  |  | 126-130 |  | Stop running water |  |
|  |  |  | 131-190 |  | From slow to fast counterclockwise flow water |  |
|  |  |  | 191-195 |  | Pattern 1 shakes from slow to fast |  |
|  |  |  | 196-200 |  | Pattern 2 shakes from slow to fast |  |
|  |  |  | 201-205 |  | Pattern 3 shakes from slow to fast |  |
|  |  |  | 206-210 |  | Pattern 4 shakes from slow to fast |  |
|  |  |  | 211-215 |  | Pattern 5 shakes from slow to fast |  |
|  |  |  | 216-220 |  | Pattern 6 shakes from slow to fast |  |
|  |  |  | 221-225 |  | Pattern 7 shakes from slow to fast |  |
|  |  |  | 226-230 |  | Pattern 8 shakes from slow to fast |  |
|  |  |  | 231-235 |  | Pattern 9 shakes from slow to fast |  |
|  |  |  | 236-240 |  | Pattern 10 shakes from slow to fast |  |
|  |  |  | 241-245 |  | Pattern 11 shakes from slow to fast |  |
|  |  |  | 246-250 |  | Pattern 12 from slow to fast |  |
|  |  |  | 251-255 |  | Pattern 13 with a jitter ranging from slow to fast |  |
| CH5 | prism |  | 0-127 |  | Remove the prism |  |
|  |  |  | 128-255 |  | Prism cut in |  |
| CH6 | Prism rotation |  | 0-127 |  | 0-400 degrees |  |
|  |  |  | 128-189 |  | From fast to slow reverse flow water |  |
|  |  |  | 190-194 |  | cease |  |
|  |  |  | 195-255 |  | From slow to fast forward flowing water |  |
| CH7 | Reserved empty |  |  |  |  |  |
| CH8 | Fog colorful |  | 0-127 |  |  |  |
|  |  |  | 128-169 |  | Insert atomization |  |
|  |  |  | 170-255 |  | Insert colorful |  |
| CH9 | focus |  | 0-255 |  | From far to near |  |
| CH10 | X axle |  | 0-255 |  | 0-540 degrees |  |
| CH11 | X axis fine-tuning |  | 0-255 |  | The X-axis 16bit fine-tuning channel |  |
| CH12 | Y axle |  | 0-255 |  | 0-240 degrees |  |
| CH13 | Y axis fine-tuning |  | 0-255 |  | Y axis 16bit tuning channel |  |
| CH14 | XY velocity |  | 0-255 |  |  |  |
| CH15 | Reserved empty |  | 0-255 |  | NC |  |
| CH16 | Light reset |  | 0-99 |  | NF |  |
|  |  |  | 100-105 |  | Off-bulb (5S) |  |
|  |  |  | 106-199 |  | NF |  |
|  |  |  | 200-205 |  | Open the light bulb (5S) |  |
|  |  |  | 206-249 |  | NF |  |
|  |  |  | 250-255 |  | All motor reset (5S) |  |

Instructions for lamp installation:

This lamp can be placed horizontally, oblique hanging and upside down, oblique hanging and upside down must pay attention to the installation method. Fixed installation of lamps: before the positioning of lamps, ensure the stability of the installation site, the lamps must not fall off the support frame, and use the support frame and the safety rope for auxiliary hanging; to ensure safety. Prevent the falling and sliding of the lamps and lanterns. During the installation and debugging, the safety rope is worn and the hook screw is loose. If the hanging installation is not stable, and all the consequences of the falling, the manufacturer will not bear any responsibility.

after-sale service

pay attention to! When the lamps leave the factory, they are passed by strict quality inspection, and the packaging is in good condition. Please operate according to the instructions. Machine failure caused by human causes is not covered by the warranty.

- The company provides technical advice to our clients for life.

- If the product is faulty and needs to be repaired, please show the product warranty card and fill in the relevant content truthfully. At the same time, we also hope that customers can feedback the problems of the product to us in time, so as to facilitate us to improve the product in the first time