Beam shaking lamp

Use to explain the book.

Please read the instructions carefully before use.

Catalogue

1.Precautions and installation 1

1.1Declaration 1

1.2Maintenance 1

1.3Product precautions 1

1.4Product Instructions 1

1.5 Connecting signal cables 2

1.6Lighting installation 2

2.Control panel 4

2.1 Key instructions

2.2.1Setting 5

2.2.2Manual 5

2.2.3Information 6

2.2.4Factory 7

3.Channel function 8

3.1Channel table 8

4.Common faults 11

## 1. Precautions and Installation Precautions and Installation

  - statement

Thank you for choosing our products! This product has good performance and complete packaging when it leaves the factory. For your safe and effective use of this product, please read this instruction manual carefully and completely before you use this product. This manual contains important information about installation and use. Please install and operate according to the requirements of the manual. At the same time, please keep this manual properly for use at any time. Our company is not responsible for any damage to lamps or other performance due to personal failure to follow the instructions during installation, use and maintenance.

This manual is subject to technical changes without prior notice.

  - maintaining

- Please disconnect the power supply before carrying out maintenance.

- This lamp should be kept dry to avoid working in wet environment.

- Intermittent use will effectively prolong the life of this lamp.

- In order to obtain good ventilation effect and lighting effect, attention should be paid to cleaning fans, fan nets and lenses frequently.

- Please don't use alcohol and other organic solvents to wipe the lamp shell to avoid damage.

  - Precautions for products

- This lamp is for professional use only.

- Before operation, ensure that the power supply voltage is consistent with the power supply voltage required by the equipment.

- Do not place this product in a place that is easy to loosen or vibrate.

- During use, if the lamps are abnormal, stop using them in time.

- In order to ensure the service life of the product, this product should not be placed in a damp or leaking place, and it should not work in an environment where the temperature exceeds 60 degrees.

- When the light bulb is used, the change of power supply voltage should not exceed 10%. If the voltage is too high, the life of the light bulb will be shortened, and if the voltage is too low, the light color of the light bulb will be affected.

- After the power is cut off, it takes 20 minutes to fully cool the lamps before they can be powered on again.

- The rotating parts and pasted fittings of lamps and lanterns must be inspected regularly, and they should be reinforced in time to prevent accidents.

- To ensure the normal use of this product, please read this instruction carefully.

  - Product introduction

- Total power: w;

- Voltage: AC100V-240V 50/60Hz.

- Color wheel: each color wheel consists of 13 color plates+white light;

- Pattern disk: 14 pattern effects;

- 540 translation, 270 tilt.

- Overheating protection;

- Control mode: voice control, self-propelled, master-slave and DMX512.

- IP20 protection level

  - Signal wire connection

Lamps and lanterns are equipped with standard DMX input and output 3-core or 5-core XLR sockets. Please use twisted-pair signal wire specially shielded for DMX 512; Generally, the connection distance of signal lines is 150 meters. When transmitting long-distance signals, DMX512 signal amplifier must be added.

Use a shielded twisted-pair signal line to connect from the DMX output port of the controller to the DMX input port of the first device, and from the DMX output port of the first device to the DMX input port of the second device, and so on until all the lamps are connected, and then install a terminal plug on the last 3-core jack connected to the lamp output of each connection. (Weld a 4/1W, 120Ω resistor between pins 2 and 3 of the 3-core Canon plug with needle).

Important: The wires must not touch each other or the metal shell.

Fig. 1 schematic diagram of DMX signal line connection

- Calculation method of initial address code of lamps and lanterns:

The starting address code of the current lamp is equal to (the starting address code of the previous lamp)+(the number of channels of the lamp).

1: The initial address code value A001 of the first lamp.

2. The number of basic channels of the controller should be greater than or equal to the total number of channels used by lamps.

3. Note: When using any controller, each lamp must have its own starting address code. If the starting address code of the first lamp is set to A001, the number of lamps is 16CH; Then the starting address code of the second lamp is set to A017; The starting address code of the third lamp is set to A033; And so on, (this setting mode also needs to be determined according to different consoles)

  - Lighting installation

Lamps and lanterns can be placed horizontally, hung obliquely and upside down. Pay attention to the installation method when hanging obliquely and upside down.

such asfigure 1As shown in the figure, before positioning the lamps, it is necessary to ensure the stability of the installation site. During the reverse hanging installation, it is necessary to ensure that the lamps do not fall off the support frame, and it is necessary to use a safety rope to pass through the support frame and the lamp handle for auxiliary hanging to ensure safety and prevent the lamps from falling and sliding.

When installing and debugging lamps, pedestrians are forbidden to pass below, and regularly check whether the safety rope is worn and the hook screw is loose.

If the hanging installation is unstable and the lamps fall, our company will not bear any responsibility.

pictureoneSchematic diagram of upside-down lamps

## control panel

##### 2.1 key description

Fig. 3 schematic diagram of panel key explanation

Let's take "Modify DMX Address Code" as an example to describe the use of keys:

1. If it is not the main interface at present, press the "Left" key (one or more times) to return to the main interface.

2. In the main interface, press the "Up" key or the "Down" key to select the "Settings" button.

3. Press the "OK" key to enter the "Settings" interface.

4. In the Settings interface, press the Up key or the Down key to select DMX Address.

5. Press the "OK" key to enter the editing state.

6. Press the "Up" or "Down" key to modify the DMX address code.

7. Press "OK" to exit the editing status.

##### 2.2 menu description

###### 2.2.1 menu display

| Menu display |  |  |  |
| --- | --- | --- | --- |
| primary menu | Submenu 1 | Function setting | Brief introduction of function |
| DMX address |  | 1-512 |  |
| system setup | Display backward | normal | . |
|  |  | turn |  |
|  | Language settings | Chinese |  |
|  |  | English |  |
|  | Channel mode | 20CH |  |
|  | operational mode | DMX |  |
|  |  | sound-controlled |  |
|  |  | automatic |  |
|  | Electronic lighting | close |  |
|  |  | open |  |
|  | Bulb default | On _ off |  |
|  | Bulb control | illumine |  |
|  | shortest route | On _ off |  |
|  | Horizontal reversal | close |  |
|  |  | open |  |
|  | Vertical inversion | close |  |
|  |  | open |  |
|  | XY speed | low speed |  |
|  |  | intermediate speed |  |
|  |  | fast |  |
|  | Optocoupler detection | close |  |
|  |  | open |  |
|  | Return to the superior | ▲ |  |
| information inquiry | Total time | 0-9999 |  |
|  | Bulb time | 0-9999 |  |
|  | binding hours | 0-9999 |  |
|  | display software | Vx001 |  |
|  | XY software | Vx001 |  |
|  | Small motor software | Vx001 |  |
|  | Return to the superior | ▲ |  |
| Channel control | Color wheel rotation | 0-255 |  |
|  | stroboscopic | 0-255 |  |
|  | Dimming " | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | prism | 0-255 |  |
|  | Prism rotation | 0-255 |  |
|  | Fogging | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine adjustment | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine tuning | 0-255 |  |
|  | reset | 0-255 |  |
|  | light a lamp | 0-255 |  |
|  | XY speed | 0-255 |  |
|  | Color wheel speed | 0-255 |  |
|  | Focusing speed | 0-255 |  |
|  | Pattern speed | 0-255 |  |
|  | Return to the superior | 0-255 |  |
| functional mode | XY communication | normal | current state |
|  |  | mistake |  |
|  | Small motor communication | normal |  |
|  |  | mistake |  |
|  | Focusing hall | normal |  |
|  |  | mistake |  |
|  | Color wheel hall | normal |  |
|  |  | mistake |  |
|  | Pattern hall | normal |  |
|  |  | mistake |  |
|  | X-optical coupler | normal |  |
|  |  | mistake |  |
|  | Y optocoupler | normal |  |
|  |  | mistake |  |
|  | Fan speed | 6500 |  |
|  | Return to the superior | ▲ |  |
| appurtenance | Check code 1 | 0-9999 |  |
|  | Total time | clear |  |
|  |  | carry out |  |
|  | Lamp time | clear |  |
|  |  | carry out |  |
|  | Restore default | default |  |
|  |  | recover |  |
|  |  | preserve |  |
|  | Horizontal rotation | 0-255 | Zero initial position |
|  | Vertical rotation | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Prism 1 | 0-255 |  |
|  | Prism 2 | 0-255 |  |
|  | Prism rotation 1 | 0-255 |  |
|  | Prism rotation 2 | 0-255 |  |
|  | Fogging | 0-255 |  |
|  | Color combination | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | colour disc | 0-255 |  |
|  | Frequency flash | 0-255 |  |
|  | Stroboscopic midpoint | 0-255 |  |
|  | Ridge-1 midpoint | 0-255 |  |
|  | Mid-point of edge two | 0-255 |  |
|  | Atomization midpoint | 0-255 |  |
|  | Colorful midpoint | 0-255 |  |
|  | Return to the superior | ▲ |  |
| Advanced settings | Check code 2 | 0-9999 |  |
|  | Signal encryption | open |  |
|  |  | close |  |
|  | DMX speed | 0-255 |  |
|  | DMX data packet | 0-255 |  |
|  | DMX type | 0-055 |  |
|  | DMX isolation | 0-65535 | The smaller the value, the more stable the signal is. |
|  | Driving plate temperature | < 110 degrees > | If it exceeds this range, red will be displayed. |
|  | Drive board 3.3V | 3.0V - 3.6V | If it exceeds this range, red will be displayed. |
|  | Drive board 5.0V | 4.5V - 5.5V | If it exceeds this range, red will be displayed. |
|  | Drive board 12V | 10.0V - 14.0V | If it exceeds this range, red will be displayed. |
|  | Drive plate 36V | 20.0V - 42.0V | If it exceeds this range, red will be displayed. |
|  | X state | be motionless |  |
|  |  | turn |  |
|  | X hall | normal |  |
|  |  | mistake | Show red |
|  | X-ray optical coupler phase | 0-3 |  |
|  | X optocoupler a | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler b | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler c | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler d | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X current angle | 000-540 |  |
|  | X drive state | normal |  |
|  |  | mistake | Show red |
|  | Y state | be motionless |  |
|  |  | turn |  |
|  | Y optocoupler | normal |  |
|  |  | mistake | Show red |
|  | Y hall | normal |  |
|  |  | mistake | Show red |
|  | Y-coupler phase | 0-3 |  |
|  | Y optocoupler a | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler b | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler c | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler d | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y current angle | 000-250 |  |
|  | Y drive state | normal |  |
|  |  | mistake | Show red |
|  | Return to the superior | ▲ |  |
| Channel level | Color wheel rotation | 0-255 | Current DMX channel value |
|  | stroboscopic | 0-255 |  |
|  | aiming | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | prism | 0-255 |  |
|  | Prism rotation | 0-255 |  |
|  | Fogging | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine adjustment | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine tuning | 0-255 |  |
|  | reset | 0-255 |  |
|  | light a lamp | 0-288 |  |
|  | XY speed | 0-255 |  |
|  | Color wheel speed | 0-255 |  |
|  | Focusing speed | 0-255 |  |
|  | Pattern speed | 0-255 |  |
|  | Return to the superior |  |  |
| reset | Reset all | cancel | manual reset |
|  |  | carry out |  |
|  | XY reset | cancel |  |
|  |  | carry out |  |
|  | Effect reset " | cancel |  |
|  |  | carry out |  |
|  | Return to the superior |  |  |
| Exit menu |  |  |  |

## Channel function

## 3.1 channel table

| passage | 16-channel mode |
| --- | --- |
| one | Color wheel |
| 2 | Cut light/strobe |
| three | aiming |
| four | Pattern disk |
| five | Prism 1 |
| six | Prism rotation |
| seven | Prism 2 |
| eight | atomize |
| nine | focus |
| 10 | X |
| 11 | X fine tuning |
| twelve | Y |
| 13 | Y fine tuning |
| 14 | XY speed |
| 15 | various colours |
| 16 | Light bulb & reset |

Channel parameter value (full version):

| passage | function | Channel value | effect |
| --- | --- | --- | --- |
| one | Color wheel | 000 - 004 005 - 009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 - 074 075 - 079 080 - 084 085 - 089 090 - 094 095 - 099 100 - 104 105 - 109 110 - 114 115 - 119 120 - 124 125 - 129 130 - 134 135 - 139 140 - 144 145 - 149 150 - 203 204 - 255 | white light White light+color 1 Color 1 Color 1+ Color 2 Color 2 Color 2+ color 3 Color 3 Color 3+ color 4 Color 4 Color 4+ color 5 Color 5 Color 5+ color 6 Color 6 Color 6+ color 7 Color 7 Color 7+ color 8 Color 8 Color 8+ color 9 Color 9 Color 9+ color 10 Color 10 Color 10+ color 11 Color 11 Color 11+ color 12 Color 12 Color 12+ color 13 Color 13 Color 13+ color 14 Color 14 Color 14+ white light Reverse flow (from fast to slow) Forward flow (from slow to fast) |
| 2 | stroboscopic | 000-007 008-015 016-131 132-167 168-203 204-247 248-255 | Shutter closed Shutter open → (controlled by dimming channel) Strobe from slow to fast Slow on fast off strobe from slow to fast. Turn on and off the strobe from slow to fast. Random strobe from slow to fast Shutter open → (controlled by dimming channel) |
| three | aiming | 000-255 | From dark to bright |
| four | Pattern disk | 000 - 004 005 - 009 010 - 014 015 - 019 020 - 024 025 - 029 030 - 034 035 - 039 040 - 044 045 - 049 050 - 054 055 - 059 060 - 064 065 - 069 070 071 - 075 076 - 080 081 - 085 086 - 090 091 - 095 096 - 100 101 - 105 106 - 110 111 - 115 116 - 120 121 - 125 126 - 130 131 - 135 136 - 139 140 - 189 190 - 194 195 - 255 | Baiguangkong Solid figure 1 Solid figure 2 Solid figure 3 Solid figure 4 Solid figure 5 Solid figure 6 Solid figure 7 Solid figure 8 Solid figure 9 Solid figure 10 Solid figure 11 Solid figure 12 Solid figure 13 Baiguangkong Figure 1 jitter (from slow to fast) Figure 2 jitter (from slow to fast) Figure 3 jitter (from slow to fast) Figure 4 jitter (from slow to fast) Figure 5 jitter (from slow to fast) Figure 6 jitter (from slow to fast) Figure 7 jitter (from slow to fast) Figure 8 jitter (from slow to fast) Figure 9 jitter (from slow to fast) Figure 10 jitter (from slow to fast) Figure 11 jitter (from slow to fast) Figure 12 jitter (from slow to fast) Figure 13 jitter (from slow to fast) Baiguangkong Forward flow (from fast to slow) Baiguangkong Reverse flow (from slow to fast) |
| five | Prism 1 | 000-015 016-255 | Prism 1 pops up. Prism 1 cuts in |
| six | Prism 1 rotates | 000-127 128-187 188-195 196-255 | stop Reverse rotation (from fast to slow) stop Forward rotation (from slow to fast) |
| seven | Prism 2 | 000-015 016-255 | Prism 1 pops up. Prism 1 cuts in |
| eight | atomize | 000-015 016-255 | without Atomization cut-in |
| nine | focus | 000-255 | Pattern clarity from far to near |
| 10 | X axis | 000-255 | Horizontal 540-degree scanning |
| 11 | X-axis fine tuning | 000-255 | Horizontal 1.2-degree fine adjustment |
| twelve | Y axis | 000-255 | Vertical 270-degree scanning |
| 13 | Y axis fine tuning | 000-255 | Vertical 1.2-degree fine adjustment |
| 14 | XY speed | 000-255 | Speed from fast to slow |
| 15 | various colours | 000-127 128-255 | without Colorful cut-in |
| 16 | Light bulb & reset | 000-099 100-105 200-205 250-255 | No, there is no action in the area where the function is not specified. Extinguish the light bulb Light the bulb All motors reset |

Common faults

Aiming at some common faults, the corresponding solutions are put forward. Any problem that cannot be solved should be handled by professionals. Please disconnect the power supply before maintaining the lamps.

#### The light bulb doesn't work

- Check whether the voltage matched with the lamp is installed;

- Check whether the power supply connection or control switch of lamps and lanterns is in poor contact;

- Check whether the power supply is insufficient;

- Check whether the DMX512 controller has sent instructions.

#### Lamps and lanterns do not accept the control of the console after normal reset.

- Check whether the digital starting address value and function options of lamps and lanterns are correct;

- Check whether the connection of the communication control line is correct, the communication line is too long or has been interrupted;

- Check whether the control equipment fails, and check whether the signal amplifier connected in series fails;

- Check whether the communication line is too long or other equipment interferes with each other;

- Optimize wiring, shorten the length of control signal line, and separate wiring for high voltage and low voltage lines;

- Add a signal amplifier;

- The signal line adopts high-quality shielded twisted pair;

- Connect the signal termination resistor (120 ohms) at the end of the lamp.

#### Lamps can't start.

- Check whether the parameters of power supply are consistent with the lamps;

- Check the lamps for poor contact due to extrusion deformation, vibration and humidity of internal parts during long-distance transportation.

Or fall off.

- Please check whether the wiring connector inside the lamp falls off or is loose.

- Check whether the electronic components of lamps (such as electronic transformer, PCB board, motor control board, etc.) are loose, short-circuited and burned out.

#### When working, the X-axis or Y-axis of the lamp is not normal.

- Check one by one according to the previous step;

- Check whether the drive belt corresponding to the X and Y axis directions in the lamp falls off and breaks;

- Check whether the data feedback receiver (optical coupler) corresponding to the X and Y directions in the lamp is damaged;

- Reboot and reset once.