Please read the manual carefully before installing and using the product

User Manual

810+479+534+A561+A555+P09-5-headed wind tornado

directory

1. Precautions and installation precautions and installation	3

1.1 Statement	3

1.2 Maintenance	3

1.3 Product precautions	3

1.4 Product introduction	4

2. Display panel and key definitions	4

2.1 Control panel	4

2.2 Menu function	5

3. Master-slave control	7

4. DMX512 control	7

4.1 Address Code	7

4.2 CH26 Channel Table	8

4.3 CH75 Channel Table	9

5. Common faults	12

1. Precautions and installation precautions and installation

1.1 Statement

Thank you for choosing our products! This product leaves the factory with good performance and complete packaging. In order for you to use this product safely and effectively, please read this instruction manual carefully and completely before using this product. This manual contains important information for installation and use, please follow the requirements of the manual for installation and operation, and at the same time, please keep this manual properly for use at any time.

This manual is subject to technical changes without notice.

1.2 Maintenance

- This fixture is intended for professional use only.

- If the external flexible cable or wire of this luminaire is damaged, it should be replaced with a special wire or wire provided by the manufacturer or its service agent.

- The light source contained in this luminaire may only be replaced by the manufacturer or its service agent or similar qualified personnel.

- Always connect the product to a protected circuit (circuit breaker or fuse). Ensure that the product has proper electrical grounding to avoid the risk of electric shock or fire.

- To avoid unnecessary wear and extend its life, when not in use, the power supply should be completely cut off by circuit breaker or unplugged.

- The housing may heat up while the product is running. This product should be installed in a well-ventilated location.

- When the product is transferred from an extreme temperature environment (e.g., from a cold truck to a warm and humid environment), the internal electronics of the product can condense. To avoid malfunctions, allow the product to adapt to its surroundings before turning on the power.

- Flash is known to trigger seizures. Users must comply with local notification laws regarding flash use.

1.3 Product precautions

- Disconnect the power supply before cleaning the product or replacing the fuse.

- When not in use, replace and secure all power, data, USB, or other port shields to IP protection.

- Use a safety rope when attaching this product above your head.

- Connect this product to a grounded and protected circuit.

- When the product is on, look at the light source.

- Do not place any flammable materials within 20  cm of this product when operating or turning on the power  supply.

- Do not operate this product if the housing, lenses, or cables are damaged.

- This product can only be handled using handles or hoisting/mounting brackets.

- To avoid unnecessary wear and extend its life, when not in use, the power supply should be completely cut off by circuit breaker or unplugged.

- In case of serious operating problems, it should be stopped immediately

1.4 Product introduction

- Voltage: DC 24+12V

- Power: 324W

- Lamp beads: 5pcs 6065 4-in-1 RGBW + 18pcs 5050 colorful 3-in-1 RGB light strip*5

- Control mode: DMX512, self-propelled, voice controlled, with RDM function

- Channels: CH26, CH75

- Dimming: 32bit 0~100% linear dimming

- Features: Electrodeless + auxiliary light + beam

- Working temperature: -30°C~50°C

- Strobe frequency: 1~30HZ

- Appearance: Metallic, black

- Connection: DMX512 input/output / power input/output.

- IP Rating: IP20

- Size:

- Weight:

2. Display panel and key definitions

2.1 Control panel

Menu key: enter/return to the main menu

Up key: The menu function selects up/adds parameters

Down key: Menu function selects down/decrements parameters

Confirm key: Select the menu function, and confirm and save it when the menu function parameters are modified.

2.2 Menu function

After powering on and resetting the whole machine, press the menu button to enter the main menu (the main menu interface exits after 30 seconds of no operation), see the main menu menu (a total of 5 main menu options) for details; Select the main menu with the up or down button, click the confirmation button to modify the function parameters of each menu, and then press the confirmation button to save the current function parameters (with power loss memory after saving).

| Main menu | Submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System settings | 1. DMX address | Modify the DMX address code up or down (the address code can be selected in the range 001-512), and confirm that the key is saved, default 001. |  |
|  | 2. X reversal | Modify X up/down to turn on/off, confirm key to save, and turn off by default. |  |
|  | 3. Y reversal | Modify Y upwards or downwards to turn on/off, confirm the key to save, and turn off by default. |  |
|  | 4. No signal | Modify the signal clear/hold up or down, confirm the key to save, and clear the default. |  |
|  | 5. Voice control sensitivity | Modify up or down (range 000-099), make sure the key is saved, default 009. |  |
|  | 6. Temperature-fan | Modify Auto/Standard/Quiet up or down, make sure key save, default auto. |  |
|  | 7. Dimming curve | Modify the linear curve, square rate curve, inverse square rate curve, and S curve up or down, and confirm that the key is saved, and the default linear curve is the default. |  |
|  | 8. Dimming curve | Modify up or down fast/smooth, confirm key save, fast by default. |  |
|  | 9. ID number | Modify up or up to close /Y1-Y5, confirm the key to save, and close by default. |  |
|  | 10. Reset | Modify yes/no up or up, confirm key save, default no. |  |
|  | 11. Restore factory settings | Modify yes/no up or up, confirm key save, default no. |  |
|  | 12. Advanced settings |  |  |
| 2. Operation mode | 1. Operation mode | Select the running mode up or down, DMX/Voice Control/Auto, confirm the key to save, default DMX. |  |
|  | 2. Channel selection | Select channel CH26, CH75 up or down, confirm key to save, default CH26. |  |
| 3. Manual control | 1. X-axis | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 2. X-axis fine-tuning | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 3. X-axis speed | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 4. X-axis macro function | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 5. Y-axis | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 6. Fine-tuning the Y-axis | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 7. Y-axis speed | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 8. Y-axis macro function | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 9. The X-axis is infinite | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 10. Dimming | Modify up or down (adjustable range 000-255), confirm the key to save, default 255. |  |
|  | 11. The main light is flashing | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 12. Halo strobe | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 13. Red | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 14. Green | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 15. Blue | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 16. White | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 17. Color temperature | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 18. Color macro | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 19. Halo red | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 20. Halo green | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 21. Halo blue | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 22. Main light mode | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 23. Main light mode speed | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 24. Halo mode | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 25. Halo mode speed | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
|  | 26. Function | Modify up or down (adjustable range 000-255), confirm the key to save, default 000. |  |
| 4. Display settings | 1. Language selection | Select Chinese/EN (English) up or down, confirm the key to save, default Chinese. |  |
|  | 2. Inverted settings | Select Turn off/on the display setting up or down, confirm that the key is saved, and turn off by default. |  |
|  | 3. Screensaver settings | Select Turn off/on backlight settings up or down, confirm the key to save, and turn on by default. |  |
|  | 4. Software version | M5RF |  |
| 5. Information settings | sensor | Used to view Hall status and temperature communication status |  |
|  | error message | Used to display error messages |  |
|  | Channel information | Used to display DMX channel data |  |
|  | Time information | Light on time Luminaire time |  |
| 6. Factory setting | Show Please enter your password! Modify it up or down to 138, and then press the confirm button to enter the factory settings. | 1. X1 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 2. Y1 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 3. F1 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 4. X2 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 5. Y2 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 6. F2 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 7. X3 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 8. Y3 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 9. F3 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 10. X4 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 11. Y4 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 12. F4 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 13. X5 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 14. Y5 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 15. F5 axis calibration | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 16. X itinerary | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 17. Y itinerary | Modify up or down (range 000-255), confirm key save, default 128. |
|  |  | 18. Open value | Modify up or down (range 000-255), confirm the key saves, default 180. |
|  |  | 19. Red | Modify up or down (range 032-255), confirm key save, default 255. |
|  |  | 20. Green | Modify up or down (range 032-255), confirm key save, default 255. |
|  |  | 21. Blue | Modify up or down (range 032-255), confirm key save, default 255. |
|  |  | 22. White | Modify up or down (range 032-255), confirm key save, default 255. |
|  |  | 23. Red 1 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 24. Green 1 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 25. Blue 1 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 26. White 1 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 27. Red 2 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 28. Green 2 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 29. Blue 2 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 30. White 2 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 31. Red 3 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 32. Green 3 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 33. Blue 3 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 34. White 3 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 35. Red 4 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 36. Green 4 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 37. Blue 4 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 38. White 4 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 39. Red 5 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 40. Green 5 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 41. Blue 5 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 42. White 5 | Modify up or down (range 032-255), confirm the key to save, default 252. |
|  |  | 43. Current | Modify up or down (range 032-255), confirm the key to save, default 240. |
|  |  | 44. Temperature | Modify the temperature parameter (range 40-65) up or down, confirm the key to save, default 060. |
|  |  | 45、LED | Modify LEDA/LEDB up or down, make sure the key is saved, and the default LEDA is the default. |
|  |  | 46. LEDXYF direction | Modify up or down (range 000-255), confirm the key saves, default 000. |
|  |  | 47、Logo | Select 000/001 up or down, confirm the key to save, default 000. |
|  |  | 48. Synchronous update | Select On/Off up or down, confirm to save, and turn off by default. |
|  |  | 49. Time is cleared | Select Yes/No up or down, confirm the key saves, no by default. |

3. Master-slave control

There are 2 or more identical lamps connected with DMX three-core signal cables, all lamps are modified to 001~512 arbitrary address code, any one is set as the host, other lamps are slaves, and all slave displays do not flicker; When the main unit is used to adjust the gradient, pulse change, jump, voice control, and self-propelled effects, all slave synchronized gradients, pulse changes, jumps, voice-controlled, and auto-propelled effects.

Special attention: 1. A group of lamps can only set one host, if there are multiple hosts, all the lamps will flash randomly and not synchronized.

- All luminaires must be active when the DMX512 console is turned off.

4. DMX512 control

4.1 Address Code

All the lamps are set with the address code, connect all the lamps to the DMX512 console in parallel with DMX three-core signal cables, the address code stops flashing, indicating that the DMX512 console signal has been sent to the lamp, and use the DMX512 console to control the relevant functions according to the channel instructions.

4.2 CH26 Channel Table

| passage | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 000-255 | X-axis |
| 2 | 000-255 | X-axis fine-tuning |
| 3 | 000-255 | X-axis speed |
| 4 | 000-255 | X-axis macro function |
| 5 | 000-255 | Y-axis |
| 6 | 000-255 | Y-axis fine-tuning |
| 7 | 000-255 | Y-axis velocity |
| 8 | 000-255 | Y-axis macro function |
| 9 | 000-255 | The X-axis is infinite |
| 10 | 000-255 | Dimming |
| 11 | 000-255 | The main light is flashing |
| 12 | 000-255 | Halo strobe |
| 13 | 000-255 | red |
| 14 | 000-255 | green |
| 15 | 000-255 | blue |
| 16 | 000-255 | white |
| 17 | 000-255 | Temperature |
| 18 | 000-255 | Color macro |
| 19 | 000-255 | The halo is red |
| 20 | 000-255 | Halo green |
| 21 | 000-255 | Halo blue |
| 22 | 000-255 | Main light mode |
| 23 | 000-255 | Main light mode speed |
| 24 | 000-255 | Halo mode |
| 25 | 000-255 | Halo mode speed |
| 26 | 000-255 | Function |

4.3 CH75 Channel Table

| passage | Channel value | Basic features: |
| --- | --- | --- |
| 1 | 000-255 | Total dimming |
| 2 | 000-255 | The main light is flashing |
| 3 | 000-255 | Halo strobe |
| 4 | 000-255 | X-axis 1 |
| 5 | 000-255 | X-axis 1 fine-tuning |
| 6 | 000-255 | X-axis 1 infinite |
| 7 | 000-255 | Y-axis 1 |
| 8 | 000-255 | Y-axis 1 fine-tuning |
| 9 | 000-255 | Red 1 |
| 10 | 000-255 | Green 1 |
| 11 | 000-255 | Blue 1 |
| 12 | 000-255 | White 1 |
| 13 | 000-255 | Halo 1 red |
| 14 | 000-255 | Halo 1 green |
| 15 | 000-255 | Aura 1 blue |
| 16 | 000-255 | Halo 1 mode |
| 17 | 000-255 | Halo 1 mode speed |
| 18 | 000-255 | X-axis 2 |
| 19 | 000-255 | X-axis 2 fine-tuning |
| 20 | 000-255 | X-axis 2 infinite |
| 21 | 000-255 | Y-axis 2 |
| 22 | 000-255 | Y-axis 2 fine-tuning |
| 23 | 000-255 | Red 2 |
| 24 | 000-255 | Green 2 |
| 25 | 000-255 | Blue 2 |
| 26 | 000-255 | White 2 |
| 27 | 000-255 | Halo 2 red |
| 28 | 000-255 | Halo 2 green |
| 29 | 000-255 | Halo 2 blue |
| 30 | 000-255 | Halo 2 mode |
| 31 | 000-255 | Halo 2 mode speed |
| 32 | 000-255 | X-axis 3 |
| 33 | 000-255 | X-axis 3 fine-tuning |
| 34 | 000-255 | X-axis 3 infinite |
| 35 | 000-255 | Y-axis 3 |
| 36 | 000-255 | Y-axis 3 fine-tuning |
| 37 | 000-255 | Red 3 |
| 38 | 000-255 | Green 3 |
| 39 | 000-255 | Blue 3 |
| 40 | 000-255 | White 3 |
| 41 | 000-255 | Halo 3 red |
| 42 | 000-255 | Halo 3 green |
| 43 | 000-255 | Halo 3 blue |
| 44 | 000-255 | Halo 3 mode |
| 45 | 000-255 | Halo 3 mode speed |
| 46 | 000-255 | X-axis 4 |
| 47 | 000-255 | X-axis 4 fine-tuning |
| 48 | 000-255 | X-axis 4 infinite |
| 49 | 000-255 | Y-axis 4 |
| 50 | 000-255 | Y-axis 4 fine-tuning |
| 51 | 000-255 | Red 4 |
| 52 | 000-255 | Green 4 |
| 53 | 000-255 | Blue 4 |
| 54 | 000-255 | White 4 |
| 55 | 000-255 | Halo 4 red |
| 56 | 000-255 | Halo 4 green |
| 57 | 000-255 | Halo 4 blue |
| 58 | 000-255 | Halo 4 mode |
| 59 | 000-255 | Halo 4 mode speed |
| 60 | 000-255 | X-axis 5 |
| 61 | 000-255 | X-axis 5 fine-tuning |
| 62 | 000-255 | X-axis 5 infinite |
| 63 | 000-255 | Y-axis 5 |
| 64 | 000-255 | Y-axis 5 fine-tuning |
| 65 | 000-255 | Red 5 |
| 66 | 000-255 | Green 5 |
| 67 | 000-255 | Blue 5 |
| 68 | 000-255 | White 5 |
| 69 | 000-255 | Halo 5 red |
| 70 | 000-255 | Halo 5 green |
| 71 | 000-255 | Halo 5 blue |
| 72 | 000-255 | Halo 5 mode |
| 73 | 000-255 | Halo 5 mode speed |
| 74 | 000-255 | XY axis speed |
| 75 | 000-255 | Function |

Function

| Channel value | Function description: |
| --- | --- |
| 000--005 | No function |
| 006--007 | No function |
| 008--009 | No function |
| 010--029 | No function |
| 030--039 | Dimming curve: linear |
| 040--049 | Dimming curve: square rate |
| 050--059 | Dimming curve: antisquare rate |
| 060--069 | Dimming curve: S-curve |
| 070--079 | Temperature-Fan Mode: Automatic |
| 080--089 | Temperature-Fan Mode: Standard |
| 090--099 | Temperature-Fan Mode: Quiet |
| 100--109 | No function |
| 110--119 | No function |
| 120--122 | No function |
| 123 | No function |
| 124 | No function |
| 125 | No function |
| 126 | No function |
| 127 | No function |
| 128 | No function |
| 129 | No function |
| 130 | No function |
| 131 | No function |
| 132 | No function |
| 133 | No function |
| 134 | No function |
| 135 | No function |
| 136 | No function |
| 137 | No function |
| 138--139 | No function |
| 140--149 | X-axis reset |
| 150--159 | Head reset |
| 160--169 | Head reversed: Yes |
| 170--179 | Head reverse: No |
| 180--199 | No function |
| 200--209 | All reset |
| 210--219 | Dimming speed: fast |
| 220--229 | Dimming speed: smooth |
| 230--255 | No function |

5. Common faults

For some common faults, corresponding solutions are proposed. Any unresolved issues should be handled by professionals. Before maintaining the lamp, disconnect the power supply.

- The lamp beads don't turn on

- Check whether the voltage matching the lamp is installed;

- Check whether the power supply connection or control switch of the lamp is in poor contact;

- Check whether the power supply is insufficient;

- Check if the DMX512 controller has sent instructions.

2. The lamp does not accept the control of the console after normal reset

- Check whether the numeric start address value and function options of the luminaire are correct;

- Check whether the connection of the communication control line is correct, the communication line is too long or has been interrupted;

- Check whether the control equipment is failing, and check whether the signal amplifier connected in series is failing;

- Check whether the communication line is too long or if other devices interfere with each other;

- Optimize the wiring, shorten the length of the control signal line, and separate the high-voltage and low-voltage lines.

- Add a signal amplifier;

- The signal line adopts high-quality shielded twisted pair wire;

- Connect the signal termination resistor (120 ohms) at the end of the luminaire.

3. The light fixture cannot be started

- Check whether the power supply parameters are consistent with the lamps;

- Check that the lamps are in poor contact or fall off due to extrusion deformation, internal parts vibration, moisture, etc. during long-distance transportation.

- Check whether the wires and connectors inside the lamp are falling off or loose.

- Check the electronic components of the lamp (such as electronic transformers, PCB boards, motor control boards, etc.) for looseness, short circuits, and burnouts.