Instructions for two-eye retro matrix lamp

- technical parameter

Power supply: AC90～240V, 50Hz/60Hz.

Rated power: 90W

Lamp bead parameters: 2 40W COB lamp beads, 2 segments of RGB auxiliary light, and 2 circles of lamp strips (62 0.2W RGB LED).

Control mode: international DMX512, 11/19/25 channels.

Dimming mode: 0~100% ultra-smooth dimming.

Control: voice control, self-propelled, master-slave and DMX console mode.

- Key operation and digital tube display instructions

MENU: function key UP: plus key DOWN: minus key ENTER: enter key.

- Digital tube display menu description

| Addr | A001-A512 |  | address code |
| --- | --- | --- | --- |
| chnd | 11/19/25 CH |  | Channel selection |
| STAT | dimm | 0-255 | Total dimming |
|  | stor | 0-255 | stroboscopic |
|  | Gol1 | 0-255 | Main light 1 |
|  | Gol2 | 0-255 | Main light 2 |
|  | Red1 | 0-255 | Auxiliary light red 1 |
|  | Gre1 | 0-255 | Auxiliary light green 1 |
|  | Blu1 | 0-255 | Auxiliary light blue 1 |
|  | Red2 | 0-255 | Auxiliary light red 2 |
|  | Gre2 | 0-255 | Auxiliary light green 2 |
|  | Blu2 | 0-255 | Auxiliary light blue 2 |
|  | Red1 | 0-255 | Lamp band red 1 |
|  | Gre1 | 0-255 | Light band green 1 |
|  | Blu1 | 0-255 | Lamp band blue 1 |
|  | Red2 | 0-255 | Lamp band red 2 |
|  | Gre2 | 0-255 | Light band green 2 |
|  | Blu2 | 0-255 | Lamp band blue 2 |
| AUTO | Au 01-10 |  | 10 self-propelled effects |
| Soud | Su 1 / Su 2 | Se 1-15 | Voice control effect/sensitivity |
| LED | On / Off |  | The screen stands by for 30 seconds and goes out. |
| disp | On / Off |  | Reverse display |
| Uer | V-10 |  | version number |

- DMX channel menu

- DMX 11 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Total stroboscopic |
| CH3 | 000-255 | hot light |
| CH4 | 000-255 | Auxiliary light red |
| CH5 | 000-255 | Auxiliary light green |
| CH6 | 000-255 | Auxiliary light blue |
| CH7 | 000-255 | The lamp is reddish. |
| CH8 | 000-255 | The light is green |
| CH9 | 000-255 | Lamp band blue |
| CH10 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH11 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |

- DMX19 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Main light stroboscopic |
| CH3 | 000-255 | Main light golden yellow 1 |
| CH4 | 000-255 | Main light golden yellow 2, (control brightness when running the main light effect) |
| CH5 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH6 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH7 | 000-255 | Auxiliary light/strip strobe |
| CH8 | 000-255 | Auxiliary light red, (control color when running auxiliary light effect) |
| CH9 | 000-255 | Auxiliary light green, (control color when running auxiliary light effect) |
| CH10 | 000-255 | Auxiliary light blue, (control color when running auxiliary light effect) |
| CH11 | 000-255 | - Auxiliary light effect (see attached table 1 for details) |
| CH12 | 000-255 | - Auxiliary light effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH13 | 000-255 | The light band is red (control the color when running the light band effect). |
| CH14 | 000-255 | The light band is green (control the color when running the light band effect). |
| CH15 | 000-255 | The light band is blue, (control the color when running the light band effect) |
| CH16 | 000-255 | - Effect of light strip (see attached table 1 for details) |
| CH17 | 000-255 | - Effect speed of light strip: the forward speed is adjusted by 0-127 and the reverse speed is adjusted by 128-255; |
| CH18 | 000-255 | Light band effect background color |
| CH19 | 000-255 | Light band effect background tone light |

- DMX 25 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Main light stroboscopic |
| CH3 | 000-255 | Main light golden yellow 1 |
| CH4 | 000-255 | Main light golden yellow 2, (control brightness when running the main light effect) |
| CH5 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH6 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH7 | 000-255 | Auxiliary light/strip strobe |
| CH8 | 000-255 | Auxiliary light red 1 |
| CH9 | 000-255 | Auxiliary light green 1 |
| CH10 | 000-255 | Auxiliary light blue 1 |
| CH11 | 000-255 | Auxiliary light red 2, (control color when running auxiliary light effect) |
| CH12 | 000-255 | Auxiliary light green 2, (control color when running auxiliary light effect) |
| CH13 | 000-255 | Auxiliary light blue 2, (control color when running auxiliary light effect) |
| CH14 | 000-255 | - Auxiliary light effect (see attached table 1 for details) |
| CH15 | 000-255 | - Auxiliary light effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH16 | 000-255 | Light with red 1 |
| CH17 | 000-255 | Light band green 1 |
| CH18 | 000-255 | Light band blue 1 |
| CH19 | 000-255 | The light band is red 2, (control the color when running the light band effect) |
| CH20 | 000-255 | The light band is green 2, (control the color when running the light band effect) |
| CH21 | 000-255 | The light band is blue 2, (control the color when running the light band effect) |
| CH22 | 000-255 | - Effect of light strip (see attached table 1 for details) |
| CH23 | 000-255 | - Effect speed of light strip: the forward speed is adjusted by 0-127 and the reverse speed is adjusted by 128-255; |
| CH24 | 000-255 | Light band effect background color |
| CH25 | 000-255 | Light band effect background tone light |

Schedule 1: Description of built-in effects and functions

| CH value |  | CH value |  | CH value |  |
| --- | --- | --- | --- | --- | --- |
| 0 - 4 | all-or-none | 0 - 4 | all-or-none | 0 | all-or-none |
| 5 - 16 17 - 28 29 - 40 41 - 52 53 - 64 65 - 76 77 - 88 89 - 100 101 - 112 113 - 124 125 - 136 137 - 148 149 - 160 161 - 172 173 - 184 185 - 196 197 - 208 209 - 220 221 - 232 233 - 249 | Main light effect 1 Main light effect 2 main light effect 3 main light effect 4 main light effect 5 main light effect 6 main light effect 7 main light effect 8 Main light effect 9 Main light effect cycle 10 Combined effect 1 Combined effect 2 Combined effect 3 Combined effect 4 Combined effect 5 Combined effect 6 Combined effect 7 Combined effect 8 Combined effect 9 Combined effect 10 | 5 - 16 17 - 28 29 - 40 41 - 52 53 - 64 65 - 76 77 - 88 89 - 100 101 - 112 113 - 124 125 - 136 137 - 148 149 - 160 161 - 172 173 - 255 | Auxiliary light effect 1 auxiliary light effect 2 auxiliary light effect 3 auxiliary light effect 4 auxiliary light effect 5 auxiliary light effect 6 auxiliary light effect 7 auxiliary light effect 8 auxiliary light effect 9 auxiliary light effect 10 auxiliary light effect 11 auxiliary light effect 12 auxiliary light effect 13 Auxiliary light effect 14 Auxiliary light effect cycle 15 | 1 - 4 5 - 8 9 - 12 13 - 16 17 - 20 21 - 24 25 - 28 29 - 32 33 - 36 37 - 40 ... 217 - 220 221 - 224 225 - 228 229 - 232 233 - 236 237 - 240 241 - 244 245 - 248 249 - 252 253 - 255 | Lamp strip effect 1 light strip effect 2 light strip effect 3 light strip effect 4 light strip effect 5 light strip effect 6 light strip effect 7 light strip effect 8 light strip effect 9 light strip effect 10 ... Light strip effect 55 Light strip effect 56 Light strip effect 57 Light strip effect 58 Light strip effect 59 Lamp Strip Effect 60 Light strip effect 61 Light strip effect 62 Light strip effect 63 Light Strip Effect Cycle 64 |
| 250 - 252 | Voice control effect 1 |  |  |  |  |
| 253 - 255 | Voice control effect 2 |  |  |  |  |