| 7 Bee Eye Menu Operation Settings |  |  |  |
| --- | --- | --- | --- |
| primary menu | Submenu 1 | Function setting | Brief introduction of function |
| DMX address |  | 1-512 |  |
| system setup | Display backward | normal | . |
|  |  | turn |  |
|  | Language settings | Chinese |  |
|  |  | English |  |
|  | Channel mode | 20CH |  |
|  | operational mode | DMX |  |
|  |  | sound-controlled |  |
|  |  | automatic |  |
|  | Master-slave mode | From the machine |  |
|  |  | host machine |  |
|  | Dimming mode | ON_ OFF |  |
|  | Horizontal reversal | close |  |
|  |  | open |  |
|  | Vertical inversion | close |  |
|  |  | open |  |
|  | XY speed | low speed |  |
|  |  | intermediate speed |  |
|  |  | fast |  |
|  | Optocoupler detection | close |  |
|  |  | Open open |  |
|  | Signal clearing | Turn Off |  |
|  |  | Open open |  |
|  | Sensitive voice control | 0-100% |  |
|  | Bai Pingheng | Turn Off |  |
|  |  | Open open |  |
|  | White balance red | 0-255 |  |
|  | White balanced green | 0-255 |  |
|  | White balance blue | 0-255 |  |
|  | White balance white | 0-255 |  |
|  | Return to the superior | ▲ |  |
| Channel control | Red color | 0-255 |  |
|  | Red fine tuning | 0-255 |  |
|  | Green color | 0-255 |  |
|  | Green fine tuning | 0-255 |  |
|  | Blue color | 0-255 |  |
|  | Blue fine tuning | 0-255 |  |
|  | White color | 0-255 |  |
|  | White fine tuning | 0-255 |  |
|  | Color temperature | 0-255 |  |
|  | Yansehong | 0-255 |  |
|  | Frequency flash | 0-255 |  |
|  | aiming | 0-255 |  |
|  | Dimming and fine tuning | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine adjustment | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine tuning | 0-255 |  |
|  | Function setting | 0-255 |  |
|  | Complex position | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Focusing rotation | 0-255 |  |
|  | LED effect | 0-255 |  |
|  | LED speed | 0-255 |  |
|  | Effect gradient | 0-255 |  |
|  | Effect red | 0-255 |  |
|  | Effect green | 0-255 |  |
|  | Effect blue | 0-255 |  |
|  | Effect white | 0-255 |  |
|  | Effect dimming | 0-255 |  |
|  | Background dimming | 0-255 |  |
|  | Excessive effect | 0-255 |  |
|  | Effect angle | 0-255 |  |
|  | Foreground stroboscopic | 0-255 |  |
|  | Background stroboscopic | 0-255 |  |
|  | Background selection | 0-255 |  |
|  | Return to the superior | 0-255 |  |
| functional mode | Total time | 0-9999 | current state |
|  | Motherboard software | Vxxx |  |
|  | Focusing hall | normal |  |
|  |  | mistake |  |
|  | Rotating hall | normal |  |
|  |  | mistake |  |
|  | X-optical coupler | normal |  |
|  |  | mistake |  |
|  | Y optocoupler | normal |  |
|  |  | mistake |  |
|  | Temperature detection | 0-120 |  |
|  | Return to the superior | ▲ |  |
| appurtenance | Check code 1 | 0-9999 |  |
|  | Check code 1 | ▲ |  |
|  | Total time | clear |  |
|  |  | carry out |  |
|  | Restore default | default |  |
|  |  | recover |  |
|  |  | preserve |  |
|  | Lamp bead 1 red | 0-255 |  |
|  | Lamp bead 1 green | 0-255 |  |
|  | Lamp bead 1 blue | 0-255 |  |
|  | Lamp bead 1 white | 0-255 |  |
|  | Lamp bead 2 red | 0-255 |  |
|  | Lamp bead 2 green | 0-255 |  |
|  | Lamp bead 2 blue | 0-255 |  |
|  | Lamp bead 2 white | 0-255 |  |
|  | Lamp bead 3 red | 0-255 |  |
|  | Lamp bead 3 green | 0-255 |  |
|  | Lamp bead 3 blue | 0-255 |  |
|  | Lamp bead 3 white | 0-255 |  |
|  | Lamp bead 4 red | 0-255 |  |
|  | Lamp bead 4 green | 0-255 |  |
|  | Lamp bead 4 blue | 0-255 |  |
|  | Lamp bead 4 white | 0-255 |  |
|  | Lamp bead 5 red | 0-255 |  |
|  | Lamp bead 5 green | 0-255 |  |
|  | Lamp bead 5 blue | 0-255 |  |
|  | Lamp bead 5 white | 0-255 |  |
|  | Lamp bead 6 red | 0-255 |  |
|  | Lamp bead 6 green | 0-255 |  |
|  | Lamp bead 6 blue | 0-255 |  |
|  | Lamp bead 6 white | 0-255 |  |
|  | Lamp bead 7 red | 0-255 |  |
|  | Lamp bead 7 green | 0-255 |  |
|  | Lamp bead 7 blue | 0-255 |  |
|  | Lamp bead 7 white | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Focusing rotation | 0-255 |  |
|  | Return to the superior | ▲ |  |
| Channel level | Red color | 0-255 | Only channel values can be queried. |
|  | Red fine tuning | 0-255 |  |
|  | Green color | 0-255 |  |
|  | Green fine tuning | 0-255 |  |
|  | Blue color | 0-255 |  |
|  | Blue fine tuning | 0-255 |  |
|  | White color | 0-255 |  |
|  | White fine tuning | 0-255 |  |
|  | Color temperature | 0-255 |  |
|  | Yansehong | 0-255 |  |
|  | Frequency flash | 0-255 |  |
|  | aiming | 0-255 |  |
|  | Dimming and fine tuning | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine adjustment | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine tuning | 0-255 |  |
|  | Function setting | 0-255 |  |
|  | Complex position | 0-255 |  |
|  | Focus adjustment | 0-255 |  |
|  | Focusing rotation | 0-255 |  |
|  | LED effect | 0-255 |  |
|  | LED speed | 0-255 |  |
|  | Effect gradient | 0-255 |  |
|  | Effect red | 0-255 |  |
|  | Effect green | 0-255 |  |
|  | Effect blue | 0-255 |  |
|  | Effect white | 0-255 |  |
|  | Effect dimming | 0-255 |  |
|  | Background dimming | 0-255 |  |
|  | Excessive effect | 0-255 |  |
|  | Effect angle | 0-255 |  |
|  | Foreground stroboscopic | 0-255 |  |
|  | Background stroboscopic | 0-255 |  |
|  | Background selection | 0-255 |  |
|  | Return to the superior | 0-255 |  |
| Reset all | cancel |  |  |
|  | carry out |  |  |
| Advanced settings | Signal encryption | open |  |
|  |  | close |  |
|  | DMX speed | 0-255 |  |
|  | DMX data packet | 0-255 |  |
|  | DMX type | 0-055 |  |
|  | DMX isolation | 0-65535 | The smaller the value, the more stable the signal is. |
|  | LED temperature | < 110 degrees > | If it exceeds this range, red will be displayed. |
|  | ADC detection | 0-4096 |  |
|  | X state | be motionless |  |
|  |  | turn |  |
|  | X hall | normal |  |
|  |  | mistake | Show red |
|  | X-ray optical coupler phase | 0-3 |  |
|  | X optocoupler a | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler b | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler c | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X optocoupler d | 40-160 | The standard setting of 100 exceeds the required range and displays red, but the red light on the panel does not give an alarm. |
|  | X current angle | 000-540 |  |
|  | X drive state | normal |  |
|  |  | mistake | Show red |
|  | Y state | be motionless |  |
|  |  | turn |  |
|  | Y optocoupler | normal |  |
|  |  | mistake | Show red |
|  | Y hall | normal |  |
|  |  | mistake | Show red |
|  | Y-coupler phase | 0-3 |  |
|  | Y optocoupler a | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler b | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler c | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y optocoupler d | 40-160 | If it exceeds this range, red will be displayed. |
|  | Y current angle | 000-250 |  |
|  | Y drive state | normal |  |
|  |  | mistake | Show red |
|  | Green-blue exchange | close |  |
|  |  | open |  |
|  | Return to the superior | ▲ |  |

Master-slave control

There are two or more identical lamps connected by DMX three-core signal lines, all lamps are modified to any address code of 001~512, and any one is set as the master, while other lamps are slaves, and all slave displays do not flicker; When the master adjusts the effects of gradual change, pulse change, jump, voice control and self-walking, all slaves synchronize the effects of gradual change, pulse change, jump, voice control and self-walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

2. All lamps and lanterns must be used when the DMX512 console is closed.

DMX512 control

Address codes of all lamps and lanterns are set, and all lamps and lanterns are connected to the DMX512 console in parallel with DMX three-core signal lines. The address code stops flashing, indicating that the DMX512 console signal has been sent to the lamps and lanterns. According to the channel description, the DMX512 console is used to control related functions.

| 24-channel mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Channel value |  | function |  | DMX value | explain |  |  |
| 1 |  | X axis control |  | 0 - 255 | X axis control |  |  |
| 2 |  | X-axis fine tuning |  | 0 - 255 | X-axis fine tuning |  |  |
| 3 |  | Y axis control |  | 0 - 255 | Y axis control |  |  |
| 4 |  | Y axis fine tuning |  | 0 - 255 | Y axis fine tuning |  |  |
| 5 |  | XY axis velocity |  | 0 - 255 | Speed from slow to fast |  |  |
| 6 |  | focus |  | 0-15 | Focus control dyeing mode Focus cannot be rotated. |  |  |
|  |  |  |  | 16- 255 | Focus control Bee-eye mode Focus can rotate. |  |  |
| 7 |  | rotate |  | 0 - 127 | 0 to 60 degrees |  |  |
|  |  |  |  | 128 - 190 | Rotate counterclockwise from slow to fast |  |  |
|  |  |  |  | 191 - 192 | stop |  |  |
|  |  |  |  | 193 - 255 | Rotate clockwise from slow to fast |  |  |
| 8 |  | Light strip control |  | 000-004 | The lamp band goes out |  |  |
|  |  |  |  | 005-013 | red |  |  |
|  |  |  |  | 014-022 | green |  |  |
|  |  |  |  | 023-031 | blue |  |  |
|  |  |  |  | 032-040 | white |  |  |
|  |  |  |  | 041-255 | Macro change of light strip color One color stops at the fastest speed every 3 DMX values. |  |  |
| 9 |  | Total dimming |  | 0 - 255 | 0-100% linear hidden weapons modulation |  |  |
| 10 |  | stroboscopic |  | 0 - 3 | Lights go out |  |  |
|  |  |  |  | 4 - 103 | Use the same steps from fast to slow. |  |  |
|  |  |  |  | 104 - 107 | Light up |  |  |
|  |  |  |  | 108 - 207 | Pulse strongpoint from fast to slow. |  |  |
|  |  |  |  | 208 - 212 | Light up |  |  |
|  |  |  |  | 213 - 225 | High speed random strobe |  |  |
|  |  |  |  | 226 - 238 | Medium speed random strobe |  |  |
|  |  |  |  | 239 - 251 | Low speed random strobe |  |  |
|  |  |  |  | 252 - 255 | Light up |  |  |
| 11 |  | red |  | 0 - 255 | Red 0-100% brightness adjustment, from dark to bright. |  |  |
| 12 |  | green |  | 0 - 255 | Green 0-100% brightness adjustment, from dark to bright. |  |  |
| 13 |  | blue |  | 0 - 255 | Blue 0-100% brightness adjustment, from dark to bright. |  |  |
| 14 |  | white |  | 0 - 255 | White 0-100% brightness adjustment, from dark to bright. |  |  |
| 15 |  | colour temperature |  | 0 - 255 | Linear color temperature control high-> low |  |  |
| 16 |  | Color macro |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 255 | The color macro changes one color every 3 DMX values. |  |  |
| 17 |  | Static effect |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 204 | A static effect for every 5 DMX values. |  |  |
|  |  |  |  | 205 - 215 | Static effects switch effects forward in turn. |  |  |
|  |  |  |  | 216 - 228 | Static effects stop switching. |  |  |
|  |  |  |  | 229 - 239 | Static effects switch effects forward in turn. |  |  |
|  |  |  |  | 240 - 245 | The static effect switches quickly at random. |  |  |
|  |  |  |  | 246 - 250 | Static effect random switching |  |  |
|  |  |  |  | 251 - 255 | Static effect random switching is slow. |  |  |
| 18 |  | Dynamic effect Channel control of foreground colors 20,21,22,23 Backcolor 11,12,13,14 channel control |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 255 | One dynamic effect for every 4 DMX values. |  |  |
| 19 |  | Dynamic effect speed |  | 0 - 127 | Forward rotation is slow to fast |  |  |
|  |  |  |  | 128 - 255 | Reverse rotation is fast to slow. |  |  |
| 20 |  | Background color red |  | 0 - 255 | 1-100% background red brightness adjustment, from dark to bright. |  |  |
| 21 |  | Background color green |  | 0 - 255 | 1-100% background blue brightness adjustment, from dark to bright. |  |  |
| 22 |  | Background color blue |  | 0 - 255 | 1-100% background green brightness adjustment, from dark to bright. |  |  |
| 23 |  | Background color is white. |  | 0 - 255 | 1-100% background blue brightness adjustment, from dark to bright. |  |  |
| 24 |  | reset |  | 0 - 250 | No function |  |  |
|  |  |  |  | 251 - 255 | The reset time is 5 seconds. |  |  |
| 35-channel mode |  |  |  |  |  |  |  |
| Channel value | function |  | DMX value |  | explain |  |  |
| 1 | red |  | 0 - 255 |  | Red 0-100% brightness adjustment, from dark to bright. |  |  |
| 2 | Red fine tuning |  | 0 - 255 |  | Red fine tuning |  |  |
| 3 | green |  | 0 - 255 |  | Green 0-100% brightness adjustment, from dark to bright. |  |  |
| 4 | Green fine tuning |  | 0 - 255 |  | Green fine tuning |  |  |
| 5 | blue |  | 0 - 255 |  | Blue 0-100% brightness adjustment, from dark to bright. |  |  |
| 6 | Blue fine tuning |  | 0 - 255 |  | Blue fine tuning |  |  |
| 7 | white |  | 0 - 255 |  | White 0-100% brightness adjustment, from dark to bright. |  |  |
| 8 | White fine tuning |  | 0 - 255 |  | White fine tuning |  |  |
| 9 | colour temperature |  | 0 - 255 |  | Linear color temperature control high-> low |  |  |
| 10 | Color macro |  | 0 - 4 |  | without |  |  |
|  |  |  | 5 - 255 |  | The color macro changes one color every 3 DMX values. |  |  |
| 11 | stroboscopic |  | 0 - 3 |  | Lights go out |  |  |
|  |  |  | 4 - 103 |  | Use the same steps from fast to slow. |  |  |
|  |  |  | 104 - 107 |  | Light up |  |  |
|  |  |  | 108 - 207 |  | Pulse strongpoint from fast to slow. |  |  |
|  |  |  | 208 - 212 |  | Light up |  |  |
|  |  |  | 213 - 225 |  | High speed random strobe |  |  |
|  |  |  | 226 - 238 |  | Medium speed random strobe |  |  |
|  |  |  | 239 - 251 |  | Low speed random strobe |  |  |
|  |  |  | 252 - 255 |  | Light up |  |  |
| 12 | Total dimming |  | 0 - 255 |  | 0-100 Linear brightness adjustment |  |  |
| 13 | Total dimming and fine tuning |  | 0 - 255 |  | Fine adjustment of dimmer |  |  |
| 14 | X axis control |  | 0 - 255 |  | X axis control |  |  |
| 15 | X-axis fine tuning |  | 0 - 255 |  | X-axis fine tuning |  |  |
| 16 | Y axis control |  | 0 - 255 |  | Y axis control |  |  |
| 17 | Y axis fine tuning |  | 0 - 255 |  | Y axis fine tuning |  |  |
| 18 | Light strip control |  | 000-004 |  | The lamp band goes out |  |  |
|  |  |  | 005-013 |  | red |  |  |
|  |  |  | 014-022 |  | green |  |  |
|  |  |  | 023-031 |  | blue |  |  |
|  |  |  | 032-040 |  | white |  |  |
|  |  |  | 041-255 |  | Macro change of light strip color One color stops at the fastest speed every 3 DMX values. |  |  |
| 19 | reset |  | 0 - 250 |  | No function |  |  |
|  |  |  | 251 - 255 |  | The reset time is 5 seconds. |  |  |
| 20 | focus |  | 0-15 |  | Focus control dyeing mode Focus cannot be rotated. |  |  |
|  |  |  | 16- 255 |  | Focus control Bee-eye mode Focus can rotate. |  |  |
| 21 | rotate |  | 0 - 127 |  | 0 to 60 degrees |  |  |
|  |  |  | 128 - 190 |  | Rotate counterclockwise from slow to fast |  |  |
|  |  |  | 191 - 192 |  | stop |  |  |
|  |  |  | 193 - 255 |  | Rotate clockwise from slow to fast |  |  |
| 22 | Dynamic effect |  | 0 - 4 |  | without |  |  |
|  |  |  | 5 - 255 |  | One dynamic effect for every 4 DMX values. Foreground Color 20-23 Channel Control Backcolor 11-14 Channel Control |  |  |
| 23 | Dynamic effect speed The foreground color is controlled by channels 24, 25, 26, 27, 28, 29 and 33. The background color is controlled by channels 1, 2, 3, 4, 5, 6, 7, 8, 9, 30 and 34. |  | 0 - 127 |  | Forward rotation is slow to fast |  |  |
|  |  |  | 128 - 255 |  | Reverse rotation is fast to slow. |  |  |
| 24 | Dynamic effect fade-in and fade-out time |  | 0 - 127 |  | Forward rotation is slow to fast |  |  |
|  |  |  | 5 - 255 |  | Reverse rotation is fast to slow. |  |  |
| 25 | Background color red |  | 0 - 255 |  | Red shape |  |  |
| 26 | Background color green |  | 0 - 255 |  | Green shape |  |  |
| 27 | Background color blue |  | 0 - 255 |  | Blue shape |  |  |
| 28 | Background color is white. |  | 0 - 255 |  | White shape |  |  |
| 29 | Foreground brightness adjustment |  | 0 - 255 |  | Foreground dimmer 0-100% |  |  |
| 30 | Background brightness adjustment |  | 0 - 255 |  | Background dimmer 0-100% |  |  |
| 31 | NC |  | 0 - 255 |  | NC |  |  |
| 32 | Dynamic effect frame selection |  | 0 - 255 |  | Frame selection |  |  |
| 33 | Foreground flashes frequently. |  | 0 - 4 |  | No stroboscopic constant light |  |  |
|  |  |  | 5 - 250 |  | Strobe fast to slow |  |  |
|  |  |  | 251 - 255 |  | No stroboscopic constant light |  |  |
| 34 | Background color stroboscopic |  | 0 - 4 |  | No stroboscopic constant light |  |  |
|  |  |  | 5 - 250 |  | Strobe fast to slow |  |  |
|  |  |  | 251 - 255 |  | No stroboscopic constant light |  |  |
| 35 | Background color macro |  | 0 - 4 |  | without |  |  |
|  |  |  | 5 - 255 |  | The color macro changes one color every 3 DMX values. |  |  |
| 52-channel mode |  |  |  |  |  |  |  |
| Channel value |  | function |  | DMX value | explain |  |  |
| 1 |  | X axis control |  | 0 - 255 | X axis control |  |  |
| 2 |  | X-axis fine tuning |  | 0 - 255 | X-axis fine tuning |  |  |
| 3 |  | Y axis control |  | 0 - 255 | Y axis control |  |  |
| 4 |  | Y axis fine tuning |  | 0 - 255 | Y axis fine tuning |  |  |
| 5 |  | XY axis velocity |  | 0 - 255 | Speed from slow to fast |  |  |
| 6 |  | focus |  | 0-15 | Focus control dyeing mode Focus cannot be rotated. |  |  |
|  |  |  |  | 16- 255 | Focus control Bee-eye mode Focus can rotate. |  |  |
| 7 |  | rotate |  | 0 - 127 | 0 to 60 degrees |  |  |
|  |  |  |  | 128 - 190 | Rotate counterclockwise from slow to fast |  |  |
|  |  |  |  | 191 - 192 | stop |  |  |
|  |  |  |  | 193 - 255 | Rotate clockwise from slow to fast |  |  |
| 8 |  | Light strip control |  | 000-004 | The lamp band goes out |  |  |
|  |  |  |  | 005-013 | red |  |  |
|  |  |  |  | 014-022 | green |  |  |
|  |  |  |  | 023-031 | blue |  |  |
|  |  |  |  | 032-040 | white |  |  |
|  |  |  |  | 041-255 | Macro change of light strip color One color stops at the fastest speed every 3 DMX values. |  |  |
| 9 |  | Total dimming |  | 0 - 255 | 0-100% linear hidden weapons modulation |  |  |
| 10 |  | stroboscopic |  | 0 - 3 | Lights go out |  |  |
|  |  |  |  | 4 - 103 | Use the same steps from fast to slow. |  |  |
|  |  |  |  | 104 - 107 | Light up |  |  |
|  |  |  |  | 108 - 207 | Pulse strongpoint from fast to slow. |  |  |
|  |  |  |  | 208 - 212 | Light up |  |  |
|  |  |  |  | 213 - 225 | High speed random strobe |  |  |
|  |  |  |  | 226 - 238 | Medium speed random strobe |  |  |
|  |  |  |  | 239 - 251 | Low speed random strobe |  |  |
|  |  |  |  | 252 - 255 | Light up |  |  |
| 11 |  | red |  | 0 - 255 | Red 0-100% brightness adjustment, from dark to bright. |  |  |
| 12 |  | green |  | 0 - 255 | Green 0-100% brightness adjustment, from dark to bright. |  |  |
| 13 |  | blue |  | 0 - 255 | Blue 0-100% brightness adjustment, from dark to bright. |  |  |
| 14 |  | white |  | 0 - 255 | White 0-100% brightness adjustment, from dark to bright. |  |  |
| 15 |  | colour temperature |  | 0 - 255 | Linear color temperature control high-> low |  |  |
| 16 |  | Color macro |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 255 | The color macro changes one color every 3 DMX values. |  |  |
| 17 |  | Static effect |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 204 | A static effect for every 5 DMX values. |  |  |
|  |  |  |  | 205 - 215 | Static effects switch effects forward in turn. |  |  |
|  |  |  |  | 216 - 228 | Static effects stop switching. |  |  |
|  |  |  |  | 229 - 239 | Static effects switch effects forward in turn. |  |  |
|  |  |  |  | 240 - 245 | The static effect switches quickly at random. |  |  |
|  |  |  |  | 246 - 250 | Static effect random switching |  |  |
|  |  |  |  | 251 - 255 | Static effect random switching is slow. |  |  |
| 18 |  | Dynamic effect Channel control of foreground colors 20,21,22,23 Backcolor 11,12,13,14 channel control |  | 0 - 4 | without |  |  |
|  |  |  |  | 5 - 255 | One dynamic effect for every 4 DMX values. |  |  |
| 19 |  | Dynamic effect speed |  | 0 - 127 | Forward rotation is slow to fast |  |  |
|  |  |  |  | 128 - 255 | Reverse rotation is fast to slow. |  |  |
| 20 |  | Background color red |  | 0 - 255 | 1-100% background red brightness adjustment, from dark to bright. |  |  |
| 21 |  | Background color green |  | 0 - 255 | 1-100% background blue brightness adjustment, from dark to bright. |  |  |
| 22 |  | Background color blue |  | 0 - 255 | 1-100% background green brightness adjustment, from dark to bright. |  |  |
| 23 |  | Background color is white. |  | 0 - 255 | 1-100% background blue brightness adjustment, from dark to bright. |  |  |
| 24 |  | reset |  | 0 - 250 | No function |  |  |
|  |  |  |  | 251 - 255 | The reset time is 5 seconds. |  |  |
| 25 |  | Lamp bead 1 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 26 |  | Lamp bead 1 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 27 |  | Lamp bead 1 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 28 |  | Lamp bead 1 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 29 |  | Lamp Bead 2 Red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 30 |  | Lamp bead 2 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 31 |  | Lamp bead 2 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 32 |  | Lamp bead 2 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 33 |  | Lamp bead 3 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 34 |  | Lamp bead 3 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 35 |  | Lamp bead 3 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 36 |  | Lamp bead 3 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 37 |  | Lamp bead 4 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 38 |  | Lamp bead 4 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 39 |  | Lamp bead 4 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 40 |  | Lamp bead 4 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 41 |  | Lamp bead 5 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 42 |  | Lamp bead 5 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 43 |  | Lamp bead 5 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 44 |  | Lamp bead 5 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 45 |  | Lamp bead 6 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 46 |  | Lamp bead 6 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 47 |  | Lamp bead 6 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 48 |  | Lamp bead 6 white |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 49 |  | Lamp bead 7 red |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 50 |  | Lamp bead 7 green |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 51 |  | Lamp bead 7 blue |  | 0 - 255 | The brightness is dark to bright. |  |  |
| 52 |  | Lamp bead 7 white |  | 0 - 255 | The brightness is dark to bright. |  |  |

Voltage: AC100~240V 50/60HZ

Power: 150W

Lamp beads: 7 4-in-1 LED lamp beads

Control mode: DMX512, self-propelled, master-slave and voice control,

Channels: CH24, CH35 and CH52

Dimming: 32bit 0~100% linear dimming

Features: shake your head+dye+flash.

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: 27*18*40cm

Weight: 6.15kg