I. Definition of Display Panel and Keys

Menu up and down confirmation

Menu key: select function

Up arrow: parameter increment

Down arrow: decreasing parameter

Confirm key: OK and save.

Second, the menu function

Press the menu key after powering on and turning on, and the menu menu will appear in turn; Press the up or down key to modify the function parameters, and press the Enter key to save the current function and parameters (with power-off memory after saving).

Menu menu:

| A001 |  | A512 | Set the address code, modify the address code (A001~A512) up or down, and confirm to save. |
| --- | --- | --- | --- |
| CH06 |  | CH363 | Switch channels CH06, CH08, CH12, CH21, CH22 and CH363 up or down, and confirm the key to save. Default CH12 |
| N127 |  | N127 | Auxiliary light effect mode, confirm key to save. |
| S255 |  | S000 | Modify the running speed of auxiliary light effect up or down (S000~ S255), and confirm to save. |
| E000 |  | E017 | In the middle main light mode, confirm the key to save. |
| S255 |  | S000 | Modify the running speed of the intermediate main light up or down (S000~ S255), and confirm to save. |
| Soud |  | Soud | Voice control mode. |
| U255 |  | U000 | Article 1: White light brightness, modify the brightness of monochromatic light (U000~U255) up or down, and press the Enter key to save. |
| V255 |  | V000 | Article 2 White light brightness: modify the brightness of monochromatic light (V000~V255) up or down, and press the Enter key to save. |
| W255 |  | W000 | Article 3 White light brightness: modify the brightness of monochromatic lights (W000~W255) up or down, and press the Enter key to save. |
| R255 |  | R000 | The internal auxiliary light is dimmed by the red light bulb. Modify the brightness of the red light bulb (R000~ R255) up or down, and press the Enter key to save. |
| G255 |  | G000 | The interior auxiliary light is dimmed by the green light bulb, and the brightness of the green light bulb (G000~ G255) is modified up or down, and the Enter key is used to save. |
| B255 |  | B000 | The internal auxiliary light is dimmed by the blue light bulb, and the brightness of the blue light bulb is modified up or down (B000~ B255), and the Enter key is used to save. |
| X255 |  | X000 | The external auxiliary light is dimmed by the red light bulb, and the brightness of the red light bulb is modified up or down (X000~ X255). Press the Enter key to save. |
| Y255 |  | Y000 | The external auxiliary light is dimmed by the green light bulb, and the brightness of the green light bulb (Y000~ Y255) is modified up or down, and the Enter key is used to save. |
| Z255 |  | Z000 | The external auxiliary light is dimmed by the blue light bulb, and the brightness of the blue light bulb (Z000~ Z255) is modified up or down, and the Enter key is used to save. |
| T000 |  |  | Display the temperature, such as T045, indicating that the current lamp temperature is 45℃. |

Third, master-slave control

Two or more identical lamps are connected by DMX three-core signal lines, all lamps are set to any address code A001~A512, and any one is set as the master, while other lamps are slaves, and all slave displays do not flicker; When the master adjusts the effects of gradual change, pulse change, jump, voice control and self-walking, all slaves synchronize the effects of gradual change, pulse change, jump, voice control and self-walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

2. All lamps and lanterns must be used when the DMX512 console is closed.

Fourth, the factory settings

Press the menu key for 3 seconds to enter the factory setting when any address code is A001~A512. Factory settings mainly include the functions of each output power of lamps and lanterns, fan setting mode, setting temperature protection point and sending parameters. The factory sets any mode and presses the menu button for 3 seconds to exit.

Factory Settings Table:

| R255 |  | R020 | R red light bulb current setting, modify the hexagonal red light bulb current (R020-R255) up or down, and confirm to save. |
| --- | --- | --- | --- |
| G255 |  | G020 | G green light bulb current setting, modify the hexagonal green light bulb current (G020-G255) up or down, and confirm to save. |
| B255 |  | B020 | B blue light bulb current setting, modify the hexagonal blue light bulb current up or down (B020-B255), and confirm to save. |
| W255 |  | W020 | W monochrome lamp current setting, modify the monochrome lamp current setting up or down (W020~W255), and press the Enter key to save. |
| FAN1 |  | FAN0 | Fan setting: start the fan as soon as the FAN0 light bulb lights up, and start the fan as soon as the FAN1 reaches the set temperature protection point, and press the Enter key to save. |
| T040 |  | T-105 | Set the temperature protection point, modify the parameters (40℃~75℃) up or down, and press the Enter key to save. |
| Send |  | Send | Send the factory setting parameters of this machine up or down to all other lamps with three-core signal lines connected in parallel; Confirm the sending parameters and press the menu button for 3 seconds. Exit, deny the parameter and press the confirm key to cancel sending. |

V. DMX512 console

After electrifying, the address codes of all lamps and lanterns are set, and all lamps and lanterns are connected to the DMX512 console in parallel with three-core signal lines, and the address codes will stop flashing, indicating that the DMX512 console signal has been sent to the lamps and lanterns. According to each channel description, the DMX512 console is used to control related functions.

CH06 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| passage | Channel value | fundamental function |
| one | 000-255 | Baiyi |
| 2 | 000-255 | White 2 |
| three | 000-255 | White 3 |
| four | 000-255 | R |
| five | 000-255 | G |
| six | 000-255 | B |

CH08 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| passage | Channel value | fundamental function |
| one | 000-255 | Baiyi |
| 2 | 000-255 | White 2 |
| three | 000-255 | White 3 |
| four | 000-255 | Internal mode |
| five | 000-255 | Internal velocity |
| six | 000-255 | External mode |
| seven | 000-255 | External velocity |
| eight | 000-255 | External direction |

CH12 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | Baiyi |
| 2 | 000-255 | White 2 |
| three | 000-255 | White 3 |
| four | 000-255 | R1 |
| five | 000-255 | G1 |
| six | 000-255 | B1 |
| seven | 000-255 | R2 |
| eight | 000-255 | G2 |
| nine | 000-255 | B2 |
| 10 | 000-255 | R3 |
| 11 | 000-255 | G3 |
| 12 | 000-255 | B3 |

CH21 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | aiming |
| 2 | 000-255 | stroboscopic |
| three | 000-255 | white |
| four | 000-255 | White strobe |
| five | 000-255 | White mode |
| six | 000-255 | White speed |
| seven | 000-255 | Inner r |
| eight | 000-255 | Neig |
| nine | 000-255 | Neib |
| 10 | 000-255 | Internal stroboscopic |
| 11 | 000-255 | Internal mode |
| 12 | 000-255 | Internal velocity |
| 13 | 000-255 | External r |
| 14 | 000-255 | Waig |
| 151 | 000-255 | Waib |
| six | 000-255 | External frequency flash |
| 17 | 000-255 | External mode |
| 18 | 000-255 | External velocity |
| 19 | 000-255 | External direction |
| 20 | 000-255 | background color |
| 21 | 000-255 | Background tone light |

CH22 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | aiming |
| 2 | 000-255 | Baiyi |
| three | 000-255 | White 2 |
| four | 000-255 | White 3 |
| five | 000-255 | White strobe |
| six | 000-255 | White mode |
| seven | 000-255 | White speed |
| eight | 000-255 | Inner r |
| nine | 000-255 | Neig |
| 10 | 000-255 | Neib |
| 11 | 000-255 | Internal stroboscopic |
| 12 | 000-255 | Internal mode |
| 13 | 000-255 | Internal velocity |
| 14 | 000-255 | External r |
| 151 | 000-255 | Waig |
| six | 000-255 | Waib |
| 17 | 000-255 | External frequency flash |
| 18 | 000-255 | External mode |
| 19 | 000-255 | External velocity |
| 20 | 000-255 | External direction |
| 21 | 000-255 | background color |
| 22 | 000-255 | Background tone light |

CH363 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | Baiyi |
| 2 | 000-255 | White 2 |
| three | 000-255 | White 3 |
| four | 000-255 | R1 |
| five | 000-255 | G1 |
| six | 000-255 | B1 |
| seven | 000-255 | R2 |
| eight | 000-255 | G2 |
| nine | 000-255 | B2 |
| … | … | … … |
| 361 | 000-255 | R120 |
| 362 | 000-255 | G120 |
| 363 | 000-255 | B120 |

Seven, technical parameters:

Voltage: AC100~220V 50/60HZ

Control mode: DMX512, self-propelled, master-slave, voice control and RDM function.

Channels: CH06, CH08, CH12, CH21, CH22, CH363.

Dimming: 32bit 0~100% linear dimming

Features: dyeing+flash+auxiliary light

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: weight: