- Display panel and key definition

Menu key: enter/return to the main menu.

Up key: menu function up selection/parameter increment.

Down key: menu function down selection/parameter decreasing.

Enter key: select the menu function, and confirm and save it when the menu function parameters are modified.

Second, the menu function

After the machine is powered on and reset, press the menu key to enter the main menu (exit after 30 seconds of no operation in the main menu interface). See the main menu menu (5 main menu options in total) for details. Press the Up or Down key to select the main menu, enter the Enter key to modify the function parameters of each menu, and then press the Enter key to save the current function parameters after modification (with power-off memory after saving).

| primary menu | submenu | Menu function description |  |
| --- | --- | --- | --- |
| 1. System Settings | 1. DMX address | Modify the DMX address code up or down (the optional range of address code is 001-512), and press the Enter key to save. The default value is 001. |  |
|  | 2. Encoder | Modify the encoder on/off up or down, save the confirmation key and turn it on by default. |  |
|  | 3. X inversion | Modify the X-reversal on/off up or down, and confirm to save, which is closed by default. |  |
|  | 4. Y inversion | Modify Y-turn on/off up or down, and confirm to save, which is closed by default. |  |
|  | 5. Advanced Settings |  |  |
| 2. Operation mode | 1. Operation mode | Select the running mode DMX/ Voice Control/Auto up or down, and press the Enter key to save. The default DMX is selected. |  |
|  | 2. Channel selection | Select up or down channels CH26, CH38, CH62. CH74. CH194 to confirm the key to save. The default is CH26. |  |
| 3. Manual control | 1. Y axis | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 2, y fine-tuning | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 3, y speed | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 4, focusing | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 5. Dimming | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 6, a strobe | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 7, a red | Modify the brightness of the red light bulb up or down (adjustable range 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 8, a green | Modify the brightness of the green light bulb up or down (adjustable range 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 9, a blue | Modify the brightness of the blue light bulb up or down (adjustable range 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 10, a white | Modify the brightness of the white light bulb up or down (adjustable range 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 11. Mode A | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 12, a speed | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 13, a background color | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 14, a background color light | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 15, b strobe | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 16, b red | Modify the brightness of the red light bulb up or down (adjustable range 000-255), and press the Enter key to save; the default value is 255. |  |
|  | 17, b green | Modify the brightness of the green light bulb up or down (adjustable range 000-255), and press the Enter key to save; the default value is 255. |  |
|  | 18, b blue | Modify the brightness of the blue bulb up or down (adjustable range 000-255), and press the Enter key to save, and the default value is 255. |  |
|  | 19, b white | Modify the brightness of white light beads up or down (adjustable range 000-255), and press the Enter key to save, and the default value is 255. |  |
|  | 20, b yellow | Modify the brightness of blue and yellow bulbs up or down (adjustable range: 000-255), and press the Enter key to save; the default value is 255. |  |
|  | 21. Mode B | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 22, b speed | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 23. direction b | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 24, b background color | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 25, b background color light | Modify the parameter value up or down (adjustable range: 000-255), and press the Enter key to save. The default value is 000. |  |
|  | 26, reset | Yes/no |  |
| 4. Display settings | 1. Language choice | Select Chinese /EN (English) up or down, and press the Enter key to save. The default is Chinese. |  |
|  | 2. Inverted display settings | Select up or down to turn off/on the reverse display setting, and press the Enter key to save, which is turned off by default. |  |
|  | 3. Screen protection | Select Close/Open Screen Saver up or down, and confirm to save, which is turned on by default. |  |
|  | 4. Software version | 069C program identification code, which cannot be modified. |  |
| 5. Factory Settings | Please enter the password for display! Change it to 138 up or down, and then press the Enter key to enter the factory settings. | 1. Y axis calibration | Modify the parameter up or down (range 000-255), and confirm to save it. The default value is 128. |
|  |  | 2, focusing 1 calibration | Modify the focus 1 calibration (000-255) up or down, and confirm the key to save. The default is 128. |
|  |  | 3, focusing 2 calibration | Modify the focus 2 calibration (000-255) up or down, and confirm the key to save. The default is 128. |
|  |  | 4, focusing stroke | Modify the focusing stroke (000-255) up or down, and press the Enter key to save. The default value is 128. |
|  |  | 5. Red | Modify the red light bulb current (032-255) up or down, and press the Enter key to save. The default value is 252. |
|  |  | 6. Green | Modify the green light bulb current up or down (032-255), and press the Enter key to save. The default value is 252. |
|  |  | 7. Blue | Modify the blue light bulb current up or down (032-255), and press the Enter key to save. The default value is 252. |
|  |  | 8. White | Modify the white light bulb current up or down (032-255), and press the Enter key to save. The default value is 252. |
|  |  | 9、LEDNum | Select 000 /0012 up or down, and confirm to save. The default value is 001. |
|  |  | 10. Current | Select 000 /007 up or down, and confirm to save. The default value is 004. |
|  |  | 11. Speed | Select 000 /007 up or down, and confirm to save. The default value is 000. |
|  |  | 12、LOGO | L Select 000 /001 up or down, and confirm to save. The default value is 000. |
|  |  | 13. Temperature | Modify the temperature parameter up or down (range 40-70), and confirm to save it. The default value is 065. |
|  |  | 14. Temperature-Fan | Select up or down to turn off/on temperature-fan control, and press the Enter key to save and turn it on by default. |
|  |  | 15、LED | Select LEDa/ledb in sequence for led lamp beads. |
|  |  | 16、ETO | ETO |

Third, master-slave control

There are two or more identical lamps connected by DMX three-core signal lines, all lamps are modified to any address code of 001~512, and any one is set as the master, while other lamps are slaves, and all slave displays do not flicker; When the master adjusts the effects of gradual change, pulse change, jump, voice control and self-walking, all slaves synchronize the effects of gradual change, pulse change, jump, voice control and self-walking.

Special attention: 1. Only one host can be set for a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

2. All lamps and lanterns must be used when the DMX512 console is closed.

Fourth, DMX512 control

Address codes of all lamps and lanterns are set, and all lamps and lanterns are connected to the DMX512 console in parallel with DMX three-core signal lines. The address code stops flashing, indicating that the DMX512 console signal has been sent to the lamps and lanterns. According to the channel description, the DMX512 console is used to control related functions.

CH26 Channel Description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | electrical machinery |
| 2 | 000-255 | Motor fine tuning |
| three | 000-255 | Motor speed |
| four | 000-255 | focus |
| five | 000-255 | aiming |
| six | 000-255 | Main light stroboscopic |
| seven | 000-255 | Main light red |
| eight | 000-255 | Main light green |
| nine | 000-255 | Main light blue |
| 10 | 000-255 | Main light white |
| 11 | 000-255 | Main light mode |
| 12 | 000-255 | Principal light velocity |
| 13 | 000-255 | Main light background color |
| 14 | 000-255 | Main light background tone light |
| 15 | 000-255 | Auxiliary light strobe |
| 16 | 000-255 | Auxiliary light red |
| 17 | 000-255 | Auxiliary light green |
| 18 | 000-255 | Auxiliary light blue |
| 19 | 000-255 | Auxiliary light white |
| 20 | 000-255 | Auxiliary light yellow |
| 21 | 000-255 | Auxiliary light mode |
| 22 | 000-255 | Auxiliary light speed |
| 23 | 000-255 | Auxiliary light direction |
| 24 | 000-255 | Auxiliary light back scenery |
| 25 | 000-255 | Auxiliary light background tone light |
| 26 | 000-255 | reset |

CH38 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | electrical machinery |
| 2 | 000-255 | Motor fine tuning |
| three | 000-255 | Motor speed |
| four | 000-255 | focus |
| five | 000-255 | aiming |
| six | 000-255 | Main light stroboscopic |
| seven | 000-255 | Main light red |
| eight | 000-255 | Main light green |
| nine | 000-255 | Main light blue |
| 10 | 000-255 | Main light white |
| 11 | 000-255 | Main light mode |
| 12 | 000-255 | Principal light velocity |
| 13 | 000-255 | Main light background color |
| 14 | 000-255 | Main light background tone light |
| 15 | 000-255 | Auxiliary light strobe |
| 16 | 000-255 | Auxiliary light red |
| 17 | 000-255 | Auxiliary light green |
| 18 | 000-255 | Auxiliary light blue |
| 19 | 000-255 | Auxiliary light white |
| 20 | 000-255 | Auxiliary light yellow |
| 21 | 000-255 | Auxiliary light mode |
| 22 | 000-255 | Auxiliary light speed |
| 23 | 000-255 | Auxiliary light direction |
| 24 | 000-255 | Auxiliary light back scenery |
| 25 | 000-255 | Auxiliary light background tone light |
| 26 | 000-255 | reset |
| 27 | 000-255 | Main light LED1 |
| 28 | 000-255 | Main light LED2 |
| 29 | 000-255 | Main light LED3 |
| 30 | 000-255 | Main light LED4 |
| 31 | 000-255 | Main light LED5 |
| 32 | 000-255 | Main light LED6 |
| 33 | 000-255 | Main light LED7 |
| 34 | 000-255 | Main light LED8 |
| 35 | 000-255 | Main light LED9 |
| 36 | 000-255 | Main light LED10 |
| 37 | 000-255 | Main light LED11 |
| 38 | 000-255 | Main light LED12 |

CH62 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | electrical machinery |
| 2 | 000-255 | Motor fine tuning |
| three | 000-255 | Motor speed |
| four | 000-255 | focus |
| five | 000-255 | aiming |
| six | 000-255 | Main light stroboscopic |
| seven | 000-255 | Main light red |
| eight | 000-255 | Main light green |
| nine | 000-255 | Main light blue |
| 10 | 000-255 | Main light white |
| 11 | 000-255 | Main light mode |
| 12 | 000-255 | Principal light velocity |
| 13 | 000-255 | Main light background color |
| 14 | 000-255 | Main light background tone light |
| 15 | 000-255 | Auxiliary light strobe |
| 16 | 000-255 | Auxiliary light red |
| 17 | 000-255 | Auxiliary light green |
| 18 | 000-255 | Auxiliary light blue |
| 19 | 000-255 | Auxiliary light white |
| 20 | 000-255 | Auxiliary light yellow |
| 21 | 000-255 | Auxiliary light mode |
| 22 | 000-255 | Auxiliary light speed |
| 23 | 000-255 | Auxiliary light direction |
| 24 | 000-255 | Auxiliary light back scenery |
| 25 | 000-255 | Auxiliary light background tone light |
| 26 | 000-255 | reset |
| 27 | 000-255 | Main light LED1 |
| 28 | 000-255 | Main light LED2 |
| 29 | 000-255 | Main light LED3 |
| 30 | 000-255 | Main light LED4 |
| 31 | 000-255 | Main light LED5 |
| 32 | 000-255 | Main light LED6 |
| 33 | 000-255 | Main light LED7 |
| 34 | 000-255 | Main light LED8 |
| 35 | 000-255 | Main light LED9 |
| 36 | 000-255 | Main light LED10 |
| 37 | 000-255 | Main light LED11 |
| 38 | 000-255 | Main light LED12 |
| 39 | 000-255 | Auxiliary light LED1 |
| 40 | 000-255 | Auxiliary light LED2 |
| 41 | 000-255 | Auxiliary light LED3 |
| forty-two | 000-255 | Auxiliary light LED4 |
| 43 | 000-255 | Auxiliary light LED5 |
| forty-four | 000-255 | Auxiliary light LED6 |
| 45 | 000-255 | Auxiliary light LED7 |
| 46 | 000-255 | Auxiliary light LED8 |
| 47 | 000-255 | Auxiliary light LED9 |
| 48 | 000-255 | Auxiliary light LED10 |
| forty-nine | 000-255 | Auxiliary light LED11 |
| 50 | 000-255 | Auxiliary light LED12 |
| 51 | 000-255 | Auxiliary light LED13 |
| fifty-two | 000-255 | Auxiliary light LED14 |
| 53 | 000-255 | Auxiliary light LED15 |
| 54 | 000-255 | Auxiliary light LED16 |
| 55 | 000-255 | Auxiliary light LED17 |
| fifty-six | 000-255 | Auxiliary light LED18 |
| 57 | 000-255 | Auxiliary light LED19 |
| 58 | 000-255 | Auxiliary light LED20 |
| 59 | 000-255 | Auxiliary light LED21 |
| 60 | 000-255 | Auxiliary light LED22 |
| 61 | 000-255 | Auxiliary light LED23 |
| 62 | 000-255 | Auxiliary light LED24 |

CH74 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | electrical machinery |
| 2 | 000-255 | Motor fine tuning |
| three | 000-255 | Motor speed |
| four | 000-255 | focus |
| five | 000-255 | aiming |
| six | 000-255 | Main light stroboscopic |
| seven | 000-255 | Main light red |
| eight | 000-255 | Main light green |
| nine | 000-255 | Main light blue |
| 10 | 000-255 | Main light white |
| 11 | 000-255 | Main light mode |
| 12 | 000-255 | Principal light velocity |
| 13 | 000-255 | Main light background color |
| 14 | 000-255 | Main light background tone light |
| 15 | 000-255 | Auxiliary light strobe |
| 16 | 000-255 | Auxiliary light red |
| 17 | 000-255 | Auxiliary light green |
| 18 | 000-255 | Auxiliary light blue |
| 19 | 000-255 | Auxiliary light white |
| 20 | 000-255 | Auxiliary light yellow |
| 21 | 000-255 | Auxiliary light mode |
| 22 | 000-255 | Auxiliary light speed |
| 23 | 000-255 | Auxiliary light direction |
| 24 | 000-255 | Auxiliary light back scenery |
| 25 | 000-255 | Auxiliary light background tone light |
| 26 | 000-255 | reset |
| 27 | 000-255 | Main light red 1 |
| 28 | 000-255 | Main light green 1 |
| 29 | 000-255 | Main light blue 1 |
| 30 | 000-255 | Main light white 1 |
| 31 | 000-255 | Main light red 2 |
| 32 | 000-255 | Main light green 2 |
| 33 | 000-255 | Main light blue 2 |
| 34 | 000-255 | Main light white 2 |
| 35 | 000-255 | Main light red 3 |
| 36 | 000-255 | Main light green 3 |
| 37 | 000-255 | Main light blue 3 |
| 38 | 000-255 | Main light white 3 |
| 39 | 000-255 | Main light red 4 |
| 40 | 000-255 | Main light green 4 |
| 41 | 000-255 | Main light blue 4 |
| forty-two | 000-255 | Main light white 4 |
| 43 | 000-255 | Main light red 5 |
| forty-four | 000-255 | Main light green 5 |
| 45 | 000-255 | Main light blue 5 |
| 46 | 000-255 | Main light white 5 |
| 47 | 000-255 | Main light red 6 |
| 48 | 000-255 | Main light green 6 |
| forty-nine | 000-255 | Main light blue 6 |
| 50 | 000-255 | Main light white 6 |
| 51 | 000-255 | Main light red 7 |
| fifty-two | 000-255 | Main light green 7 |
| 53 | 000-255 | Main light blue 7 |
| 54 | 000-255 | Main light white 7 |
| 55 | 000-255 | Main light red 8 |
| fifty-six | 000-255 | Main light green 8 |
| 57 | 000-255 | Main light blue 8 |
| 58 | 000-255 | Main light white 8 |
| 59 | 000-255 | Main light red 9 |
| 60 | 000-255 | Main light green 9 |
| 61 | 000-255 | Main light blue 9 |
| 62 | 000-255 | Main light white 9 |
| 63 | 000-255 | Main light red 10 |
| 64 | 000-255 | Main light green 10 |
| 65 | 000-255 | Main light blue 10 |
| 66 | 000-255 | Main light white 10 |
| 67 | 000-255 | Main light red 11 |
| sixty-eight | 000-255 | Main light green 11 |
| sixty-nine | 000-255 | Main light blue 11 |
| 70 | 000-255 | Main light white 11 |
| 71 | 000-255 | Main light red 12 |
| seventy-two | 000-255 | Main light green 12 |
| 73 | 000-255 | Main light blue 12 |
| 74 | 000-255 | Main light white 12 |

CH194 channel description:

| passage | Channel value | fundamental function |
| --- | --- | --- |
| one | 000-255 | electrical machinery |
| 2 | 000-255 | Motor fine tuning |
| three | 000-255 | Motor speed |
| four | 000-255 | focus |
| five | 000-255 | aiming |
| six | 000-255 | Main light stroboscopic |
| seven | 000-255 | Main light red |
| eight | 000-255 | Main light green |
| nine | 000-255 | Main light blue |
| 10 | 000-255 | Main light white |
| 11 | 000-255 | Main light mode |
| 12 | 000-255 | Principal light velocity |
| 13 | 000-255 | Main light background color |
| 14 | 000-255 | Main light background tone light |
| 15 | 000-255 | Auxiliary light strobe |
| 16 | 000-255 | Auxiliary light red |
| 17 | 000-255 | Auxiliary light green |
| 18 | 000-255 | Auxiliary light blue |
| 19 | 000-255 | Auxiliary light white |
| 20 | 000-255 | Auxiliary light yellow |
| 21 | 000-255 | Auxiliary light mode |
| 22 | 000-255 | Auxiliary light speed |
| 23 | 000-255 | Auxiliary light direction |
| 24 | 000-255 | Auxiliary light back scenery |
| 25 | 000-255 | Auxiliary light background tone light |
| 26 | 000-255 | reset |
| 27 | 000-255 | Main light red 1 |
| 28 | 000-255 | Main light green 1 |
| 29 | 000-255 | Main light blue 1 |
| 30 | 000-255 | Main light white 1 |
| 31 | 000-255 | Main light red 2 |
| 32 | 000-255 | Main light green 2 |
| 33 | 000-255 | Main light blue 2 |
| 34 | 000-255 | Main light white 2 |
| 35 | 000-255 | Main light red 3 |
| 36 | 000-255 | Main light green 3 |
| 37 | 000-255 | Main light blue 3 |
| 38 | 000-255 | Main light white 3 |
| 39 | 000-255 | Main light red 4 |
| 40 | 000-255 | Main light green 4 |
| 41 | 000-255 | Main light blue 4 |
| forty-two | 000-255 | Main light white 4 |
| 43 | 000-255 | Main light red 5 |
| forty-four | 000-255 | Main light green 5 |
| 45 | 000-255 | Main light blue 5 |
| 46 | 000-255 | Main light white 5 |
| 47 | 000-255 | Main light red 6 |
| 48 | 000-255 | Main light green 6 |
| forty-nine | 000-255 | Main light blue 6 |
| 50 | 000-255 | Main light white 6 |
| 51 | 000-255 | Main light red 7 |
| fifty-two | 000-255 | Main light green 7 |
| 53 | 000-255 | Main light blue 7 |
| 54 | 000-255 | Main light white 7 |
| 55 | 000-255 | Main light red 8 |
| fifty-six | 000-255 | Main light green 8 |
| 57 | 000-255 | Main light blue 8 |
| 58 | 000-255 | Main light white 8 |
| 59 | 000-255 | Main light red 9 |
| 60 | 000-255 | Main light green 9 |
| 61 | 000-255 | Main light blue 9 |
| 62 | 000-255 | Main light white 9 |
| 63 | 000-255 | Main light red 10 |
| 64 | 000-255 | Main light green 10 |
| 65 | 000-255 | Main light blue 10 |
| 66 | 000-255 | Main light white 10 |
| 67 | 000-255 | Main light red 11 |
| sixty-eight | 000-255 | Main light green 11 |
| sixty-nine | 000-255 | Main light blue 11 |
| 70 | 000-255 | Main light white 11 |
| 71 | 000-255 | Main light red 12 |
| seventy-two | 000-255 | Main light green 12 |
| 73 | 000-255 | Main light blue 12 |
| 74 | 000-255 | Main light white 12 |
| 75 | 000-255 | Auxiliary light red 1 |
| 76 | 000-255 | Auxiliary light green 1 |
| 77 | 000-255 | Auxiliary light blue 1 |
| seventy-eight | 000-255 | Auxiliary light white 1 |
| 79 | 000-255 | Auxiliary light yellow 1 |
| 80 | 000-255 | Auxiliary light red 2 |
| 81 | 000-255 | Auxiliary light green 2 |
| 82 | 000-255 | Auxiliary light blue 2 |
| 83 | 000-255 | Auxiliary light white 2 |
| 84 | 000-255 | Auxiliary light yellow 2 |
| eighty-five | 000-255 | Auxiliary light red 3 |
| 86 | 000-255 | Auxiliary light green 3 |
| 87 | 000-255 | Auxiliary light blue 3 |
| 88 | 000-255 | Auxiliary light white 3 |
| eighty-nine | 000-255 | Auxiliary light yellow 3 |
| 90 | 000-255 | Auxiliary light red 4 |
| 91 | 000-255 | Auxiliary light green 4 |
| 92 | 000-255 | Auxiliary light blue 4 |
| 93 | 000-255 | Auxiliary light white 4 |
| 94 | 000-255 | Auxiliary light yellow 4 |
| 95 | 000-255 | Auxiliary light red 5 |
| 96 | 000-255 | Auxiliary light green 5 |
| 97 | 000-255 | Auxiliary light blue 5 |
| 98 | 000-255 | Auxiliary light white 5 |
| 99 | 000-255 | Auxiliary light yellow 5 |
| 100 | 000-255 | Auxiliary light red 6 |
| 101 | 000-255 | Auxiliary light green 6 |
| 102 | 000-255 | Auxiliary light blue 6 |
| 103 | 000-255 | Auxiliary light white 6 |
| 104 | 000-255 | Auxiliary light yellow 6 |
| 105 | 000-255 | Auxiliary light red 7 |
| 106 | 000-255 | Auxiliary light green 7 |
| 107 | 000-255 | Auxiliary light blue 7 |
| 108 | 000-255 | Auxiliary light white 7 |
| 109 | 000-255 | Auxiliary light yellow 7 |
| 110 | 000-255 | Auxiliary light red 8 |
| 111 | 000-255 | Auxiliary light green 8 |
| 112 | 000-255 | Auxiliary light blue 8 |
| 113 | 000-255 | Auxiliary light white 8 |
| 114 | 000-255 | Auxiliary light yellow 8 |
| 115 | 000-255 | Auxiliary light red 9 |
| 116 | 000-255 | Auxiliary light green 9 |
| 117 | 000-255 | Auxiliary light blue 9 |
| 118 | 000-255 | Auxiliary light white 9 |
| 119 | 000-255 | Auxiliary light yellow 9 |
| 120 | 000-255 | Auxiliary light red 10 |
| 121 | 000-255 | Auxiliary light green 10 |
| 122 | 000-255 | Auxiliary light blue 10 |
| 123 | 000-255 | Auxiliary light white 10 |
| 124 | 000-255 | Auxiliary light yellow 10 |
| 125 | 000-255 | Auxiliary light red 11 |
| 126 | 000-255 | Auxiliary light green 11 |
| 127 | 000-255 | Auxiliary light blue 11 |
| 128 | 000-255 | Auxiliary light white 11 |
| 129 | 000-255 | Auxiliary light yellow 11 |
| 130 | 000-255 | Auxiliary light red 12 |
| 131 | 000-255 | Auxiliary light green 12 |
| 132 | 000-255 | Auxiliary light blue 12 |
| 133 | 000-255 | Auxiliary light white 12 |
| 134 | 000-255 | Auxiliary light yellow 12 |
| 135 | 000-255 | Auxiliary light red 13 |
| 136 | 000-255 | Auxiliary light green 13 |
| 137 | 000-255 | Auxiliary light blue 13 |
| 138 | 000-255 | Auxiliary light white 13 |
| 139 | 000-255 | Auxiliary light yellow 13 |
| 140 | 000-255 | Auxiliary light red 14 |
| 141 | 000-255 | Auxiliary light green 14 |
| 142 | 000-255 | Auxiliary light blue 14 |
| 143 | 000-255 | Auxiliary light white 14 |
| 144 | 000-255 | Auxiliary light yellow 14 |
| 145 | 000-255 | Auxiliary light red 15 |
| 146 | 000-255 | Auxiliary light green 15 |
| 147 | 000-255 | Auxiliary light blue 15 |
| 148 | 000-255 | Auxiliary light white 15 |
| 149 | 000-255 | Auxiliary light yellow 15 |
| 150 | 000-255 | Auxiliary light red 16 |
| 151 | 000-255 | Auxiliary light green 16 |
| 152 | 000-255 | Auxiliary light blue 16 |
| 153 | 000-255 | Auxiliary light white 16 |
| 154 | 000-255 | Auxiliary light yellow 16 |
| 155 | 000-255 | Auxiliary light red 17 |
| 156 | 000-255 | Auxiliary light green 17 |
| 157 | 000-255 | Auxiliary light blue 17 |
| 158 | 000-255 | Auxiliary light white 17 |
| 159 | 000-255 | Auxiliary light yellow 17 |
| 160 | 000-255 | Auxiliary light red 18 |
| 161 | 000-255 | Auxiliary light green 18 |
| 162 | 000-255 | Auxiliary light blue 18 |
| 163 | 000-255 | Auxiliary light white 18 |
| 164 | 000-255 | Auxiliary light yellow 18 |
| 165 | 000-255 | Auxiliary light red 19 |
| 166 | 000-255 | Auxiliary light green 19 |
| 167 | 000-255 | Auxiliary light blue 19 |
| 168 | 000-255 | Fuguangbai 19 |
| 169 | 000-255 | Auxiliary light yellow 19 |
| 170 | 000-255 | Auxiliary light red 20 |
| 171 | 000-255 | Auxiliary light green 20 |
| 172 | 000-255 | Auxiliary light blue 20 |
| 173 | 000-255 | Auxiliary light white 20 |
| 174 | 000-255 | Auxiliary light yellow 20 |
| 175 | 000-255 | Auxiliary light red 21 |
| 176 | 000-255 | Auxiliary light green 21 |
| 177 | 000-255 | Auxiliary light blue 21 |
| 178 | 000-255 | Auxiliary light white 21 |
| 179 | 000-255 | Auxiliary light yellow 21 |
| one hundred and eighty | 000-255 | Auxiliary light red 22 |
| 181 | 000-255 | Auxiliary light green 22 |
| 182 | 000-255 | Auxiliary light blue 22 |
| 183 | 000-255 | Auxiliary light white 22 |
| 184 | 000-255 | Auxiliary light yellow 22 |
| 185 | 000-255 | Auxiliary light red 23 |
| 186 | 000-255 | Auxiliary light green 23 |
| 187 | 000-255 | Auxiliary light blue 23 |
| 188 | 000-255 | Auxiliary light white 23 |
| 189 | 000-255 | Auxiliary light yellow 23 |
| 190 | 000-255 | Auxiliary light red 24 |
| 191 | 000-255 | Auxiliary light green 24 |
| 192 | 000-255 | Auxiliary light blue 24 |
| 193 | 000-255 | Auxiliary light white 24 |
| 194 | 000-255 | Auxiliary light yellow 24 |

V. Technical parameters:

Voltage: AC100~240V 50/60HZ

Power: 500W

Lamp beads: 12 40W four-in-one foot Tim Xin lamp beads 144 0.3W 5050 three-in-one lamp beads.

Control mode: DMX512, self-propelled, master-slave, voice control and RDM function.

Channels: CH26, CH38, ch62.ch74.ch194.

Dimming: 32bit 0~100% linear dimming

Features: Y shake head+beam+flash+auxiliary light.

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size:

weight: