7×7-49 matrices

| MENU: Select the function mode or return to the previous menu. |  |  |
| --- | --- | --- |
| UP: select up |  |  |
| DOWN: select down. |  |  |
| ENTER: confirm |  |  |

Technical parameters: Voltage: AC100~240V 50/60HZ Power: 180W.

Lamp beads: 441pcs 5050RGB(0.25w) +49pcs 5730U(5w) Control mode: DMX512,Auto, master-slave, voice control, with RDM function.

Channels: CH4, CH13 and CH14

Linear dimming features: 441RGB+49W single point control.

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Connection mode: DMX512 input/output/power input/output IP grade: IP20. Primary menu; Sub menu 1; Sub menu 2; Remarks. Address; D001/D512; Address code, press enter key to enter sub menu 1, press up key or down key to select the address code, and press enter key to save the address code.. CHAL; CH5、CH13、CH14; Channel, press OK to enter sub menu 1, press the up key or the down key to select the channel, and press OK to save the channel.. AUTO; CA50; 01-50; Press the Enter key to enter the second level, press the Up key or the Down key to select the self-propelled effect (254 kinds of effects), and then press the Enter key to enter the third level. The third level menu includes: s00-s50 speed, c000-c255 color (most effects support 255 color styles, and the unsupported effect is that the interval value switches different original colors). In the third level menu, press the Enter key to switch between speed and color and save the self-propelled type and speed, and press.. CB50; 01-50. CC50; 01-50. CD50; 01-50. CE50; 01-50. CF50; 01-50. CG50; 01-50. CH50; 01-50. CJ50; 01-50. SOUD; SO01-SO99; Voice control mode secondary menu: A00-A99 Sensitivity effect N000-N254. STAT; r000; r000-r255; Red manual dimming. g000; g000-g255; Green manual dimming. b000; b000-b255; Blue manual dimming. w000; w000-w255; White manual dimming. c000; c000-c094; character. d000; d000-d003; direction. t000; t000-b255; stroboscopic. Rest; On/Off; Restore factory settings.

| backstage |  |  |
| --- | --- | --- |
| r000 | r000-r255 | Power adjustment of red lamp bead |
| g000 | g000-g255 | Power adjustment of green lamp bead |
| b000 | b000-b255 | Power adjustment of blue lamp bead |
| w000 | w000-w255 | White light bulb power adjustment |
| n000 | n000-n255 | Reduce power |
| l000 | l000-l255 | Maximum rate |
| OFF | OFF/ON | send |

| Channel 4 | Channel value | fundamental function |  |
| --- | --- | --- | --- |
| 1 | 000-255 | R red light bulb dimming |  |
| 2 | 000-255 | G green light bulb dimming |  |
| 3 | 000-255 | B blue light bulb dimming |  |
| 4 | 000-255 | W white light bulb dimming |  |
| Channel 13 | Channel value | fundamental function |  |
| 1 | 000-255 | Total dimming |  |
| 2 | 000-255 | stroboscopic |  |
| 3 | 000-255 | Position lever |  |
| 4 | 000-255 | Color bar |  |
| 5 | 000-255 | Effect, 255 is voice control |  |
| 6 | 000-255 | speed |  |
| 7 | 000-255 | White light flashing |  |
| 8 | 000-255 | White light effect |  |
| 9 | 000-255 | White light speed |  |
| 10 | 000-255 | R red light bulb dimming |  |
| 11 | 000-255 | G green light bulb dimming |  |
| 12 | 000-255 | B blue light bulb dimming |  |
| 13 | 000-255 | W white light bulb dimming |  |
| Channel 14 | Channel value | fundamental function |  |
| 1 | 000-255 | Total dimming |  |
| 2 | 000-255 | stroboscopic |  |
| 3 | 000-255 | direction |  |
| 4 | 000-255 | Color/style |  |
| 5 | 000-255 | Translation (0-200), automatic left translation (200-225), automatic right translation (225-255) | Note: After turning on the general dimming, you can complete the writing of words only by operating the No.5 and No.6 levers. After writing, push the No.5 rod to more than 200, and then you can pan and display the content written by it. |
| 6 | 000-255 | Letter/number/symbol selection |  |
| 7 | 000-255 | speed |  |
| 8 | 000-255 | Auxiliary lamp stroboscopic |  |
| 9 | 000-255 | Auxiliary lamp effect |  |
| 10 | 000-255 | Auxiliary lamp speed |  |
| 11 | 000-255 | R red light bulb dimming |  |
| 12 | 000-255 | G green light bulb dimming |  |
| 13 | 000-255 | B blue light bulb dimming |  |
| 14 | 000-255 | W white light bulb dimming |  |