beam light

operation instruction

catalogue

1.尺寸图2

2.主要功能3

3.DMX 通道表3

4.显示板功能操作6

5.操作说明7

6.故障排除8

# major function

- Input voltage: AC 100V-264V/47-63HZ.

- Control mode: DMX512 control/built-in self-propelled/voice-activated self-propelled/master-slave.

- Channel mode: 16 channels

- A color wheel with 13 colors and white light, and a pattern wheel with 13 patterns and white light. The color wheel has a half-color effect of running water rolling. The pattern disk has the effect of running water rolling and shaking.

- Two prisms have the effects of arbitrary angle positioning and positive and negative rotation.

- Strobe has a variety of stroboscopic functions, and it is smooth and jitter-free when the light is switched on and off gradually.

- Protection grade: IP20

# DMX channel table

- DMX channel table

| 16 channels | numerical value | function |
| --- | --- | --- |
| CH1 | 0-255 | level |
| CH2 | 0-255 | perpendicular/vertical |
| CH3 | 0-255 | Horizontal fine adjustment |
| CH4 | 0-255 | Vertical fine tuning |
| CH5 | 0-255 | Horizontal vertical speed, fast to slow |
| CH6 | 0-127 | No atomization |
|  | 128-255 | Atomization 100% |
| CH7 | 0-5 | Closed light |
|  | 6-20 | consecrate |
|  | 21-70 | Synchronous strobe is slow to fast. |
|  | 71-100 | Asynchronous strobe is slow to fast. |
|  | 101-150 | Gradually open and close slowly to fast. |
|  | 151-190 | Turn on gradually, turn off quickly, turn off slowly, and turn on quickly. |
|  | 191-230 | Turn off gradually, drive slowly, and drive quickly. |
|  | 231-255 | consecrate |
| CH8 | 0-255 | Dimming, 0~100% |
| CH9 | 0-5 | Colorless disk |
|  | 6-10 | White light to color wheel 1 |
|  | 11-15 | Color wheel 1 |
|  | 16-20 | Color wheel 1 to color wheel 2 |
|  | 21-25 | Color wheel 2 |
|  | 26-30 | Color wheel 2 to color wheel 3 |
|  | 31-35 | Color wheel 3 |
|  | 36-40 | Color wheel 3 to color wheel 4 |
|  | 41-45 | Color wheel 4 |
|  | 46-50 | Color wheel 4 to color wheel 5 |
|  | 51-55 | Color wheel 5 |
|  | 56-60 | Color wheel 5 to color wheel 6 |
|  | 61-65 | Color wheel 6 |
|  | 66-70 | Color wheel 6 to color wheel 7 |
|  | 71-75 | Color wheel 7 |
|  | 76-80 | Color wheel 7 to color wheel 8 |
|  | 81-85 | Color wheel 8 |
|  | 86-90 | Color wheel 8 to color wheel 9 |
|  | 91-95 | Color wheel 9 |
|  | 96-100 | Color wheel 9 to color wheel 10 |
|  | 101-105 | Color wheel 10 |
|  | 106-110 | Color wheel 10 to color wheel 11 |
|  | 111-115 | Color wheel 11 |
|  | 116-120 | Color wheel 11 to color wheel 12 |
|  | 121-125 | Color wheel 12 |
|  | 126-130 | Color wheel 12 to color wheel 13 |
|  | 131-135 | Color wheel 13 |
|  | 136-140 | Color wheel 13 to white light |
|  | 141-190 | Turn slowly to fast. |
|  | 191-205 | stop |
|  | 206-255 | Reverse slowly to quickly. |
| CH10 | 0-5 | No pattern |
|  | 6-10 | Pattern 1 |
|  | 11-15 | Pattern 2 |
|  | 16-20 | Pattern 3 |
|  | 21-25 | Pattern 4 |
|  | 26-30 | Pattern 5 |
|  | 31-35 | Pattern 6 |
|  | 36-40 | Pattern 7 |
|  | 41-45 | Pattern 8 |
|  | 46-50 | Pattern 9 |
|  | 51-55 | Pattern 10 |
|  | 56-60 | Pattern 11 |
|  | 61-65 | Pattern 12 |
|  | 66-70 | Pattern 13 |
|  | 71-77 | Pattern 1 jitter is slow to fast. |
|  | 78-84 | Pattern 2 jitter is slow to fast. |
|  | 85-91 | Pattern 3 jitter is slow to fast. |
|  | 92-98 | Pattern 4 jitter is slow to fast. |
|  | 99-105 | Pattern 5 jitter is slow to fast. |
|  | 106-112 | Pattern 6 jitter is slow to fast. |
|  | 113-119 | Pattern 7 jitter is slow to fast. |
|  | 120-126 | Pattern 8 jitter is slow to fast. |
|  | 127-133 | Pattern 9 jitter is slow to fast. |
|  | 134-140 | The pattern 10 jitter is slow to fast. |
|  | 141-147 | The pattern 11 jitter is slow to fast. |
|  | 148-154 | The pattern 12 jitter is slow to fast. |
|  | 155-161 | The pattern 13 jitter is slow to fast. |
|  | 162-204 | Turn slowly to fast. |
|  | 205-212 | stop |
|  | 213-255 | Reverse slowly to quickly. |
| CH11 | 0-127 | Prism-free 1 |
|  | 128-255 | Prism 1 cuts in |
| CH12 | 0-127 | Prism-free 2 |
|  | 128-255 | Prism 2 cuts in |
| CH13 | 0-2 | Prism rotation stops |
|  | 3-150 | Prism angle 0-400 degrees |
|  | 151-200 | The prism is turning slowly to fast. |
|  | 201-205 | Prism rotation stops |
|  | 206-255 | Prism reversal is slow to fast. |
| CH14 | 0-127 | Wuqicai |
|  | 128-255 | Colorful 100% (colorful is invalid when there is fog) |
| CH15 | 0-255 | Focus, far to near |
| CH16 | 0-99 | without |
|  | 100-105 | The light bulb is turned off for at least 5S. |
|  | 106-199 | without |
|  | 200-205 | The light bulb is turned on for at least 5S. |
|  | 206-239 | without |
|  | 240-255 | All motors are reset for at least 3S. |

# Display panel function operation

- MENU: Cycle switching at the top menu, and return to the previous menu at other levels.

- ENTER: enter the next menu or confirm and save the selected parameters.

- UP: menu up, switch the selected parameter, or increase the value.

- DOWN: menu down, switch the selected parameter, or decrease the value.

Menu structure

| Top menu | first step | secondary | third stage |
| --- | --- | --- | --- |
| address code | 001-512 |  |  |
| lamp bulb | Light bulb control | No operation and exit/turn on/turn off the light bulb |  |
|  | Turn on the light | On/off |  |
|  | Light up after reset. | On/off |  |
|  | The bulb is controlled by DMX. | On/off |  |
| Parameter setting | Channel mode setting | 16 channels |  |
|  | XY error correction enabled | On/off |  |
|  | X-axis inversion | On/off |  |
|  | Y-axis inversion | On/off |  |
|  | XY axis exchange | On/off |  |
|  | Acoustic sensitivity | 1-15-30 (bigger is more sensitive) |  |
| display setting | language | ENGLISH/ Chinese |  |
|  | Display reversal | Yes/no |  |
|  | Backlight setting | Normally On /30 Seconds Off |  |
| Manual control | X axis | 0-255 |  |
|  | X-axis fine tuning | 0-255 |  |
|  | Y axis | 0-255 |  |
|  | Y axis fine tuning | 0-255 |  |
|  | XY axis velocity | 0-255 |  |
|  | stroboscopic | 0-255 |  |
|  | aiming | 0-255 |  |
|  | pattern | 0-255 |  |
|  | colour | 0-255 |  |
|  | focus | 0-255 |  |
|  | Prism 1 | 0-255 |  |
|  | Prism 1 rotates | 0-255 |  |
|  | Prism 2 | 0-255 |  |
|  | Prism 2 rotates | 0-255 |  |
|  | Atomization cut-in | 0-255 |  |
|  | Colorful cut-in | 0-255 |  |
| Self-propelled mode | Host built-in self-propelled | Running ... * |  |
|  | Single machine built-in self-propelled | Running ... * |  |
|  | The main engine is voice-activated and self-propelled | Running ... * |  |
|  | Single machine voice control self-propelled | Running ... * |  |
| Restore factory settings | Yes/no |  |  |
| information | Current bulb status | On/off |  |
|  | Light bulb turn-on time | Xxxx hours |  |
|  |  | clear | Yes/no |
|  | Starting time of lamps | Xxxx hours |  |
|  |  | clear | Yes/no |
|  | RDM UID code | xxx ... |  |
|  | software release | Vx.x |  |
|  | Reset detection information | X-axis optical coupler |  |
|  |  | Y-axis optocoupler |  |
|  |  | X axis hall |  |
|  |  | Y axis hall |  |
|  |  | Focusing hall |  |
|  |  | Pattern hall |  |
|  |  | Color wheel hall |  |
| reset | Yes/no |  |  |
| Send native parameters | Yes/no |  |  |

# handling instruction

- When the entry menu is marked with "*" and the position of "*" will be displayed on the display screen, it will jump to this position and run by itself after the next boot. If the master operation mode is selected at this time, instead of a single machine, data will be sent to the outside at the same time. Other lamps do not need to be set as slaves, and any address code will receive this data and run synchronously.

- When factory settings are restored, only the underlined parameters in the menu will be restored, and other parameters will not be automatically restored or cleared.

- [Send Local Parameters] menu, parameter settings with underlined items in this machine can be sent to another or more lamps through DMX data cable to avoid repeated setting work. When the received lamp receives the correct parameters, it will generate a reset.

# debugging

| breakdown | solution |
| --- | --- |
| The lamps and lanterns are not electrified and have no response at all. | - Check the fuse for damage. - Check whether the AC voltage is normal. - The switching power supply inside the lamp body may be damaged. - The control panel inside the lamp body may be damaged. |
| Uncontrolled lamps and lanterns | - Check whether the DMX signal is normal. When it is normal, the DMX signal indicator on the display panel will flash. - Check whether the address code is set correctly. - Check whether the channel modeler is set correctly. - Check whether the DMX signal line is too long, or whether there are too many lamps in series. |
| The XY axis is not in normal position. | - The optocoupler sensor may be damaged or improperly installed, so that the original correct position will be corrected. Set it to [No] in the [XY Error Correction Enable] option in the menu. Either adjust the optical coupler sensor circuit board to a proper position or replace this circuit board. |