Large electronic display  Instructions for the use of stage lighting equipment

Beam Lamp

(Please read the instructions carefully and manage them properly before using the product)

In order to use this product correctly and safely, please read the instructions carefully before installing and using this product. This manual contains important installation and application information. Please strictly follow the instructions when installing and operating the product. At the same time, please keep this manual properly.

Our company's  beam lamp A new beautiful high-temperature-resistant metal body is used. This product is designed and produced in strict accordance with CE standards. It conforms to the international standard DMX512 signal protocol. It can be used alone or online. It has the characteristics of fast rotation, low noise and powerful functions. It is suitable for small and medium-sized concerts, theaters, studios, nightclubs, bars and other places.

Please carefully remove the packaging, check whether the product is damaged during transportation after the packaging is removed, and check whether the following contents are complete. 。

Beam lamp--------1 instruction manual--------1 book

Power cord--------1 signal line--------1

This product is in good condition before leaving the factory. In order to keep the product in good condition and ensure safe operation, users should follow the safety matters and the warnings in this manual.

Important:Damage caused by non-compliance with this manual is not covered by the warranty. The supplier is not responsible for the resulting product problems.

If the product has been exposed to an extremely unstable temperature environment (such as after transportation), please do not connect the product to the power supply immediately, because water droplets caused by temperature changes may damage the product. Please use it after the product returns to normal temperature.

This product can Used in the AC110-240V voltage range, it is a product for indoor use. Please make sure that the voltage used is not higher than the range that the product can withstand!! The power plug must be plugged into a protective I.Kind of socket. The green or tea-blue conductor must be grounded.

Please check this product and power cord frequently. Make sure that the power cord is not folded or scratched, and is not connected to other cables! Special attention should be paid when connecting the power cord or related connection. Be sure to unplug the power supply when not using this product or before cleaning.

Before using the product, please familiarize yourself with the operation function of the product. Please do not expose children or non-professionals to the product. Please do not shake this product. Do not use brute force when installing or operating products. Don't let non-professionals operate the product. Most of the damage is caused by unprofessional operation.

The product is not equipped with relevant repair accessories. The maintenance and repair of products can only be carried out by professionals!

Please do not change this product without authorization, otherwise it may damage the product, so the damage caused is not covered by the warranty. Moreover, unprofessional operation may lead to short circuit, burn or electric shock, etc.

Connection of DMX512 signal:

This lamp uses DMX512 signal control mode. The control signal of each lamp is in parallel. When connecting multiple lamp signals, it is best to use a double-core shielded cable. When connecting, each lamp is connected through the DMX signal jack (Canon seat) INPUT (input) and OUTPUT (output) on the lamp. The 3-core XLR plug terminal connecting the signal line of the lamp must correspond to each other. When connecting the lamp signal, it is recommended to use the DMX signal terminal. . It can be avoided that the control signal is damaged due to electrical noise. The DMX signal terminal connects a 120 ohm 1W resistor between the 2 pins and 3 pins of an XLR plug and connects it to the OUTPUT (output) jack of the last lamp.

Calculation method of the starting address code of the lamp:

The starting address code of the current lamp is equal to (the starting address code of the previous lamp) + (the number of channels of the lamp) Description:

1: The starting address value of the first lamp is A001.

2: The basic number of channels of the controller should be greater than or equal to the total number of channels used by the lamp.

3: Note: When using any controller, each lamp must have its own starting address code. If the starting address code of the first lamp is set to A001 and the number of lamp channels is 16CH; then the starting address code of the second lamp is set to A01. 7The starting address code of the third lamp is set to A03.3By analogy, (this setting also needs to be determined according to different consoles).

| menu operation settings |  |  |  |
| --- | --- | --- | --- |
| Main menu | Sub-menu one | Function settings | Introduction to functions |
| DMX address |  | 1-512 |  |
| System settings | Show inverted | Normal | . |
|  |  | Flip over |  |
|  | Language settings | Chinese |  |
|  |  | English |  |
|  | Channel mode | 20CH |  |
|  | Operation mode | DMX |  |
|  |  | Sound-activated |  |
|  |  | Voluntary |  |
|  | Electronic lighting | Shut |  |
|  |  | Open |  |
|  | The light bulb defaults | Open_Close |  |
|  | Light bulb control | Light up. |  |
|  | The shortest path | Open_Close |  |
|  | Horizontal reverse | Shut |  |
|  |  | Open |  |
|  | Vertical and reverse | Shut |  |
|  |  | Open |  |
|  | XY speed | Low speed |  |
|  |  | Medium speed |  |
|  |  | Fast |  |
|  | Optocoupler detection | Shut |  |
|  |  | Open |  |
|  | Return to the superior | ▲ |  |
| Information inquiry | Total time | 0-9999 |  |
|  | Light bulb time | 0-9999 |  |
|  | Time limit | 0-9999 |  |
|  | Display software | Vx001 |  |
|  | XY software | Vx001 |  |
|  | Small motor software | Vx001 |  |
|  | Return to the superior | ▲ |  |
| Channel control | Rotation of the color palette | 0-255 |  |
|  | Strobe | 0-255 |  |
|  | Dim the light | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | Prism | 0-255 |  |
|  | Prism rotation. | 0-255 |  |
|  | Atomization | 0-255 |  |
|  | Focusing. | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine-tuning | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine-tuning | 0-255 |  |
|  | Reduce | 0-255 |  |
|  | Light up | 0-255 |  |
|  | XY speed | 0-255 |  |
|  | Color disk speed | 0-255 |  |
|  | Focusing speed | 0-255 |  |
|  | Pattern speed | 0-255 |  |
|  | Return to the superior | 0-255 |  |
| Working status | XY communication | Normal | Current status |
|  |  | Wrong |  |
|  | Small motor communication | Normal |  |
|  |  | Wrong |  |
|  | Focusing Hall | Normal |  |
|  |  | Wrong |  |
|  | Color plate Hall | Normal |  |
|  |  | Wrong |  |
|  | Pattern Hall | Normal |  |
|  |  | Wrong |  |
|  | X-optical coupling | Normal |  |
|  |  | Wrong |  |
|  | Y optocoupler | Normal |  |
|  |  | Wrong |  |
|  | Fan speed. | 6500 |  |
|  | Return to the superior | ▲ |  |
| Auxiliary tools | Check code 1 | 0-9999 |  |
|  | Total time | Reset |  |
|  |  | Carry out |  |
|  | Light and cannon time | Reset |  |
|  |  | Carry out |  |
|  | Restore the default | Tacitly approve |  |
|  |  | Resume |  |
|  |  | Preserve |  |
|  | Horizontal rotation | 0-255 | Zero initial position |
|  | Vertical rotation | 0-255 |  |
|  | Focusing. | 0-255 |  |
|  | Prism 1 (the prism) | 0-255 |  |
|  | Prism 2 (Skiw) | 0-255 |  |
|  | Prism rotation 1st | 0-255 |  |
|  | Prism rotation 2 (two) | 0-255 |  |
|  | Atomization | 0-255 |  |
|  | Color combination | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | Color plate | 0-255 |  |
|  | Strobes | 0-255 |  |
|  | The midpoint of strobe | 0-255 |  |
|  | The middle point of the first point | 0-255 |  |
|  | The middle point of the two edges | 0-255 |  |
|  | Midpoint of atomization | 0-255 |  |
|  | Colorful points | 0-255 |  |
|  | Return to the superior | ▲ |  |
| Advanced settings | Check code 2 | 0-9999 |  |
|  | Encryption of signals | Open |  |
|  |  | Shut |  |
|  | DMX speed | 0-255 |  |
|  | DMX data package | 0-255 |  |
|  | DMX type | 0-055 |  |
|  | DMX isolation | 0-65535 | The smaller the value, the more stable the signal. |
|  | Drive board temperature | <110 degrees> | If it exceeds this range, it will show red. |
|  | Drive board 3.3V | 3.0V - 3.6V | If it exceeds this range, it will show red. |
|  | Drive board 5.0V | 4.5V - 5.5V | If it exceeds this range, it will show red. |
|  | Drive board 12V | 10.0V - 14.0V | If it exceeds this range, it will show red. |
|  | Drive board 36V | 20.0V - 42.0V | If it exceeds this range, it will show red. |
|  | X state | Be static |  |
|  |  | Turn |  |
|  | X Hall | Normal |  |
|  |  | Wrong | Show red |
|  | X-coupler phase | 0-3 |  |
|  | X-coupler A | 40-160 | The standard setting of 100 exceeds the required range to display red, but the red light on the panel does not alarm. |
|  | X-optical coupling B | 40-160 | The standard setting of 100 exceeds the required range to display red, but the red light on the panel does not alarm. |
|  | X-optical coupling C | 40-160 | The standard setting of 100 exceeds the required range to display red, but the red light on the panel does not alarm. |
|  | X-optical coupling D | 40-160 | The standard setting of 100 exceeds the required range to display red, but the red light on the panel does not alarm. |
|  | X current angle | 000-540 |  |
|  | X-driven state | Normal |  |
|  |  | Wrong | Show red |
|  | Y state | Be static |  |
|  |  | Turn |  |
|  | Y optocoupler | Normal |  |
|  |  | Wrong | Show red |
|  | Y Hall | Normal |  |
|  |  | Wrong | Show red |
|  | Y photocoupler phase | 0-3 |  |
|  | Y optocoupler A | 40-160 | If it exceeds this range, it will show red. |
|  | Y optocoupler B | 40-160 | If it exceeds this range, it will show red. |
|  | Y optocoupler C | 40-160 | If it exceeds this range, it will show red. |
|  | Y optocoupler D | 40-160 | If it exceeds this range, it will show red. |
|  | Y current angle | 000-250 |  |
|  | Y drive state | Normal |  |
|  |  | Wrong | Show red |
|  | Return to the superior | ▲ |  |
| Channel level. | Rotation of the color palette | 0-255 | Current DMX channel value |
|  | Strobe | 0-255 |  |
|  | Dim the light | 0-255 |  |
|  | Fixed pattern | 0-255 |  |
|  | Prism | 0-255 |  |
|  | Prism rotation. | 0-255 |  |
|  | Atomization | 0-255 |  |
|  | Focusing. | 0-255 |  |
|  | Horizontal rotation | 0-255 |  |
|  | Horizontal fine-tuning | 0-255 |  |
|  | Vertical rotation | 0-255 |  |
|  | Vertical fine-tuning | 0-255 |  |
|  | Reduce | 0-255 |  |
|  | Light up | 0-288 |  |
|  | XY speed | 0-255 |  |
|  | Color disk speed | 0-255 |  |
|  | Focusing speed | 0-255 |  |
|  | Pattern speed | 0-255 |  |
|  | Return to the superior |  |  |
| Reduce | Re-re reset all | Cancel | Manual reset. |
|  |  | Carry out |  |
|  | XY reset | Cancel |  |
|  |  | Carry out |  |
|  | Effect reset " | Cancel |  |
|  |  | Carry out |  |
|  | Return to the superior |  |  |
| Exit the menu |  |  |  |

●20Channel table

| Passageway |  | Name | Numerical value | Describe |
| --- | --- | --- | --- | --- |
| CH1 | CH1 | Color | 0-4 | White light |
|  |  |  | 5-9 | White light + color 1 |
|  |  |  | 10-14 | Color1 |
|  |  |  | 15-19 | Color1+Color 2 |
|  |  |  | 20-24 | Color2 |
|  |  |  | 25-29 | Color2+Color3 |
|  |  |  | 30-34 | Color3 |
|  |  |  | 35-39 | Color3+Color4 |
|  |  |  | 40-44 | Color4 |
|  |  |  | 45-49 | Color4+Color5 |
|  |  |  | 50-54 | Color5 |
|  |  |  | 55-59 | Color5+Color6 |
|  |  |  | 60-64 | Color6 |
|  |  |  | 65-69 | Color6+Color7 |
|  |  |  | 70-74 | Color7 |
|  |  |  | 75-79 | Color7+Color8 |
|  |  |  | 80-84 | Color8 |
|  |  |  | 85-89 | Color8+ Color9 |
|  |  |  | 90-94 | Color9 |
|  |  |  | 95-99 | Color9+ Color 10 |
|  |  |  | 100-104 | Color10 |
|  |  |  | 105-109 | Color10+ Color 11 |
|  |  |  | 110-114 | Color11 |
|  |  |  | 115-119 | Color11+ Color 12 |
|  |  |  | 120-124 | Color12 |
|  |  |  | 125-129 | Color12+ Color 13 |
|  |  |  | 130-134 | Color13 |
|  |  |  | 135-139 | Color 13 + White light |
|  |  |  | 140-149 | White light |
|  |  |  | 150-203 | From fast to slow, forward flowing water |
|  |  |  | 204-255 | Reverse flow from slow to fast |
| CH2 | CH2 | Strobe | 0-3 | The light gate is closed. |
|  |  |  | 4-103 | Synchronous strobe |
|  |  |  | 104-107 | The light gate opens. |
|  |  |  | 108-207 | Divide the strobe equally. |
|  |  |  | 208-212 | The light gate opens. |
|  |  |  | 213-225 | Low-speed random strobe |
|  |  |  | 226-238 | Medium-speed random strobe |
|  |  |  | 239-251 | High-speed random strobe |
|  |  |  | 252-255 | The light gate opens. |
| CH3 | CH3 | Dim the light | 0-255 | 0-100% dimming |
| CH4 | CH4 | Pattern | 0-3 | White light hole |
|  |  |  | 4-7 | Pattern 1 |
|  |  |  | 8-11 | Pattern 2 |
|  |  |  | 12-15 | Pattern 3 |
|  |  |  | 16-19 | Pattern 4 |
|  |  |  | 20-23 | Pattern 5 |
|  |  |  | 24-27 | Pattern 6 |
|  |  |  | 28-31 | Pattern 7 |
|  |  |  | 32-35 | Pattern 8 |
|  |  |  | 36-39 | Pattern 9 |
|  |  |  | 40-43 | Pattern 10 |
|  |  |  | 44-47 | Pattern 11 |
|  |  |  | 48-51 | Pattern 12 |
|  |  |  | 52-55 | Pattern13 |
|  |  |  | 56-59 | Pattern14 |
|  |  |  | 60-63 | Pattern15 |
|  |  |  | 64-67 | Pattern16 |
|  |  |  | 68-71 | Pattern17 |
|  |  |  | 72-111 | Forward rotation-Fast-slow |
|  |  |  | 112-115 | Stop the running water |
|  |  |  | 1116-155 | Reverse rotation- Slow - Fast |
|  |  |  | 156-161 | Pattern1From slow to fast jitter |
|  |  |  | 162-167 | Pattern 2 shakes from slow to fast |
|  |  |  | 168-173 | Pattern 3 from slow to fast jitter |
|  |  |  | 174-179 | Pattern 4 from slow to fast jitter |
|  |  |  | 180-185 | Pattern5From slow to fast jitter |
|  |  |  | 186-191 | Pattern6From slow to fast jitter |
|  |  |  | 192-197 | Pattern7From slow to fast jitter |
|  |  |  | 198-203 | Pattern8From slow to fast jitter |
|  |  |  | 204-209 | Pattern9From slow to fast jitter |
|  |  |  | 210-216 | Pattern10From slow to fast jitter |
|  |  |  | 217-222 | Pattern11From slow to fast jitter |
|  |  |  | 223-228 | Pattern12From slow to fast jitter |
|  |  |  | 229-234 | Pattern13From slow to fast jitter |
|  |  |  | 235-240 | Pattern 14From slow to fast jitter |
|  |  |  | 241-246 | Pattern 15From slow to fast jitter |
|  |  |  | 247-252 | Pattern 16From slow to fast jitter |
|  |  |  | 253-255 | Pattern 17From slow to fast jitter |
| CH5 | CH5 | Prism | 0-127 | Remove the prism. |
|  |  |  | 128-255 | Prism cut in |
| CH6 | CH6 | Prism rotation. | 0-127 | 0-400 degrees |
|  |  |  | 128-190 | From fast to slowTurn overFlowing water |
|  |  |  | 191-192 | Stop |
|  |  |  | 193-255 | From slow to fastFirst month of the lunar yearFlowing water |
| CH7 | CH7 | Reserve | 0-255 | No function |
| CH8 | CH8 | Colorful atomization | 0-127 | Not have |
|  |  |  | 128-192 | Insert atomization |
|  |  |  | 193-255 | Insert seven colors |
| CH9 | CH9 | Focus on | 0-255 | From far to near |
| CH10 | CH10 | X axis | 0-255 | 0-540Linear measure |
| CH11 | CH11 | X-axis fine-tuning | 0-255 | UnknownAxis 16Bit fine-tuning channel |
| CH12 | CH12 | Y-axis | 0-255 | 0-240 degrees |
| CH13 | CH13 | Y-axis fine-tuning | 0-255 | Y-axis 16Bit fine-tuning channel |
| CH14 | CH14 | Reserve | 0-255 | No function |
| CH15 | CH15 | Reduce | 0-25 | No function |
|  |  |  | 26-76 | Effect motor reset (5S) |
|  |  |  | 77-127 | Scanning motor reset (5S) |
|  |  |  | 128-255 | All motor reset (5S) |
| CH16 | CH16 | Reduce | 0-9 | No function |
|  |  |  | 10-100 | Extinguish the light bulb (5S) |
|  |  |  | 101-255 | Light bulb (5S) |
|  | CH17 | XY speed | 0-255 | From fast to slow |
|  | CH18 | Color disk speed | 0-255 | From fast to slow |
|  | CH19 | Prism atomization speed | 0-255 | From fast to slow |
|  | CH20 | Pattern disk speed | 0-255 | From fast to slow |

Installation instructions for lamps and lanterns:

This lamp can be placed horizontally, hanging obliquely and upside down. You must pay attention to the installation method when hanging obliquely and upside down. Fixed installation of lamps and lanterns: Before positioning the lamps and lanterns, it is necessary to ensure the stability of the installation site. When reverting the hanging installation, it is necessary to ensure that the lamps and lanterns do not fall off the support frame. The safety rope should be used to pierce the support frame and the lamp handle to carry out auxiliary hanging to ensure safety. To prevent the lamp from falling and sliding, when the lamp is installed and debugged, pedestrians are prohibited from passing through the lower part. Regularly check whether the safety rope is worn and whether the hook screw is loose. If the hanging installation is unstable, the manufacturer shall not bear any responsibility for all the consequences caused by the falling of the lamp.

After-sales service

Attention! When the lamps leave the factory, they have undergone strict quality inspection and the packaging is intact. Please follow the instructions. Machine failures caused by human causes are not covered by the warranty.

- The company provides technical consultation to customers throughout their lives.

- If the product fails and needs to be repaired, please show the product warranty card and fill in the relevant content truthfully. At the same time, we also hope that customers can give us feedback on the problems of the product in time, so that we can improve the product as soon as possible.