Instructions for four-eye retro matrix lamp

- technical parameter

Power supply: AC90～240V, 50Hz/60Hz.

Rated power: 180W

Lamp bead parameters: 4 40W COB lamp beads, 4 segments of RGB auxiliary light, and 4 circles of lamp strips (124 0.2W RGB LED).

Control mode: international DMX512, 11/23/32 channel.

Dimming mode: 0~100% ultra-smooth dimming.

Control: voice control, self-propelled, master-slave and DMX console mode.

- Key operation and digital tube display instructions

MENU: function key	UP: DOWN: ENTER: Enter: Enter.

- Digital tube display menu description

| Address | A001-A512 |  | address code |
| --- | --- | --- | --- |
| Channel | 11/23/32 CH |  | Channel selection |
| STAT | dimm | 000-255 | Total dimming |
|  | stor | 000-255 | Main light stroboscopic |
|  | Gol1 | 000-255 | Main light golden yellow 1 |
|  | Gol2 | 000-255 | Main light golden yellow 2 |
|  | Gol3 | 000-255 | Main light golden yellow 3 |
|  | Gol4 | 000-255 | Main light golden yellow 4 (Control the brightness when running the main light effect) |
|  | efet | 005-255 | Main light/combination effect |
|  | sped | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
|  | stor | 000-255 | Auxiliary light/strip strobe |
|  | Red | 000-255 | Auxiliary light red (Control the color when running the auxiliary light effect) |
|  | Gre | 000-255 | Auxiliary light green (Control the color when running the auxiliary light effect) |
|  | Blu | 000-255 | Auxiliary light blue (Control the color when running the auxiliary light effect) |
|  | efet | 000-255 | - Auxiliary light effect |
|  | sped | 000-255 | - Auxiliary light effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
|  | bgcl | 000-255 | Auxiliary light effect background color |
|  | bgdi | 000-255 | Auxiliary light effect background tone light |
|  | Red | 000-255 | The light is reddish. (Control the color when running the light strip effect) |
|  | Gre | 000-255 | The light is green. (Control the color when running the light strip effect) |
|  | Blu | 000-255 | The light is blue. (Control the color when running the light strip effect) |
|  | efet | 000-255 | - Lamp strip effect |
|  | sped | 000-255 | - Effect speed of light strip: the forward speed is adjusted by 0-127 and the reverse speed is adjusted by 128-255; |
|  | bgcl | 000-255 | Light band effect background color |
|  | bgdi | 000-255 | Light band effect background tone light |
| AUTO | Au 1-10 | Speed 1-15 | 10 self-propelled effects |
| Soud | Su 1-2 | Sensitivity 1-15 | Voice control effect/sensitivity |
| LED | On / Off |  | The screen stands by for 30 seconds and goes out. |
| disp | On / Off |  | Reverse display |
| Uer | V-11 |  | version number |

- DMX channel menu

- DMX 11 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Total stroboscopic |
| CH3 | 000-255 | hot light |
| CH4 | 000-255 | Auxiliary light red |
| CH5 | 000-255 | Auxiliary light green |
| CH6 | 000-255 | Auxiliary light blue |
| CH7 | 000-255 | The lamp is reddish. |
| CH8 | 000-255 | The light is green |
| CH9 | 000-255 | Lamp band blue |
| CH10 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH11 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |

- DMX 23 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Main light stroboscopic |
| CH3 | 000-255 | Main light golden yellow 1 |
| CH4 | 000-255 | Main light golden yellow 2 |
| CH5 | 000-255 | Main light golden yellow 3 |
| CH6 | 000-255 | Main light golden yellow 4, (control brightness when running the main light effect) |
| CH7 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH8 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH9 | 000-255 | Auxiliary light/strip strobe |
| CH10 | 000-255 | Auxiliary light red, (control color when running auxiliary light effect) |
| CH11 | 000-255 | Auxiliary light green, (control color when running auxiliary light effect) |
| CH12 | 000-255 | Auxiliary light blue, (control color when running auxiliary light effect) |
| CH13 | 000-255 | - Auxiliary light effect (see attached table 1 for details) |
| CH14 | 000-255 | - Auxiliary light effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH15 | 000-255 | Auxiliary light effect background color |
| CH16 | 000-255 | Auxiliary light effect background tone light |
| CH17 | 000-255 | The light band is red (control the color when running the light band effect). |
| CH18 | 000-255 | The light band is green (control the color when running the light band effect). |
| CH19 | 000-255 | The light band is blue, (control the color when running the light band effect) |
| CH20 | 000-255 | - Effect of light strip (see attached table 1 for details) |
| CH21 | 000-255 | - Effect speed of light strip: the forward speed is adjusted by 0-127 and the reverse speed is adjusted by 128-255; |
| CH22 | 000-255 | Light band effect background color |
| CH23 | 000-255 | Light band effect background tone light |

- DMX 32 channel table

| passage | numerical value | function declaration |
| --- | --- | --- |
| CH1 | 000-255 | Total dimming |
| CH2 | 000-255 | Main light stroboscopic |
| CH3 | 000-255 | Main light golden yellow 1 |
| CH4 | 000-255 | Main light golden yellow 2 |
| CH5 | 000-255 | Main light golden yellow 3 |
| CH6 | 000-255 | Main light golden yellow 4, (control brightness when running the main light effect) |
| CH7 | 005-255 | Main light/combination effect (see attached table 1 for details) |
| CH8 | 000-255 | - Main light/combined effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH9 | 000-255 | Auxiliary light/strip strobe |
| CH10 | 000-255 | Auxiliary light red 1 |
| CH11 | 000-255 | Auxiliary light green 1 |
| CH12 | 000-255 | Auxiliary light blue 1 |
| ... | ... | ... |
| ... | ... | ... |
| CH19 | 000-255 | Auxiliary light red 4, (control color when running auxiliary light effect) |
| CH20 | 000-255 | Auxiliary light green 4, (control color when running auxiliary light effect) |
| CH21 | 000-255 | Auxiliary light blue 4, (control color when running auxiliary light effect) |
| CH22 | 000-255 | - Auxiliary light effect (see attached table 1 for details) |
| CH23 | 000-255 | - Auxiliary light effect speed: the forward speed is adjusted by 0-127, and the reverse speed is adjusted by 128-255; |
| CH24 | 000-255 | Auxiliary light effect background color |
| CH25 | 000-255 | Auxiliary light effect background tone light |
| CH26 | 000-255 | The light band is red (control the color when running the light band effect). |
| CH27 | 000-255 | The light band is green (control the color when running the light band effect). |
| CH28 | 000-255 | The light band is blue, (control the color when running the light band effect) |
| CH29 | 000-255 | - Effect of light strip (see attached table 1 for details) |
| CH30 | 000-255 | - Effect speed of light strip: the forward speed is adjusted by 0-127 and the reverse speed is adjusted by 128-255; |
| CH31 | 000-255 | Light band effect background color |
| CH32 | 000-255 | Light band effect background tone light |

Schedule 1: Description of built-in effects and functions

| CH value |  | CH value |  | CH value |  |
| --- | --- | --- | --- | --- | --- |
| 0 - 4 | all-or-none | 0 - 4 | all-or-none | 0 | all-or-none |
| 5 - 12 13 - 20 21 - 28 29 - 36 37 - 44 45 - 52 53 - 60 61 - 68 69 - 76 77 - 84 85 - 92 93 - 100 101 - 108 109 - 116 117 - 124 125 - 132 133 - 140 141 - 148 149 - 156 157 - 164 165 - 172 173 - 180 181 - 188 189 - 196 197 - 204 205 - 212 213 - 220 221 - 228 229 - 236 237 - 249 | Main light effect 1 Main light effect 2 Main light effect 3 Main light effect 4 Main light effect 5 Main light effect 6 Main light effect 7 Main light effect 8 Main light effect 9 Main light effect 10 Main light effect 11 Main light effect 12 Main light effect 13 Main light effect 14 Main light effect 15 Main light effect 16 Main light effect 17 Main light effect 18 Main light effect 19 Main light effect cycle 20 Combined effect 1 Combined effect 2 Combined effect 3 Combined effect 4 Combined effect 5 Combined effect 6 Combined effect 7 Combined effect 8 Combined effect 9 Combined effect 10 | 5 - 12 13 - 20 21 - 28 29 - 36 37 - 44 45 - 52 53 - 60 61 - 68 69 - 76 77 - 84 85 - 92 93 - 100 101 - 108 109 - 116 117 - 124 125 - 132 133 - 140 141 - 148 149 - 156 157 - 164 165 - 172 173 - 180 181 - 188 189 - 196 197 - 204 205 - 212 213 - 220 221 - 228 229 - 236 237 - 255 | Auxiliary light effect 1 Auxiliary light effect 2 Auxiliary light effect 3 Auxiliary light effect 4 Auxiliary light effect 5 Auxiliary light effect 6 Auxiliary light effect 7 Auxiliary light effect 8 Auxiliary light effect 9 Auxiliary light effect 10 Auxiliary light effect 11 Auxiliary light effect 12 Auxiliary light effect 13 Auxiliary light effect 14 Auxiliary light effect 15 Auxiliary light effect 16 Auxiliary light effect 17 Auxiliary light effect 18 Auxiliary light effect 19 Auxiliary light effect 20 Auxiliary light effect 21 Auxiliary light effect 22 Auxiliary light effect 23 Auxiliary light effect 24 Auxiliary light effect 25 Auxiliary light effect 26 Auxiliary light effect 27 Auxiliary light effect 28 Auxiliary light effect 29 Auxiliary light effect cycle 30 | 1 - 3 4 - 6 7 - 9 10 - 12 13 - 15 16 - 18 19 - 21 22 - 24 25 - 27 28 - 30 31 - 33 34 - 36 37 - 39 40 - 42 ... 211 - 213 214 - 216 217 - 219 220 - 222 223 - 225 226 - 228 229 - 231 232 - 234 235 - 237 238 - 240 241 - 243 244 - 246 247 - 249 250 - 252 253 - 255 | Lamp strip effect 1 Lamp strip effect 2 Lamp strip effect 3 Light strip effect 4 Light strip effect 5 Lamp strip effect 6 Light strip effect 7 Light strip effect 8 Lamp strip effect 9 Light Strip Effect 10 Light Strip Effect 11 Lamp Strip Effect 12 Lamp Strip Effect 13 Light Strip Effect 14 ... Light strip effect 71 Light strip effect 72 Light strip effect 73 Light strip effect 74 Light strip effect 75 Light strip effect 76 Light strip effect 77 Light strip effect 78 Light strip effect 79 Light strip effect 80 Light strip effect 81 Light strip effect 82 Light strip effect 83 Light strip effect 84 Light Strip Effect Cycle 85 |
| 250 - 252 | Voice control effect 1 |  |  |  |  |
| 253 - 255 | Voice control effect 2 |  |  |  |  |