Instructions for 24 Thunder Curtain Lights

Because the products are constantly updated, the contents of this manual may be slightly different from the actual products, please understand!

product catalog

- Display panel key function

- Menu function description

- Channel description

- Use of common functions

- Master-slave setting

- technical parameter

One: Display panel operation

Menu up and down confirmation

Basic usage:

MENU key: select various functions.

UP: Incremental selection of function parameters.

DOWN: function parameters are selected in descending order.

ENTER: determines the current menu.

Second, the menu function

Press the MENU key (menu key) after power-on and start-up, and the following functions will appear in turn, and then press the confirm key to confirm. (This order is the new machine order. If the lamp has used other modes and saved this mode before turning off, the mode before turning off will be displayed after turning on again.) After setting other functions, you also need to press the Save button to save, otherwise the effect of setting before turning off will not appear after turning on next time.

| A001 |  | 001-512 | address code |
| --- | --- | --- | --- |
| 10CH | Channel mode |  | Channel selection 05CH,10CH,128CH,138CH. |
| -512 | operational mode |  | DMX512, voice control, self-propelled |
| E001 | Effect mode | E001-E250 | Effect selection (250 kinds) |
| P000 | Effect speed | P000-P255 | The UP/DOWN key selects the running speed (P000-P255), and the ENTER key saves the current running speed. |
| CL00 | Effect color selection | CL00-CL07 | 7 color choices |
| R255 | Manual red | R000-R255 | Manual red adjustment, when there is no console, use the UP/DOWN key to adjust the brightness between R000 and R255. |
| G255 | Manual green | G000-G255 | Manual green adjustment, when there is no console, use the UP/DOWN key to adjust the brightness between G000 and G255. |
| B255 | Manual blue | B000-B255 | Manual blue adjustment, when there is no console, use the UP/DOWN key to adjust the brightness between B000 and B255. |
| W255 | Manual white light | W000-W255 | Manual white light adjustment, when there is no console, use UP/DOWN key to adjust brightness between W000 and W255. |
| Y255 | Manual cold white light | Y000-Y255 | Manual cold white light adjustment, when there is no console, use the UP/DOWN key to adjust the brightness between Y000 and Y255. |
| F000 | Manual stroboscopic | F000-F255 | Use the UP or DOWN key to select the strobe speed. |
| T000 | Temperature display | T000-T105 | Display temperature |
| Press the OK key to save after setting, and the display will flash when saving. |  |  |  |

- Channel description

| 5 channels | function | Channel value | describe |
| --- | --- | --- | --- |
| one | Red (r) | 0-255 | 0-%100 linear dimming |
| 2 | Green (g) | 0-255 | 0-%100 linear dimming |
| three | Blue (b) | 0-255 | 0-%100 linear dimming |
| four | White (w) | 0-255 | 0-%100 linear dimming |
| five | Cold white (y) | 0-255 | 0-%100 linear dimming |

| 10 channels | function | Channel value | describe |
| --- | --- | --- | --- |
| one | Total dimming | 0-255 | 0-%100 linear dimming |
| 2 | stroboscopic | 0-9 | neutrality |
|  |  | 10-255 | Strobe from slow to fast |
| three | Combined effect | 0 | all-or-none |
|  |  | one | R |
|  |  | 2 | G |
|  |  | three | B |
|  |  | four | RG |
|  |  | five | RB |
|  |  | six | GB |
|  |  | seven | RGB |
|  |  | eight | W |
|  |  | nine | Y |
|  |  | 10 | RGBWY |
|  |  | 11 | gradual change |
|  |  | 12 | Pulse change |
|  |  | 13 | jump |
|  |  | 14 | Effect 1 |
|  |  | … | … |
|  |  | 250 | Effect 237 |
|  |  | 251-255 | sound-controlled |
| four | Combined effect speed | 0-255 | Speed from slow to fast |
| five | Combination effect color selection | 0-15 | be invalid |
|  |  | 16-50 | R |
|  |  | 51-85 | G |
|  |  | 86-120 | B |
|  |  | 121-155 | RG |
|  |  | 156-190 | RB |
|  |  | 191-225 | GB |
|  |  | 226-255 | RGB |
| six | Red (r) | 0-255 | 0-%100 linear dimming |
| seven | Green (g) | 0-255 | 0-%100 linear dimming |
| eight | Blue (b) | 0-255 | 0-%100 linear dimming |
| nine | White (w) | 0-255 | 0-%100 linear dimming |
| 10 | Cold white (y) | 0-255 | 0-%100 linear dimming |

| 128 channel | function | Channel value | describe |
| --- | --- | --- | --- |
| one | Cold white Y1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| 24 | Cold white Y24 | 0-255 | 0-%100 linear dimming |
| 25 | White W1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| fifty-six | White W32 | 0-255 | 0-%100 linear dimming |
| 57 | Auxiliary light R1 | 0-255 | 0-%100 linear dimming |
| 58 | Auxiliary light G1 | 0-255 | 0-%100 linear dimming |
| 59 | Auxiliary light B1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| 126 | Auxiliary light R24 | 0-255 | 0-%100 linear dimming |
| 127 | Auxiliary light G24 | 0-255 | 0-%100 linear dimming |
| 128 | Auxiliary light B24 | 0-255 | 0-%100 linear dimming |

| 10 channels | function | Channel value | describe |
| --- | --- | --- | --- |
| one | Total dimming | 0-255 | 0-%100 linear dimming |
| 2 | stroboscopic | 0-9 | neutrality |
|  |  | 10-255 | Strobe from slow to fast |
| three | Combined effect | 0 | all-or-none |
|  |  | one | R |
|  |  | 2 | G |
|  |  | three | B |
|  |  | four | RG |
|  |  | five | RB |
|  |  | six | GB |
|  |  | seven | RGB |
|  |  | eight | W |
|  |  | nine | Y |
|  |  | 10 | RGBWY |
|  |  | 11 | gradual change |
|  |  | 12 | Pulse change |
|  |  | 13 | jump |
|  |  | 14 | Effect 1 |
|  |  | … | … |
|  |  | 250 | Effect 237 |
|  |  | 251-255 | sound-controlled |
| four | Combined effect speed | 0-255 | Speed from slow to fast |
| five | Combination effect color selection | 0-15 | be invalid |
|  |  | 16-50 | R |
|  |  | 51-85 | G |
|  |  | 86-120 | B |
|  |  | 121-155 | RG |
|  |  | 156-190 | RB |
|  |  | 191-225 | GB |
|  |  | 226-255 | RGB |
| six | Red (r) | 0-255 | 0-%100 linear dimming |
| seven | Green (g) | 0-255 | 0-%100 linear dimming |
| eight | Blue (b) | 0-255 | 0-%100 linear dimming |
| nine | White (w) | 0-255 | 0-%100 linear dimming |
| 10 | Cold white (y) | 0-255 | 0-%100 linear dimming |
| 11 | Cold white Y1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| 34 | Cold white Y24 | 0-255 | 0-%100 linear dimming |
| 35 | White W1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| 66 | White W32 | 0-255 | 0-%100 linear dimming |
| 67 | Auxiliary light R1 | 0-255 | 0-%100 linear dimming |
| sixty-eight | Auxiliary light G1 | 0-255 | 0-%100 linear dimming |
| sixty-nine | Auxiliary light B1 | 0-255 | 0-%100 linear dimming |
| … | … | … | … |
| 136 | Auxiliary light R24 | 0-255 | 0-%100 linear dimming |
| 137 | Auxiliary light G24 | 0-255 | 0-%100 linear dimming |
| 138 | Auxiliary light B24 | 0-255 | 0-%100 linear dimming |

Fourth, the use of common functions

1, DMX512 control

Set the address code after power-on. If it is the first set, set the address code to 001. Connect the DMX512 console with a three-core signal line, and the digital display tube will stop flashing, indicating that the 512 signal has entered the lamps and lanterns, and the lamps and lanterns can be controlled. At this time, according to the channel table, the console is used to control related functions.

When there are multiple lamps, as long as the lamps at the back are connected with the lamps at the front with signal lines and the address code is set, the LED display panel will stop flashing because the lamps at the back are connected with the 512 signal, and then it can be controlled as the lamps at the front.

2. Other functions can be used as long as the related functions are pressed by the menu key (use the UP key or DOWM key to select the corresponding speed).

Fifth, the master-slave machine setting

Two or more identical lamps and lanterns are connected by 3-core signal lines. If any one of the main machines is set to voice control or gradual change, pulse change and jump, and other lamps and lanterns are set to 001 mode, the master-slave machine mode will be entered and the display screen of the slave machine will not flash (flashing means that the machine has not received the signal, so the connecting line must be checked).

Special attention: only one host can be set in a group of lamps. If there are multiple hosts, all lamps will flash without synchronization.

VI. Technical parameters:

Voltage: AC100~240V 50/60HZ 180W

Power: 140W

Lamp beads: 24 3535*3W cold white lamp beads, 384 5050*0.2W RGB lamp beads and 256 5730*0.5W white lamp beads.

Control mode: DMX512, self-propelled, master-slave function. Each lamp bead can be controlled by points and zones.

Channels: CH05, CH10, CH128, CH138.

Dimming: 32bit 0~100% linear dimming

Features: dyeing+flash+code running point control light

Working temperature: -30 ~50℃

Strobe frequency: 1~30HZ

Appearance: metal, black

Connection mode: DMX512 input/output/power input/output.

IP grade: IP20

Size: weight: