| Product Technical Specification (Moving Head Light Series) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product Image |  | Product Dimension Drawing |  |  |  |
| Lighting Specifications |  |  |  |  | Parameters |
| Product Name | Outdoor Waterproof Moving Head 3-in-1 | Product Model | 450 BSW IP |  | Input Voltage: AC100V-240V 50/60HZ Power Consumption: 600W Light Source: USHIO NSLU7 Light source lifespan: 1500 hours Diameter of Frontal Lens: 140mm Beam angle: 2°-31° CCT：7900K CRI：＞70 Operating Temperature: -10-40℃ Control Mode: DMX512, Master-Slave, Auto Run, RDM Channel: 28CH/20CH Color: Linear CMY color mixing& Linear CMY color mixing Linear CTO Color Correction3200K-6700K 1 Color Wheel: 6 Colors 2 Color Wheel: 6 Colors 1 Static Gobo Wheel：16 gobos 1 Rotating Gobo Whee: 9 interchangeable gobos Prism: 2 Prisms, Independent Rotation, Can be Overlapped Frost Filter: 1 Frost Filter Homogenizer：1 homogenizer lens Dimming: 0-100% Smooth Dimming Strobe: 1-20 Times/Second, Adjustable Speed IP Rating: IP66, Waterproof Pan scan: 540 Degrees 16 bit Tilt scan: 270 Degrees 16 bit Display: LCD Display with 4 Touch Buttons Cooling Fan: Mute Fan, Variable Speed Shell: Magnesium Alloy Data In/Out: DMX 3-pin Input/Output + 5-pin Input/Output Power In/Out: Waterproof power connector in/out Product Dimensions: 709*386*290mm Packing Dimensions:790*405*485mm Net Weight: 27kg Gross Weight: 30kg |
|  |  | Light Color | Black |  |  |
| Net Weight | 27kg | Gross Weight | 30kg |  |  |
| Product Dimensions | 709*386*290mm | Packing Dimensions | 790*405*485mm |  |  |
| Protection Rating | IP66 | Lighting Material | All-magnesium alloy body |  |  |
| Optical System |  | Effect System |  |  |  |
| Light Source Information | USHIO NSLU7 | Rotating Gobos | 1 rotating gobo wheel with 9 interchangeable gobos |  |  |
| Light Source Life | 1500 hours | Static Gobos | 1 Static Gobo Wheel with 16 Gobos, Variable Speed Jitter and Bidirectional Rotation |  |  |
| Beam Angle | 2°-31° | Color | 1、Linear CMY color mixing system and linear CTO color temperature adjustment 2、2 Color Wheels with 12 Colors |  |  |
| Color Temperature | 7900K | Prism | 2 Prisms（6-facet prism , 6 facet linear prism）, Bidirectional Variable Speed Rotation, Achievable Overlay Effect |  |  |
| Dimming | 0-100% Smooth Dimming | Fogging | 1 Independent Frost Effect, Soft and Natural Light Spot |  |  |
| Focus | 0~100% Electronic High-Definition Focus | Strobe | 1-20Hz Fast Strobe per Second, with Pulse Strobe, Synchronous and Asynchronous Strobe Effects |  |  |
| Homogenizer | 1 homogenizer lens |  |  |  |  |
| Electrical Connection |  | Control System |  |  |  |
| Input Voltage | AC 100V-240V，50/60HZ | Mechanical Movement | Horizontal Scanning Angle 540°, Vertical Scanning Angle 270°, Automatic Return Correction and 16bit Precision Fine-Tuning Function |  |  |
| Total Lamp Power | 600W | Communication Design | DMX Wired/Wireless Transmission, Convenient and Fast Software Upgrade via DMX Data Cable |  |  |
| Signal Connection | - 3-pin/5-pin Waterproof Signal Cable & Xishang Waterproof 3-pin XLR Socket | Heat Dissipation System | Fan Heat Dissipation Structure and Temperature Intelligent Monitoring Technology, Automatically Adjusts Heat Dissipation System to Effectively Control Lamp Temperature |  |  |
| Power Connection | Waterproof Power Cable Input/Output & Xishang Waterproof Power Socket | Control Protocol | Auto Run, Master-Slave Mode, DMX512 |  |  |
| Display Screen | 2.8-inch Display Screen with Automatic Flip Function | Channel Mode | 28CH/20CH |  |  |
| Others |  |  |  |  |  |
| Photometric Diagram |  |  |  | Color: Linear CMY Mixing System, Linear CTO Color Temperature Adjustment, 2 Color Wheels with 12 Colors |  |
| Gobo Wheel |  |  |  | Effect |  |