| Product Technical Specification (Moving Head Light Series) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Product Image |  | Product Dimension Drawing |  |  |  |
| Lighting Specifications |  |  |  |  | Parameters |
| Product Name | Outdoor Waterproof Moving Head Beam Light | Product Model | 380 BEAM IP |  | Input Voltage: AC100V-240V 50/60HZ Power Consumption: 550W Light Source: USHIO NSLU6C Light source lifespan: 1500 hours Diameter of Frontal Lens: 160mm Beam angle: 1.7° Operating Temperature: -10-40℃ Control Mode: DMX512, Master-Slave, Auto Run, RDM Channel: 16 Channels 1 Color Wheel: 14 colors + plus open, Features half-color and linear adjustment, enabling bi-directional rainbow effects 1 Static Gobos: 13 gobos + plus open, Variable Speed Jitter and Bidirectional Rotation Prism: 2 Prisms, Independent Rotation, Can be Overlapped Frost Filter: 1 Frost Filter Dimming: 0–100% linear dimming Strobe: 1-20 Times/Second, Adjustable Speed IP Rating: IP66, Waterproof Pan scan: 540 Degrees 16 bit Tilt scan: 270 Degrees 16 bit Display: LCD Display with 4 Touch Buttons Cooling Fan: Mute Fan, Variable Speed Shell: Die-cast Aluminum Data In/Out: DMX 3-pin Input/Output + 5-pin Input/Output Power In/Out: Waterproof power connector in/out Product Dimensions: 631*403*289mm Packing Dimensions: 755*445*460mm Net Weight: 25kg Gross Weight: 28kg With folding clamp and arm handle |
|  |  | Light Color | Black |  |  |
| Net Weight | 25kg | Gross Weight | 28kg |  |  |
| Product Dimensions | 631*403*289mm | Packing Dimensions | 755*445*460mm |  |  |
| Protection Rating | IP66 | Lighting Material | All die-cast aluminum body |  |  |
| Optical System |  | Effect System |  |  |  |
| Light Source Information | USHIO NSLU6C | Rotating Gobos | None |  |  |
| Light Source Life | 1500 hours | Static Gobos | 1 Static Gobo Wheel with 13 Gobos, Variable Speed Jitter and Bidirectional Rotation |  |  |
| Beam Angle | 1.7° | Color | 1 color wheel with 14 colors plus open white, Features half-color and linear adjustment, enabling bi-directional rainbow effects |  |  |
| Color Temperature | / | Prism | 2 Prisms（8-facet prism, 24-facet prism）, Bidirectional Variable Speed Rotation, Achievable Overlay Effect |  |  |
| Dimming | 0–100% mechanical linear dimming | Fogging | 1 Independent Frost Effect, Soft and Natural Light Spot |  |  |
| Focus | 0~100% electronic linear high-definition Focus | Strobe | 1-20Hz Fast Strobe per Second, with Pulse Strobe, Synchronous and Asynchronous Strobe Effects |  |  |
| Electrical Connection |  | Control System |  |  |  |
| Input Voltage | 100V-240V AC，50/60HZ | Mechanical Movement | Horizontal Scanning Angle 540°, Vertical Scanning Angle 270°, Automatic Return Correction and 16bit Precision Fine-Tuning Function |  |  |
| Total Lamp Power | 550W | Communication Design | DMX Wired/Wireless Transmission, Convenient and Fast Software Upgrade via DMX Data Cable |  |  |
| Signal Connection | Waterproof signal cable | Heat Dissipation System | Fan Heat Dissipation Structure and Temperature Intelligent Monitoring Technology, Automatically Adjusts Heat Dissipation System to Effectively Control Lamp Temperature |  |  |
| Power Connection | Waterproof power input & output | Control Protocol | Auto Run, Master-Slave Mode, DMX512 |  |  |
| Display Screen | Fixture and lamp usage time can be queried for easy maintenance | Channel Mode | 16CH |  |  |
| Others |  |  |  |  |  |
| Photometric Diagram |  |  |  | 1 Color Wheel: 14colors+plus open |  |
| Gobo+Prisms/Frost Filters |  |  |  | Effect |  |
| 1 Static Gobos：13 gobos + plus open |  |  |  |  |  |